Semi-Annual Environmental Monitoring Report

Project Number: 49107-006

Period: November 2019–April 2019

Submission Date: September 2020

IND: WEST BENGAL DRINKING WATER SECTOR IMPROVEMENT PROJECT (WBDWSIP)

VOL 1: MAIN REPORT

Prepared by Project Management Unit, Government of West Bengal for Asian Development Bank. This document is made publicly available in accordance with ADB’s Access to Information Policy and does not necessarily reflect the views of ADB.

This environmental monitoring report is a document of the borrower. The views expressed herein do not necessarily represent those of ADB's Board of Directors, management, or staff, and may be preliminary in nature. Your attention is directed to the “terms of use” section of this website.

In preparing any country program or strategy, financing any project, or by making any designation of or reference to a particular territory or geographic area in this document, the Asian Development Bank does not intend to make any judgments as to the legal or other status of any territory or area.

|  | WEST BENGAL DRINKING WATER SECTOR IMPROVEMENT PROJECT (WBDWSIP) |
| --- | --- |
|  | PROJECT MANAGEMENT UNIT |

3rd

SEMI ANNUAL ENVIRONMENT MONITORING REPORT

ADB Loan 3696-IND

(Period November 2019 to April 2020)

September 2020

PUBLIC HEALTH ENGINEERING DEPARTMENT

GOVT. OF WEST BENGAL

TABLE OF CONTENTS

I.	INTRODUCTION	 5

II.	COMPLIANCE STATUS WITH NATIONAL/ STATE/ LOCAL SATATUTORY ENVIRONMENTAL REQUITREMENTS	 27

III. COMPLIANCE STATUS WITH ENVIRONMENTAL LOAN COVENANTS	 50

IV. COMPLIANCE STATUS WITH THE ENVIRONMENTAL MANAGEMENT PLAN 	54

V. APPROACH AND METHODOLOGY FOR ENVIRONMENTAL MONITORING OF THE PROJECT 	171

VI. MONITORING OF ENVIRONMENTAL IMPACTS ON PROJECT SURROUNDINGS 	172

VII.	SUMMARY OF KEY ISSUES AND REMEDIAL ACTIONS 	223

LIST OF APPENDICS- VOLUME 2

APPENDIX 1: PHOTO ILLUSTRATION

APPENDIX 2: SMPLE PUC CERTIFICATES

APPENDIX 3: LABOUR LICENSE

APPENDIX 4: SCANNED COPY OF CONTRACTOR’S INSURANCE FOR WORKERS and ALL RISK POLICY

APPENDIX 5: NOC RECEIVED and APPLICATION TREE FELLING NOC

APPENDIX 5A:CONSENT TO ESTABLISH (CTE) FOR WATER TREATMENT PLANT AND BATCHING PLANT

APPENDIX 6: NOC FROM SUNDATBAN BIOSPHERE RESERVE

APPENDIX 6A: NOC – EAST KOLKATA WETLAND MANAGEMENT AUTHORITY (EKWMA) FOR LAYING OF PIPE

APPENDIX 7: SITE ENVIRONMENT MANAGEMENT PLAN – SAMPLE

APPENDIX 8: SAMPLE SITE INSPECTION REPORT

APPENDIX 8A: SAMPLE MONTHLY ENVIRONMENTAL COMPLIANCE MONITORING RECORDS

APPENDIX 9: SAMPLE EMP IMPLEMENTATION CHECKLIST

APPENDIX 10: MONITORING RESULTS

APPENDIX 11: FIELD LEVELTRAININGS CONDUCTED DURING REPORTING PERIOD

APPENDIX 12:  SAMPLE ACCIDENT / FIRST AID RECORD

APPENDIX 13:  TIE UP LETTER WITH HOSPITAL AND MEDICAL HEALTH  CHECKUP FOR WORKERS

APPENDIX 14: GRC NOTIFICATIONS AND COMMITTEE

APPENDIX 15: SAMPLE GRIEVANCE REGISTRATION FORM

APPENDIX 16: LIST OF GRIEVANCES (November 2019 to April.2020)

APPENDIX 17: PUBLIC CONSULTATION AT FIELD LEVEL

APPENDIX 18: DETAIL OF THE ENVIRONMENT SAFEGUARD MEETING CONDUCTED BY PMC

APPENDIX 19: FOCUS GROUP DISCUSSION – ENVIRONMENT & SOCIAL

APPENDIX 20: LAND POSSESSION STATUS

ABBREVATIONS

| AC | - | Asbestos Cement |
| --- | --- | --- |
| ADB | - | Asian Development Bank |
| BOD | - | Biochemical Oxygen Demand |
| COD | - | Chemical Oxygen Demand |
| CPCB | - | Central Pollution Control Board |
| CTE | - | Consent to Establish |
| CTO | - | Consent to Operate |
| DBO | - | Design Build Operate |
| DSC | - | District Steering Committee |
| DG | - | Diesel Generator |
| DO | - | Dissolved Oxygen |
| DSISC | - | Design, Supervision and Institutional Support Consultants |
| EA | - | Executing Agency |
| EARF | - | Environmental Assessment and Review Framework |
| EHS | - | Environment, Health & Safety |
| EIA | - | Environmental Impact Assessment |
| EMP | - | Environmental Management Plan |
| EMR | - | Environment Monitoring Report |
| ESSR | - | Environment and Social Safeguard Unit |
| GESI | - | Gender Equality and Social Inclusion |
| GLSR | - | Ground Level Storage Reservoir |
| GRC | - | Grievance Redressal Committee |
| GRM | - | Grievance Redress Mechanism |
| HDPE |  | High Density Poly ethylene |
| HSGO | - | Head Safeguards Gender Officer |
| IEE | - | Initial Environmental Examination |
| LOA | - | Letter of Acceptance |
| LPG | - | Liquefied Petroleum Gas |
| MoEFCC | - | Ministry of Environment and Forest & Climate Change, Government of India |
| NGO | - | Non-Government Organization |
| NTP | - | Notice to Proceed |
| OHR | - | Overhead Reservoir |
| O&M | - | Operation and Maintenance |
| PHED | - | Public Health Engineering Department |
| PIU | - | Project Implementation Unit |
| PMC | - | Project Management Consultant |
| PMU | - | Project Management Unit |
| PMx | - | Particulate Matter with size x micron |
| PUC |  | Pollution Under Control |
| RF | - | Resettlement Framework |
| RP | - | Resettlement Plan |
| SBR | - | Sundarban Biosphere Reserve |
| SEMP | - | Site Environment Management Plan |
| SGS | - | Safeguard and Gender Cell |
| SPS | - | Safeguard Policy Statement |
| TMP | - | Traffic Management Plan |
| USD | - | US Dollar |
| WBDWSIP | - | West Bengal Drinking Water Sector Improvement Project |
| WBPCB | - | West Bengal Pollution Control Board |
| WTP | - | Water Treatment Plant |

# INTRODUCTION

- Background – Overall Project Description and Objective

- Public Health Engineering Department (PHED), Govt of West Bengal (the Executing Agency) through its Project Management Unit and Project Implementing Unit is implementing the “West Bengal Drinking Water Sector Improvement Project” (WBDWSIP).  The Loan for WBDWSIP (ADB Loan 3696- IND) was signed on 3rd October 2018 between Government of India and the Asian Development Bank. Government of India has agreed to make the proceeds of the loan available to the Government of West Bengal upon terms and conditions satisfactory to the Asian Development Bank.

- WBDWSIP will be implemented over years between 2018 and 2024. Implementation of individual package works for 3 years after award of contract and Operation & Maintenance of 2 years after construction. In order to provide safe and sustainable drinking water as per the standards set by the Government of India, the Government of West Bengal through Government of India has requested Asian Development Bank for financing to create infrastructure and strengthen institutions of the PHED and the institutions of the identified districts, blocks, and villages for piped water supply. The selected areas identified are affected with arsenic, fluoride, and salinity in the districts of Bankura, North 24 Parganas and Purba Medinipur of West Bengal where around 1.65 million people are residing.

- The project adopted a sector approach. All subprojects selected and proposed for funding adhering to the agreed subproject selection criteria for the project. The subprojects proposed under the project stem from a district-wide comprehensive water quality and sustainability planning and completion of the Drinking Water Quality Action Plan for the concerned district. The Drinking Water Quality Action Plan for the project districts supported by the project were prepared by the executing Contractor, the Public Health Engineering Department of the Government of West Bengal, with support of project preparatory consultants from the Asian Development Bank, and has been adopted by Public Health and Engineering Department to guide present and future drinking water improvement in the districts.

- Total WBDWSIP investment envisaged is USD 349 million and based on 70% ADB financing. ADB’s loan would be USD (240+3) million ADB funding (Ordinary Capital Resources +Trust Fund) and 30% counterpart funding by Government of West Bengal would be around USD 106 million. The implementation period of WBDWSIP is from 2018 to 2024.

- The impact of the project would ensure drinking water security in West Bengal as envisioned in Vision 2020 and national sub-mission on Arsenic and Fluoride to have an outcome of safe, sustainable, and inclusive drinking water service received in project districts. The outputs of the project are,

- Output 1: Climate resilient drinking water infrastructure constructed.

- Output 2: Institutions and capacity of stakeholders for drinking water service delivery strengthened

- The following figures show the area covered under WBDWSIP works and the works proposed under WBDWSIP.

.

Figure 1: Arsenic, Fluoride and Salinity affected area

Figure 2: Location of District under the project

Figure 3: Location of different Blocks under South and North 24 pgs

Figure 4A: Location of Two Work (Indpur & Taldangra) Blocks under Bankura District

Figure 4B: Location of Two Work Blocks (Mejhia & Gangajalghati) under Bankura District

Figure 5: Water Supply infrastructure planned in East Medinipur District

- Environmental category as per ADB Safeguard Policy Statement, 2009

- Sub Projects under WBDWSIP has been classified by ADB as environmental assessment Category B (some negative impacts but less significant than category A) and the impacts of subprojects were assessed through Initial Environmental Examination (IEE), prepared according to ADB Safeguard Policy (SPS 2009).

- Environmental category of each subproject as per national laws and regulations

- None of the 10 subprojects (civil work contracts) under WBDWSIP are included in the list of projects requiring Environmental Clearance, therefore EIA and Environmental Clearance is not required. Under National rules no categorization is considered for water supply project.

- Project Safeguards Team

- Environment safeguard team for the project is given in Table 1.

Table 1: Project Safeguard Team

| Name | Designation/Office | Email Address | Contact Number |
| --- | --- | --- | --- |
| 1. PMU |  |  |  |
| Mr. Tanay Banerjee | Executive Engineer, Mechanical / Electrical Head Safeguards Gender Officer (HSGO), Safeguard Gender Cell (SGC) | eeme.pmu.adb@wbphed.gov.in | 9433484589 |
| 2. PIUs |  |  |  |
| North 24 Parganas |  |  |  |
| Mr. Sourav Bose | AE-III Safeguard Officer (SO), Environmental and Social Safeguard Unit (ESSU) | ae3.piun24p.adb@wbphed.gov.in | 9434468635 |
| Bankura |  |  |  |
| Mr. Dhiraj Mondal | A.E. – I Safeguard Officer (SO), Environmental and Social Safeguard Unit (ESSU) | ae2.piubankura.adb@wbphed.gov.in | 9038517645 |
| East Medinipur |  |  |  |
| Mr. Tapash Bera | AE-I Environment Officer (EO), Environmental and Social Safeguard Unit (ESSU) | tapasberaphe@gmail.com | 8597245053 |
| 3. Consultants |  |  |  |
| PMC |  |  |  |
| Dr. Ardhendu Mitra | Environment Specialist | ardhendumitra@gmail.com | 9830415953 |
| DSISC |  |  |  |
| North 24 pgs |  |  |  |
| Mr. Swarnabha Bandyopadhyay | Env. Specialist | swarnabhab@gmail.com | 8017668861 |
| Mr. Rohan Kumar | Support Environment Safeguard, - Junior Env. Scientist | rohangbu2011@gmail.com | 7364804746 |
| Bankura |  |  |  |
| Dr. Bidisha Mukherjee | Environmental Specialist | bidisha1.1983@gmail.com | 9830592875 |
| Mr. Chandan Kr. Majee | Support Environment Safeguard | majee.dgp54@gmail.com | 8918611629 |
| East Medinipur |  |  |  |
| Mr. Partho Sarathi Mukherjee | Environmental Specialist | parthosarathi05@rediffmail.com | 7003881499 |
| Mr. Arpan Shome | Support Environmental Safeguard | arpan.shome7@gmail.com | 8017925978 |

- Safeguard management system for the project is shown below.

Note: PMU – Project Management Unit; PMC – Project Management Consultant;

DSISC – Design, Supervision and Institutional Support Consultant

Figure 6: Safeguard Management system

- Overall project and sub-project progress and status

- There are 10 work sub project packages under WBDWSIP. Summary of work contracts including type of contract (upto 30th April, 2020) is shown below.

| Parameters | DBO Type Contract | Item Rate Contract |
| --- | --- | --- |
| Total Number of Packages | 4 | 6 |
| Contracts Executed/Completed | 0 | 0 |
| Contracts under Implementation | 4 | 6 |
| Bidding under Progress | 0 | 0 |
| Planning and Design | 0 | 0 |

- Other than that, of 10 civil work packages, there are 2 small packages of $3 million as grant from the Japan Fund for Poverty Reduction. Package (JFPR). They are (i) SAN/01: Pilot Faecal Sludge and Septage Management Plant and (ii) STWM/01: Smart Water Management: Gram Panchayat level smart water management equipment These 2 packages are under Smart Water Management for “Institutions and capacity of stakeholders for drinking water service delivery strengthened”. Till report period package SAN/01 has been awarded. Study location of the said package is not yet finalized. Preparation of IEE will be done after selection of project construction locations and finalization of scope.

- Table 2 shows the sub projects and the work packages including the status of award of contracts as on 30th April, 2020. The contract agreements for all 10 packages have been signed and project work has started for 3 packages at North 24 pgs (N-24P/01, N-24P/02A & N-24P/02B) and 4 packages at Bankura (BK/02A, BK/02B, BK/03 and BK/04). Preliminary activities are started for one package at Bankura (BK/01) and one package at East Medinipur (EM/02). Work will be started shortly for the package EM/01 at East Medinipur.

Table 2: Summary status of Subprojects under WBDWSIP (on 30th April, 2020)

| Sr. No. | Package No. | Packages | Status |
| --- | --- | --- | --- |
| 1 | WBDWSIP/DWW/ N-24P/NCB/01/2017-18 | Design, Construction and Operation of Water Treatment Plant, Reservoirs, Transmission Mains and Pumping Stations works in Haroa, Rajarhat, and Bhangar II. | Tendering process completed. Agreement done on 14.12.2018 NTP issued on 11.01.2019 |
| 2 | WBDWSIP/DWW/N-24P/ NCB/02A/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Haroa Block. | Tendering process completed. Agreement done on 11.10.2018 NTP issued on 05.12.2018 |
| 3 | WBDWSIP/DWW/NCB/ N-24P/02B/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Bhangar II Block. | Tendering process completed. Agreement done on 11.10.2018 NTP issued on 05.12.2018 |
| 4 | WBDWSIP/DWW/NCB/ BK/01/2017-18 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Reservoir, Transmission Main for Indpur and Taldangra block in Bankura. | Tendering process completed. Agreement done on 22.01.2019 NTP issued on 24.01.2019 |
| 5 | WBDWSIP/DWW/NCB/ BK/02A/2018-19 | Design and Construction of Intermediate Pumping Station, ground storage reservoirs, overhead reservoirs, water distribution network and metering works in Indpur block. | Tendering process completed. Agreement done on 10.04.2019 NTP issued on 27.05.2019 |
| 6 | WBDWSIP/DWW/NCB/ BK/02B/2018-19 | Design and Construction of Intermediate Pumping Station, Secondary transmission mains, overhead reservoirs including water distribution network and metering works in Taldangra Block. | Tendering process completed. Agreement done on 10.04.2019 NTP issued on 27.05.2019 |
| 7 | WBDWSIP/DWW/NCB/ BK/03/2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Mejhia and Gangajalghati Block in Bankura. | Tendering process completed. Agreement done on 10.04.2019 NTP issued on 27.05.2019 |
| 8 | WBDWSIP/DWW/NCB/ BK/04/2018-19 | Design and Construction of Overhead Reservoir including Water Supply Distribution Network and Metering Works in Mejhia and Gangajalghati Blocks including Rehabilitation of Existing Schemes. | Tendering process completed. Agreement done on 10.04.2019 NTP issued on 27.05.2019 |
| 9 | WBDWSIP/DWW/ICB/ EM/01/2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Nandakumar, Chandpur, Nandigram-I and II blocks in East Medinipur. | Tendering process completed. Agreement done on 03.02.2020 NTP issued on 19.02.2020 |
| 10 | WBDWSIP/DWW/ICB/ EM/02/2018-19 | Construction of Intermediate Pumping Station, Secondary transmission mains, overhead tanks including water distribution network and metering works in Nandigram-I and Nandigram-II block in East Medinipur. | Tendering process completed. Agreement done on 24.10.2019 NTP issued on 07.11.2019. |
| 11 | SAN/01 | Pilot Faecal Sludge and Septage Management Plant under JFPR (small package) | Contract awarded NTP issued on 06.12.2019 |
| 12 | STWM/01 | Smart Water Management)/ Gram Panchayat level smart water management equipment under JFPR (multiple packages) | Not yet finalized |

- For all awarded packages, contractors have been mobilized and works are in different stages of implementation. Photo illustration of project activities including training and consultations as well as good work practices are shown in Appendix 1.

- Description of subprojects (package-wise) and status of implementation

- Table 3 below shows implementation status of awarded sub project packages upto April 2020. List of submitted zone-wise SEMPs for the packages where final design has been completed are furnished in Table 3A.

Table 3: Status of Implementation of Sub project (30th April 2020)

(Suspension of construction activities for all packages due to COVID-19 Pandemic from 23rd March 2020. Govt. notice attached below the table. Resumption of work will be as per Govt. order and protocol of COVID 19 management in ref. to SOP of the project)

| Package Number | Components/List of Works | Status of Implementation (Preliminary Design/Detailed Design/On-going Construction/Completed/O&M) | Contract Status (specify if under bidding or contract awarded) | If On-going Construction |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | %Physical Progress | Expected Completion Date |
| WBDWSIP/DWW/ N-24P/NCB/01/2017-18 (Awarded to FFIL -RIL JV) | Design, Construction and Operation of Water Treatment Plant, Reservoirs, Transmission Mains and Pumping Stations works in Haroa, Rajarhat, and Bhangar II. - Construction of WTP of 100 MLD (22 MGD) within existing WTP complex at New Town, Rajarhat - Clear water transmission main of 4.9 km (1200 mm DI pipe) from WTP to clear water reservoir at booster pumping station - Intermediate booster pumping station cum 1 GLSR of capacity 4600 cum at Rajarhat, within existing pumping station campus - Boosting station cum GLSRs of capacity 5000 cum and 3200 cum at Bhangar II and Haroa respectively - SCADA Smart water management | Design: Hydraulic design, Soil investigation, design of chemical house, Pile design for Administrative building, Inlet well, Parshall flume, flash mixer and flocculator. Superstructure of flocculator, pile design of Plate settler, MCC building, GA of CWR, pile caps for inlet well, parshall flume, flash mixer and flocculator. Construction: Piling work at inlet well, parshall flume, flash mixer and administrative building. Piling work at plate settler, pile testing, pile cap of Inlet well, Parshall flume, flash mixer & flocculator. | Contract awarded | 1.9 % | 10.01.2022 + O & M period* |
| WBDWSIP/DWW/N-24P/NCB/02A/2017-18 (Awarded to NCC Ltd) | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Haroa Block. Construction of overhead storage reservoir 21 nos. (capacity 250 to 1000 m3) - Distribution network 851 km DI and HDPE pipes of dia 63 – 400 - Household service connection. No.. of household approx. -60000 - SCADA Smart water management | Detailed Design zone wise continued and construction On-going Design: Design of distribution network of 17 zones, OHR survey & layout plan of 8 zones, boundary wall of 14 zones, OHR foundation design of 16 zones and superstructure of 9 zones, Chlorination cum SCADA building GA & Structural design of 8 zones, Super structure design of 18 zones Construction: Pipe laying work at 16 zones – 1 to 6, 10, 12 to 19, 21 - ongoing, Foundation of boundary walls at 6 zones -1,2,12,14,17,18- ongoing, OHR construction at 8 zones – 1,2,12,14,15,17,18,21 - ongoing, bamboo walling. Distribution pipe laid: 503.667 km out of 851 km Overhead Reservoir (OHR) construction stage: Zone-1: 1st lift of column for 4th Bracing work in progress Zone-2: Vertical load test and boundary wall work are in progress. Zone-12: 1st lift of column for 2nd Bracing work and boundary wall construction are in progress. Zone-14: Vertical/Lateral load test completed. Boundary wall construction in progress at PCC stage Zone-15: 1st lift of column for 4th Bracing work in progress. Zone-17: 1st lift of column for 3rd Bracing work in progress Zone-18: concreting 2nd Brace is completed. Zone-21: Pile work is in progress. | Contract awarded | 19.0 % | 04.12.2021 + O & M period* |
| WBDWSIP/DWW/NCB/ N-24P/02B/2017-18 (Awarded to NCC Ltd) | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Bhangar II Block - Construction of overhead storage reservoir 18 nos... .(capacity 300 to 1000 m3) - Distribution network 910 km HDPE - Household service connection No of household approx. -60000 - SCADA Smart water management | Detailed Design zone wise continued and construction On-going Design: Design of distribution network of 13 zones, OHR survey & layout plan of 8 zones, boundary wall of 4 zones, OHR foundation design of 4 zones and superstructure of 4 zones, Chlorination cum SCADA building Structural design of 1 zone Construction: Pipe laying work at 13 zones - 1,3,4,7 to 16 -ongoing, Foundation of boundary walls at 4 zones -4,10,12,13 - ongoing, OHR construction at 4 zones- 4,10,12, 13 - ongoing. Distribution pipe laid: 457.0 km out of 910.0 km Overhead Reservoir (OHR) construction stage: Zone – 4: 1st lift wall of Tank complete and Centering, shuttering for 2nd lift in progress Zone – 10: Centering and shuttering complete for casting of 2nd bracing Zone – 12: Centering and shuttering for 2nd bracing work in progress Zone – 13: Centering and shuttering for 5th bracing work in progress | Contract awarded | 17.1 % | 04.12.2021 + O & M period* |
| WBDWSIP/DWW/NCB/ BK/01/2017-18 (Awarded to Technofab Engineering and SN Envirotech JV) | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Reservoir, Transmission Main for Indpur and Taldangra block in Bankura. - Construction of Intake of 44MLD - Raw water pumping main from intake to WTP of approx. 3 km - Construction of conventional WTP of 44 MLD at Khatra block - Primary clear water rising main of 21 km - SCADA Smart water management | Detailed Design and Preliminary activity continued - Finalization of Intake Well location done. - Bathymetric Survey completed for construction of intake well. - Setting of site office at WTP area started. Setting of laboratory yet to be started. - Sample for raw water analysis has given for testing. - Soil investigation work started for new location of WTP. It is completed for GLSR area. - Resistivity survey completed. - Topographical Survey completed. - Hydraulic Design calculation – commented. - Hydraulic flow diagram – commented - Layout plan for new WTP area – commented All Design under revision and response from contractor awaited | Contract awarded | 0.0% | 05.02.2022 + O & M period* |
| WBDWSIP/DWW/NCB/ BK/02A/2018-19 (Awarded to L & T Ltd) | Design and Construction of Intermediate Pumping Station, ground storage reservoirs, overhead reservoirs, water distribution network and metering works in Indpur block. - Construction of Intermediate booster pumping station at Gobindapur in Indpur block - Construction of Intermediate booster pumping station cum Ground level storage reservoirs (IBPS cum GLSR-II- 1400 KL) at Raghunathpur in Indpur block - Laying of transmission main of approx. 155.48 km - Construction of 19 overhead reservoirs (OHRs) and associated works within the OHR premises - Water supply distribution network of approx. 757 km - Providing of Household service connection | Detailed Design zone wise continued and construction On-going - Site office and laboratory yet to be set up as per requirement. - Topographical survey completed. - Geotechnical survey completed for 18 No. of zones. - Design and drawing of clear water transmission main completed and approved. - Design and drawing of distribution network approved for 8 zones and rest under review. - Layout plan of OHSR has approved for 4 zones. - Clear water transmission main pipe laying started at 9 zones; out of which 6 are under BK 02A (zone 1, 12, 13, 18, 19, 20) and 3 under BK 02B (Zone 16, 20 and 21). Total 17.0465 km laid (out of total 155.48 km of transmission main). - Distribution main pipe laying started at 3 zones (Zone 1, 13 and 20). Total 2.830 km laid (out of total 757 km of distribution network). | Contract awarded | 4.56% | 26.05.2022+ O & M period* |
| WBDWSIP/DWW/NCB/ BK/02B/2018-19 (Awarded to L & T Ltd) | Design and Construction of Intermediate Pumping Station, Secondary transmission mains, overhead reservoirs including water distribution network and metering works in Taldangra Block. - Intermediate booster pumping station cum ground level storage reservoir (GLSR of capacity 1500 KL) at Chenchurya - Design of Transmission mains including laying of (rising) mains of approx. 77.28 km. - Water supply distribution network of approx. 1091 km at 24 zones - 24 Overhead reservoirs (OHR) and associated works within the OHR premises in Taldangra Block - Providing of Household service connection | Detailed Design zone wise continued and construction On-going - Site office yet to be set up. Laboratory established at Taldangra. - Topography survey completed. - Geotechnical survey completed for 21 No. of zones - Design and drawing of clear water transmission main completed and approved. - Design and drawing of distribution main submitted approved for 5 zones, rest under review. - Layout plan of OHSR approved for 16 zones and also for 1 IBPS. - Construction of overhead reservoir (OHR) started and ongoing at 7 Zones i.e. 1, 3, 4, 10,11, 15,18. - Distribution main pipe laying started at 3 zones (1, 3 and 15). Total 4.66 km laid (out of total 1091.35 km). | Contract awarded | 4.20 % | 26.05.2022+ O & M period* |
| WBDWSIP/DWW/NCB/BK/03/2018-19 (Awarded to L & T Ltd) | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Mejhia and Gangajalghati Block in Bankura. - Raw water intake of 36 MLD capacity will be constructed at Nutangram mouza of Barjora Block with arrangement for housing pumping machineries - Raw water transmission Main of 16.4 Km will be laid from Intake to Proposed WTP at Basudebpur mouza of Gangajalghati Block. - Water Treatment Plant (WTP) of 36 Mld capacity with booster pumping facilities upto design period 2050 would constructed at Basudebpur mouza of Gangajalghati Block. - Clear Water Main of approx. 146 km will be laid from WTP to 34 nos. OHT of Mejhia & Gangajaghati Block | Detailed Design and Construction work continued - Site office established at WTP area. - Bathymetric Survey completed for construction of intake well. - Topographical Survey completed. - Geotechnical survey completed for Intake well & WTP. - Field test for clear water Raising Main in progress. - Pipeline route survey- Submitted by the contractor - Well Kerb work in progress at intake well. - Boundary wall construction in progress at WTP. - Raw (0.247 km) and clear water transmission main (22.649 km) pipe line laid upto 22.896 km out of total 162 km. | Contract awarded | 12.0% | 26.05.2022+ O & M period* |
| WBDWSIP/DWW/NCB/ BK/04/2018-19 (Awarded to L & T Ltd) | Design and Construction of Overhead Reservoir including Water Supply Distribution Network and Metering Works in Mejhia and Gangajalghati Blocks including Rehabilitation of Existing Schemes - Construction of 30 overhead reservoirs (OHRs) in Mejia and Gangajaghati Blocks - Laying of approx. 1400 km distribution network - Provision of domestic water meters for household water connections with water meters. | Detailed Design zone wise continued and construction On-going - Site office established at Gangajalghati. - Topographical survey completed - Geotechnical survey completed for all 34 reservoir sites. - Household survey completed for 15 zones., rest, under progress. - Layout plan of OHSR completed for 14 No. of zones. - Construction of overhead reservoir (OHR) started and ongoing at 11 Zones i.e. 2, 4, 5, 7, 9, 11, 15, 19, 20, 27 and 28. Distribution main pipe laying started at 2 zones (zone 5 and 20) only. Total 9.869 km laid (out of 1400 km). | Contract awarded | 11.0% | 26.05.2022+ O & M period* |
| WBDWSIP/DWW/ICB/ EM/01/2018-19 (Awarded to L & T Ltd) | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Nandakumar, Chandpur, Nandigram-I and II blocks in East Medinipur. - Construction of Intake system - Raw water rising main approx. 18 km - Water treatment plant capacity 112 MLD - Clear water rising main approx. 133 km - Pipe Supporting RCC Bridge | Detailed design and Preliminary activity continued. - Topographical survey of Raw water rising main is going on. - Topographical survey of Clear water rising main (WTP to Nandigram I & II) is going on | Contract awarded | 0.0% | 05.03.2023 + O & M period* |
| WBDWSIP/DWW/ICB/ EM/02/2018-19 (Awarded to L & T Ltd) | Construction of Intermediate Pumping Station, Secondary transmission mains, overhead tanks including water distribution network and metering works in Nandigram-I and Nandigram-II block in East Medinipur. - 2 nos. IBPS – GLSR of capacity 4300 and 2500 KL at Nandigram I and II respectively - 20 new overhead reservoirs (OHRs) in Nandigram-I block and 9 new overhead reservoirs in Nandigram II block - Approx. 1707.7 km of distribution network: 651.7 km in Nandigram II and 1056 km in Nandigram I. - Providing domestic water meters | Detailed Design and Preliminary activity continued - Topography survey is under process - Distribution network design for all 9 Zones of Nandigram II & 4 zones of Nandigram I has been partially approved - 19 OHSR location field soil investigation work has been completed | Contract awarded | 0.0% | 07.11. 2022 + O & M period* |

Official order from Government of West Bengal for Suspension of Work due to COVID-19

Table 3A: Zone-wise Submitted SEMP Status for North 24 Parganas, Bankura and East Medinipur Packages

(Disclosed or to be disclosed in WBDWSIP website)

| North 24 Pgs: Package N24P/01 – Water Treatment Plant |  |  |  |
| --- | --- | --- | --- |
| Zone | Description | Location | Submitted/ Disclosed SEMP* |
| 1 | Water Treatment Plant | Rajarhat, New Town | August 2019 |
| North 24 Pgs : Packageg-N24P/02A:– HAROA BLOCK |  |  |  |
| Overhead Reservoir (OHR) | Gram Panchayat | Name of Village |  |
| Zone -1 | Gopalpur-II | Amta | August 2019 |
| Zone-2 | Gopalpur -I | Nebutala Abad | February 2020 |
| Zone-8 | Khasbalanda | Bantosa | Finalized. To be disclosed in project website |
| Zone -12 | Bokjuri | Parchandpur | July 2019 |
| Zone-14 | Shalipur | Khalisadi | Jan 2020 |
| Zone -15 | Salipur | Jinkia | July 2019 |
| Zone -17 | Salipur-Borjuri | Salipur | July 2019 |
| Zone -18 | Gopalpur-I, Haroa- Gopalupur-II | Gopalpur | September 2019 |
| Zone -21 | Haroa | Haroa | Finalized. To be disclosed in project website |
| Distribution Network | Gram Panchayat | Remarks |  |
| Zone -1 | Gopalpur-II |  | August 2019 |
| Zone-2 | Gopalpur -I |  | February 2020 |
| Zone-3 | Haroa |  | December 2019 |
| Zone -4 | Kulti |  | September 2019 |
| Zone -5 | Kulti |  | July 2019 |
| Zone -6 | Kulti |  | September 2019 |
| Zone-8 | Khasbalanda |  | Finalized. To be disclosed in project website |
| Zone-10 | Sonarpur & Sankarpur |  | January 2020 |
| Zone -12 | Bokjuri |  | July 2019 |
| Zone -13 | Salipur |  | June 2019 |
| Zone -14 | Salipur |  | August 2019 |
| Zone -15 | Salipur |  | July 2019 |
| Zone-16 | Salipur |  | November 2019 |
| Zone -17 | Salipur-Borjuri |  | July 2019 |
| Zone -18 | Gopalpur-I, Haroa- Gopalupur-II |  | September 2019 |
| Zone-19 | Haroa |  | December 2019 |
| Zone -21 | Khasbalanda-Haroa |  | September 2019 |
| North 24 Pgs: Package-N24P/02B – BHANGAR II BLOCK |  |  |  |
| Overhead Reservoir (OHR) | Gram Panchayat | Name of Village |  |
| Zone -4 | Polerhat-I | Sastyangachhi | May 2019 |
| Zone -10 | Shanpukur | Shanpukuria | June 2019 |
| Zone -12 | Bhogali-II | Bankochua | July 2019 |
| Zone -13 | Bhogali-II | Baniara | May 2019 |
| Zone -15 | Chaltaberia | Kachua | Finalized. To be disclosed in project website |
| Distribution Network | Gram Panchayat | Remarks |  |
| Zone-1 | Bamanghata & Benota -II |  | January 2020 |
| Zone-3 | Benota II |  | Finalized to be disclosed in project website |
| Zone -4 | Polerhat-I |  | May 2019 |
| Zone-7 | Polerhat-I |  | December 2019 |
| Zone-8 | Shan Pukur |  | Finalized to be disclosed in project website |
| Zone -9 | Bhogali-I |  | September 2019 |
| Zone -10 | Shanpukur |  | June 2019 |
| Zone-11 | Sanpukur |  | January 2020 |
| Zone -12 | Bhogali-II |  | July 2019 |
| Zone -13 | Bhogali-II |  | May 2019 |
| Zone -14 | Bhagabanpur |  | November 2019 |
| Zone-15 | Bhagbanpur and Chaltaberia |  | November 2019 |
| Zone -16 | Chaltaberia |  | October 2019 |
| Bankura: Package BK/02A- INDPUR BLOCK |  |  |  |
| Transmission Main | Gram Panchayat | Name of Village |  |
| Zone 10 (OHR, Distribution) | Indpur | Nayekhir | February 2020 |
| Zone 12(OHR, Distribution) | Indpur | Maukuri | February 2020 |
| Zone 13(OHR, Distribution) | Gourbazar | Bholarkhap | February 2020 |
| Zone 14 (OHR, Distribution) | Gourbazar | Dumurtor | February 2020 |
| Zone 18 (OHR, Distribution) | Bheduasole | Tunamara | February 2020 |
| Zone 20 (OHR, Distribution) | Bheduasole | Golakpur | February 2020 |
| Zone 1 (OHR, Distribution) | Hatagram | Uttarkendabona | Finalized. To be disclosed in project website |
| Zone 3 (OHR, Distribution) | Hatagram | Surulia | Finalized. To be disclosed in project website |
| Zone 16 (Transmission Main) (OHR & Distribution Main under BK 2B) |  | Belasuli-Shyamsundarpur | Finalized. To be disclosed in project website |
| Zone 20 (Transmission Main) (OHR & Distribution Main under BK 2B) |  | Manipur | Finalized. To be disclosed in project website |
| Bankura: Package BK/02B- TALDANGRA BLOCK |  |  |  |
| Overhead Reservoir Zones | Gram Panchayat | Name of Village |  |
| Zone 1(OHR, Distribution) | Amdangra | Amdangra | Finalized. To be disclosed in project website |
| Zone 3 (OHR, Distribution) | Saltora | Koniara | February 2020 |
| Zone 4 (OHR, Distribution) | Satmouli | Chandabila | February 2020 |
| Zone 10 (OHR, Distribution) | Saharghati | Saharghati | OHR- November 2020 Pipeline- Finalized. To be disclosed in project website |
| Zone 11 (OHR, Distribution) | Fulmati | Manikmara | Finalized. To be disclosed in project website |
| Zone 15 (OHR, Distribution) | Taldangra | Bendalaxmanpur | February 2020 |
| Zone 18 (OHR, Distribution) | Hamasra | Hamasra | OHR – November 2019 Pipeline- Finalized. To be disclosed in project website |
| Bankura: Package BK/03 MEJIA GANGAJALGHATI BLOCK for Intake, WTP, Raw and Clear water transmission main |  |  | March 2020 |
| Bankura: Package BK/04 MEJIA-GANGAJALGHATI BLOCK |  |  |  |
| Overhead Reservoir Zones | Gram Panchayat | Name of Village |  |
| Zone 2 (OHR) | Lachmanpur | Sagariya | February 2020 |
| Zone 4(OHR) | Bhaktabundh | Ranganathpu | February 2020 |
| Zone 5 (OHR, Distribution) | Bhaktabundh | Lakhiyara | Finalized. To be disclosed in project website |
| Zone 7 (OHR, Distribution) | Pirrabani | Pirrabani | Finalized. To be disclosed in project website |
| Zone 9(OHR) | Kapistha | Kapistha | February 2020 |
| Zone 11(OHR) | Gobindadham | Machaparulia | February 2020 |
| Zone 15 (OHR, Distribution) | Lachmanpur | Lachmanpur | Finalized. To be disclosed in project website |
| Zone 19 (OHR) | Gobindadham | Gobindadham | February 2020 |
| Zone 20 (OHR, Distribution) | Banasuriya | ChotoNabagram | OHR- November 2019 Pipeline - Finalized. To be disclosed in project website |
| Zone 27 (OHR) | Ardhadgram | Mohona | OHR- November 2019 |
| Zone 28 (OHR) | Banjora | Purbatore | February 2020 |
| East Medinipur: Package EM/02 – NANDIGRAM I AND NANDIGRAM II BLOCK |  |  |  |
| Nandigram I Block |  |  |  |
| Overhead Reservoir | Gram Panchayat | Name of Village |  |
| Zone 1 | Sonachura | Gangra | March 2020 |
| Zone 5 | Samsabad | Samsabad | March 2020 |
| Zone 6 | Gokulnagar | Gokulnagar | March 2020 |
| Zone 7 | Gokulnagar | Jambari | March 2020 |
| Zone 9 | Dautpur | Marisdanga | March 2020 |
| Zone 10 | Vekutia | Durgapur | March 2020 |
| Zone 13 | Mahammadpur | Parbatipur | March 2020 |
| Zone 18 | Nandigram | Nandigram CT | March 2020 |
| Zone 19 | Haripur | Haripur | March 2020 |
| Nandigram II Block |  |  |  |
| Zone 3 | Gholpukuria | Birulia | March 2020 |
| Zone 7 | Boyal-I | Kanakpur | March 2020 |

(* Project website link - https://wbdwsip.org/safeguard-documents)

# II.	COMPLIANCE STATUS WITH NATIONAL/ STATE/ LOCAL SATATUTORY ENVIRONMENTAL REQUITREMENTS

- For implementation of the project both national and state rules & regulations need to be followed. Table 4 below indicates environment legal compliance status for the awarded projects under WBDWSIP.

- Before and during implementation of the project, compliance with environmental policy, law and legislation is necessary.

- Present status of Environment, forest and other clearances are mentioned below.

Table 4: National/State/Local Statutory Environmental Requirements

| Package No. | Subproject Name | Statutory Environmental Requirements | Status of Compliance | Validity if obtained | Action Required | Specific Conditions that will require environmental monitoring as per Environment Clearance, Consent/Permit to Establish |
| --- | --- | --- | --- | --- | --- | --- |
| WBDWSIP/DWW/ N-24P/NCB/01/2017-18 | Design, Construction and Operation of Water Treatment Plant, Reservoirs, Transmission Mains and Pumping Stations works in Haroa, Rajarhat, and Bhangar II | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | Not Applicable (NA) | None | NA |
|  |  | Water (Prevention and Control of Pollution) Act of 1974, Rules of 1975, and amendments During implementation (construction phase) compliance will be required Consent to Establish (CTE) has to be received from West Bengal Pollution Control Board (WBPCB) for new WTP | Application has been sent to West Bengal Pollution Control Board for obtaining CTE of Water Treatment Plant CTE obtained from West Bengal Pollution Control Board, attached as Appendix 5A | Validity upto 30.11.2026 | Should obtain CTO prior to operation of WTP | Ambient noise level shall not exceed permissible limit Monitoring & Compliance status – specific and general condition of CTE attached in Appendix 5A |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for diesel generator/s, Hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | No establishment of diesel generator/s, Hot mix plants, wet mix plants, stone crushers, Application send by contractor to WBPCB for establishment of Batching Plant. CTE received. Attached in Appendix 5A PUC obtained (sample attached in Appendix 2) | NA Validity upto 17.03.2027 Validity upto January 2021 | CTE & CTO will be collected as per requirement and with advancement of project Compliance of condition (CTE just received before COVID 19 Lockdown). PUC certificate will be collected for new vehicle- equipment if any. Also, renewal will be required as per validity | To be updated in next SEMR when the consent is Obtained (if required) Conditions shown in CTE of Batching plant. Monitoring of emission – liquid and gaseous is required. Compliance statement will be prepared and submit in next SEMR. NA |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | To be maintained | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. No tree felling till date | NA To be updated if any tree felling required | None Till report period no tree felling is required. Tree felling NOC will be obtained if required with advancement of work | NA |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Complied Labour licence obtained and enclosed as Appendix 3 | Valid upto 18.05.2020 | To comply with the requirements as specified in the issued license. | NA |
|  |  | Labour compensation insurance | Complied Worker compensation insurance policy obtained and attached as Appendix 4 | Valid upto 31.03.2021 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/N-24P/ NCB/02A/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Haroa Block. | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | NA | None | NA |
|  |  | Water (Prevention and Control of Pollution) Act of 1974, Rules of 1975, and amendments During implementation (construction phase) compliance will be required | Complied as per requirement and in line of rules. Protection of water body continued at project site | NA | None | NA |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project - Till report period not required Diesel Generator less than 5 kVA, - CTE & CTO not required PUC certificate has been collected (sample attached as --- Appendix 2) | NA Validity period varied from May 2020 to June 2021 | CTE to be obtained as per requirement CTO to be obtained before commencement of operations. Timely renewal will be required | To be updated in next SEMR when the consent is obtained (as per requirement) |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. Tree felling (11 nos.) NOC obtained for Gopalpur OHR site (See Appendix 5) As a compensation, 55 trees need to be planted in the site. Till date not done | NA | The replantation process is being coordinated with local forest department. Tree felling NOC for other sites if required as per final design and advancement of work | - Submission of report on completion of plantation to DFO - Maintaining of proposed plantation for five years - Assess possibility of transplanting of trees |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | Protection of land under Sundarban Biosphere Reserve (SBR) | NOC received from Director Sundarban Biosphere Reserve on 5th March 2019 indicating non requirement of any clearance from SBR authority since no forest area involved under the package (Copy attached as Appendix 6) | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Complied. Labour licence obtained and enclosed as Appendix 3 | Valid upto 04.12.2020 | To comply with the requirements as specified in the issued license. | NA |
|  |  | Labour compensation insurance | Complied. Worker compensation insurance policy obtained and attached as Appendix 4 | Valid upto 16.08.2020 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/NCB/ N-24P/02B/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Bhangar II Block. | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | NA | None | NA |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project- Till report period not required Diesel Generator less than 5 kVA, - CTE & CTO not required PUC certificate has been collected (sample attached as ---Appendix 2) | NA Validity period varied from May 2020 to June 2021 | CTE to be obtained prior to start of construction. CTO to be obtained before commencement of operations. Timely renewal will be required | To be updated in next SEMR when the consent is obtained as per requirement |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. No tree felling required till date | NA | Tree felling NOC if required as per final design and advancement of work | To be updated tree felling status and NOC in next SEMR as per requirement |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | Protection of land under Sundarban Biosphere Reserve (SBR) | NOC received from Director Sundarban Biosphere Reserve on 5th March 2019 indicating non requirement of any clearance from SBR authority since no forest area involved under the package (Copy attached as Appendix 6) | NA | No action required | NA |
|  |  | East Kolkata Wetland (Conservation and Management) Act, 2006 This legislation regulates all activities within the notified area of East Kolkata Wetlands. | Zone 1, 2, and 18 falls within EKW area hence prior permission/ NOC should be obtained from the East Kolkata Management Authority for laying water distribution pipe networks within EKW area. NOC received on 08.01.2020 for laying of pipes. Appendix 6A shows NOC from EKW Management Authority | Valid upto 31.12.2021 | OHR sites for 2 and 18 of Bhangar-II block have been relocated outside the EKWMA area. And at Zone-1, Bhangar II Distribution will be done using control valves | Site observation/ monitoring to check, - No construction of permanent structure - No filling of water body - No change in characteristics of land - No discharge of effluent, waste in the water body |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Complied Labour licence obtained and enclosed as Appendix 3 | Valid upto 26.12.2020 | To comply with the requirements as specified in the issued license. | NA |
|  |  | Labour compensation insurance | Complied Worker compensation insurance policy obtained and attached as Appendix 4 | Valid upto 16.08.2020 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/NCB/ BK/01/2017-18 & WBDWSIP/DWW/NCB/ BK/03/2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Reservoir, Transmission Main for Indpur and Taldangra block in Bankura & Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Mejhia and Gangajalghati Block in Bankura. | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | NA | None | NA |
| WBDWSIP/DWW/NCB/ BK/01/2017-18 & WBDWSIP/DWW/NCB/ BK/03/2018-19 | - DO – - DO - | Water (Prevention and Control of Pollution) Act of 1974, Rules of 1975, and amendments During implementation (construction phase) compliance will be required CTE has to be received from West Bengal Pollution Control Board (WBPCB) for new WTP | For both the packages - Application has been sent to West Bengal Pollution Control Board for getting CTE for Water Treatment Plant On November 2019 online application sent to WBPCB for WTP CTE under package BK/01 CTE for WTP obtained from WBPCB on 12.03.2020 only for package BK/03 | NA CTE validity date – 28.02.2027 | Follow up for getting CTE Should obtain CTO prior to operation of WTP | Ambient noise level shall not exceed permissible limit Monitoring & Compliance status – specific and general condition of CTE attached in Appendix 5A |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project- Till report period not required Green – Silent Diesel Generator in use. No requirement of CTE and CTO PUC certificate to be obtained for package BK/01 PUC certificate has been collected for package Bk/03 (sample attached as ---Appendix 2) | NA Validity period - June 2020 | CTE to be obtained (as per requirement) prior to start of construction. CTO to be obtained (if required) before commencement of operations. Timely renewal will be required | To be updated in next SEMR when the consent is obtained as per requirement |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved . For Package BK/03- Tree felling is required for construction of WTP. - Application send to DFO, Forest dept. Permission obtained on 05.12.2019 for felling of 232 mos. tree (Copy attached as Appendix 5), | - 18.03.2020- Expired. Tree felling was planned before lockdown by Forest Dev. Corporation, however sudden declaration of the complete lockdown it was not possible, meanwhile validity of the permission expired. Applied for renewal of the same. Tree felling will be done shortly after renewal of NOC and lifting of lockdown | Tree felling NOC for other sites if required as per final design and advancement of work Felling of trees and plantation of 1160 nos. tree as compensatory plantation | Site observation/ monitoring and recording, - Information to Forest Dept. after completion of plantation - Within a week of carrying out felling of trees submission of report to forest dept. indicating the total number of logs and firewood produced against each species - Retention of stamps of the felled trees for at least one month from date of felling - Felling & carriage of trees shall not be carried out after the sun-set and before sun-set |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | WBDWSIP/DWW/NCB/ BK/01/2017-18 Complied Labour licence obtained and enclosed as Appendix 3 WBDWSIP/DWW/NCB/ BK/03/2018-19 Complied Labour licence obtained and enclosed as Appendix 3 | Valid upto 08.06.2020 Valid upto 11.08.2020 | To comply with the requirements as specified in the issued license. | NA |
|  |  | Labour compensation insurance | WBDWSIP/DWW/NCB/ BK/01/2017-18 Complied Worker compensation insurance policy attached as Appendix 4 WBDWSIP/DWW/NCB/ BK/03/2018-19 Complied Group Insurance WC policy. Obtained and enclosed (Appendix 4) | Valid upto 29.04.2020 (Renewal under process) Valid upto 30.3.2021 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/NCB/ BK/02A/2018-19 WBDWSIP/DWW/NCB/ BK/02B/2018-19 WBDWSIP/DWW/NCB/ BK/04/2018-19 | Design and Construction of Intermediate Pumping Station, ground storage reservoirs, overhead reservoirs, water distribution network and metering works in Indpur block. Design and Construction of Intermediate Pumping Station, Secondary transmission mains, overhead reservoirs including water distribution network and metering works in Taldangra block Design and Construction of Overhead Reservoir including Water Supply Distribution Network and Metering Works in Mejhia and Gangajalghati Blocks including Rehabilitation of Existing Scheme | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed projects | NA | None | NA |
| WBDWSIP/DWW/NCB/ BK/02A/2018-19 & WBDWSIP/DWW/NCB/ BK/02B/2018-19 & WBDWSIP/DWW/NCB/ BK/04/2018-19 | - DO – - DO – - DO - | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project- Till report period not required For all the packages Green – Silent Diesel Generator in use. No requirement of CTE and CTO WBDWSIP/DWW/NCB/ BK/02A/2018-19 Received. Sample attached as Appendix 2 WBDWSIP/DWW/NCB/ BK/02B/2018-19 Received. Sample attached as Appendix 2 WBDWSIP/DWW/NCB/ BK/04/2018-19 Received. Sample attached as Appendix 2 | NA Validity period May 2020 Validity period July 2020 Validity period November 2020 | CTE to be obtained prior to start of construction as per requirement CTO to be obtained (as per requirement) before commencement of operations. Timely renewal will be required | To be updated in next SEMR when the consent is obtained as per requirement |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. No tree felling required till date for all the 3 packages | NA | Tree felling NOC for other sites if required as per final design and advancement of work | To be updated tree felling status and NOC in next SEMR as per |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Complied, Obtained for all WBDWSIP/DWW/NCB/ BK/02A/2017-18 Labour licence enclosed as Appendix 3 WBDWSIP/DWW/NCB/ BK02B/2018-19 Labour licence enclosed as Appendix 3 WBDWSIP/DWW/NCB/ BK04/2018-19 Labour licence enclosed as Appendix 3 | Valid upto 19.08.2020 Valid upto 19.08.2020 Valid upto 11.08.2020 | To comply with the requirements as specified in the issued license. | NA |
|  |  | Labour compensation insurance | Group compensation insurance policy in favor of contractors for all packages obtained (Appendix 4) | Valid upto 30.03.2021 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/ICB/ EM/01/ 2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Nandakumar, Chandpur, Nandigram-I and II blocks in East Medinipur. | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | NA | None | NA |
|  |  | Water (Prevention and Control of Pollution) Act of 1974, Rules of 1975, and amendments During implementation (construction phase) compliance will be required CTE has to be received from West Bengal Pollution Control Board (WBPCB) for new WTP | Application of CTE for WTP is delayed due to COVID19 issue and lockdown. Will be applied shortly | NA | Requirement of online application for getting CTE | NA- |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project Green- Silent Diesel Generator will be used, - CTE & CTO not required PUC certificate has been collected (sample attached as ---Appendix 2) | NA Validity period upto August 2020 | CTE to be obtained (as per requirement) prior to start of construction. CTO to be obtained (if required) before commencement of operations. Timely renewal will be required | To be updated in next SEMR when the consent is obtained as per requirement |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. No tree felling required till date | NA | Tree felling NOC for other sites if required as per final design and advancement of work | To be updated tree felling status and NOC in next SEMR as per |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Not yet obtained. Process initiated for getting Labour license before commencement of construction work | NA | Follow up | NA |
|  |  | Labour compensation insurance | obtained Group compensation insurance policy and attached as Appendix 4 | Valid upto 30.03,2021 | To ensure all potential risks are covered. | NA |
| WBDWSIP/DWW/ICB/EM02/ 2018-19 | Construction of Intermediate Pumping Station, Secondary transmission mains, overhead tanks including water distribution network and metering works in Nandigram-I and Nandigram-II block in East Medinipur. | The EIA notification, 2006 (and its subsequent amendments in 2009) provides for categorization of projects into category A and B, based on extent of impact | The sub project is not covered in the ambit of the EIA notification, as this is not covered under either Category A or Category B of the notification. As a result, the categorization, and the subsequent environmental assessment and clearance requirements, either from the state or the central Government is not triggered. Environmental Clearance is not required for the proposed project | NA | None | NA |
|  |  | Water (Prevention and Control of Pollution) Act of 1974, Rules of 1975, and amendments During implementation (construction phase) compliance will be required | Complied as per requirement and in line of rules | NA | None | NA |
|  |  | The Air (Prevention and Control of Pollution) Act 1981, amended 1987 and The Air (Prevention and Control of Pollution) Rules, 1982 During implementation (construction phase) compliance will be required CTE and CTO for (i) diesel generator/s and (ii) hot mix plants, wet mix plants, stone crushers, batching plant etc. if installed for construction. Pollution under control certificate (PUC) to be obtained by the contractor for all vehicles and equipment engaged in the project | CTE & CTO will be collected as per requirement and with advancement of project Green- Silent Diesel Generator will be used, - CTE & CTO not required PUC certificate has been collected (sample attached as ---Appendix 2) | Validity period upto April 2020 | CTE to be obtained (as per requirement) prior to start of construction. CTO to be obtained (if required) before commencement of operations. Immediate renewal is required | To be updated in next SEMR when the consent is obtained as per requirement |
|  |  | Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009 | Construction activities are being carried out in compliance with the requirements | NA | None | NA |
|  |  | The Indian Forest Act, 1927; Forest (Conservation) Act, 1980, amended 1988; Forest (Conservation) Rules, 1981 amended 1992 and 2003 West Bengal Trees (Protection and Conservation in Non- Forest Areas) Act, 2006- Tree felling permission In case of tree felling NOC needs to be obtained from Forest dept./ concerned dept. | No forest land is involved. No tree felling required till date | NA | Tree felling NOC for other sites if required as per final design and advancement of work | To be updated tree felling status and NOC in next SEMR as per requirement |
|  |  | Wild Life (Protection) Act 1972, Amendment Act, 1993 and 2002 and Wildlife (Protection) Rules, 1995 | No Wildlife protected area within or nearby the project area | NA | None | NA |
|  |  | The Ancient Monument and Archaeological Sites and Remains (Amendment and Validation) Act 2010 | No protected area within or nearby the project area. But chance find protocol will be maintained | NA | Chance find protocol to be maintained | NA |
|  |  | Labour licence under The Contract Labour (Regulation & Abolition) Act, 1970. (Central Act w.e.f. 07-09-70) | Complied. Labour licence obtained and enclosed as Appendix 3 | Valid upto 03.02.2021 | Timely renewal will be required |  |
|  |  | Labour compensation insurance | Applied for Group compensation insurance policy and attached as Appendix 4 | Valid upto 30.03.2021 | To ensure all potential risks are covered. | NA |

- Status of land clearances are given below. Details are discussed in Social safeguard monitoring report.

Table 5: Land Clearances status (Upto April 2020)

| District | Major Work components | Status |
| --- | --- | --- |
| North 24 pgs | Lands for OHRs | - Out of 39 nos. land for OHRs 31 are private lands and 8 are government land parcels. - In case of private lands, till the reporting period out of 31 land parcels, 14 numbers have been purchased and rest are under of process of purchase. - In case of 8 vested lands for the OHRs, Record of Right (ROR) or Possession Certificate has already been received for 7 land parcels. Rest one is under process for possession. |
|  | Land for GLSR | - Under package N24P/ 01, there are two GLSRs to be constructed and both on private land, purchase process of both the land parcels are under progress. |
|  | Water Treatment plant- Intermediate Booster pumping station (IBPS) | - Land for WTP and IBPS, both are Govt land have already been transferred to PHED and handed over to concerned contractor. (Appendix 20) |
| Bankura | Lands for OHRs | - Out of 76 nos. land for OHRs 58 are private lands and 18 are Government land parcels. - In case of private land, till the reporting period out of 58 land parcels, 35 numbers have already been purchased and rest are under process of purchase. - In case of 18 vested lands for the OHRs, Record of Right (ROR) or Possession Certificate has already been received for all the land parcels. |
|  | Intake – under BK01 & BK03 (Two numbers, one at Khatra Block and the other at Barjora Block) | - Permissive possession obtained from I & WD for construction of Intake well to be constructed under Khatra Block (BK 01) (Appendix 20) - Relinquishment order obtained from I & WD for the Intake to be constructed under Barjora Block (BK 03) (Appendix 20) |
|  | WTP – under BK01 & BK03 (Two numbers, one at Khatra Block and the other at Gangajalghati Block) | - Possession obtained for WTP under Package BK 03. (Appendix 20) - District level land purchase committee has completed registration process for BK 01 Possession letter obtained. (Appendix 20) |
|  | GLSR land at Govindapur (BK01) | - Cabinet approval obtained & placed before District Land Purchase Committee. |
|  | OHR & IBPS land at Raghunathpur (BK02A) | - Cabinet approval obtained & placed before District Land Purchase Committee for construction of OHR and IBPS within same plot of land |
|  | OHR & IBPS land at Taldangra (Chenchuriya)(BK02B) | - Land required for the construction of OHR & IBPS at Taldangra has been purchased by PHED., which is also located within same plot of land District level land purchase committee has completed registration process. Possession letter waited. |
| East Medinipur | Water storage reservoir lands – various stages | - Out of 29 numbers of land for OHRs all the land parcels are government lands. - In case of 29 government lands for the OHRs, Record of Right (ROR) or Possession Certificate has already been received for 26 land parcels. Rest three are under process for possession. |
|  | Intake – Electric substation | - Land Registry has been completed by land purchase committee on 17. 01.2020 and handed over to the concerned Contractor (Appendix 20). |
|  | WTP land | - Possession obtained for the land required to construct the WTP (Appendix 20) |
|  | IBPS cum GLSR (Two numbers, one each in Nandigram I and Nandigram II) | - Possession obtained for the land required to construct IBPS cum GLSR at Nandigram I. (Appendix 20) - IDT approved (Nandigram II) letter initiated to the EE, Contai division, I&WD for hand over the plot as on 18.02.2020 vide memo no 51/EE/PIU/PM. Till the reporting period, mentioned land has not been handed over. |
| Overall status Total Land – 159 nos. Government Land: Possession obtained - 45 Nos. Under Process at L&LR Dept.- 2 Nos. Relinquishment order obtained from I & WD -1 No Under process at PHED - 1 No. PHED lands - 13 Nos. Pending under EE, I&WD for Hand Over -2 Nos. Private Land: Land already Purchased -50 Nos. Placed before land purchase committee - 38 Nos. Under Process at PHED - 4 Nos. Yet to be Submitted -3 Nos. |  |  |

# III. 	COMPLIANCE STATUS WITH ENVIRONMENTAL LOAN COVENANTS

- The loan agreement for WBDWSIP was signed on 3rd October 2018 and details are available in ADB website (http://www.adb.org/projects/documents/ind-49107-006-lna). Table 6 provides a summary of compliance to the loan covenants related to environmental safeguards.

Table 6: Compliance of Loan Covenants – Environment part

| Serial no. as per loan agreement | Program Specific Covenants | Status / Issues | Action required |
| --- | --- | --- | --- |
| Environment |  |  |  |
| 6 | The Borrower shall ensure or cause the EA to ensure that the preparation, design, construction, implementation, operation and decommissioning of the Project, and all projects’ facilities comply with (a) all applicable laws and regulations of the Borrower and the State relating to environment, health, and safety; (b) the Environmental Safeguards; (c) the EARF; and (d) all measures and requirements set forth in the respective IEE and EMP, and any corrective or preventative actions set forth in a Safeguards Monitoring Report. | Under compliance Document is prepared/ or under preparation by complying all relevant State and National Laws, Safeguard Policy Statement (SPS 2009) of ADB, Environment Assessment Review Framework (EARF) Draft Initial Environmental Examination (IEE), Environment Management Plan (EMP) report prepared and approved by ADB. Updation of IEEs zone wise done for package N 24P/01, 02A and 02B; BK/02A, BK/03, BK/04. Updation under process for BK/02B, EM/02. Updation to be done for package BK/01 and EM/01 after finalization of the design. All measures as mentioned in Environment Management Plan (EMP) and Site-Specific Environment Management plan (SEMP) has been followed in running packages, which under implementation – N 24pgs/01, N 24 pgs/02A and 02B; BK/02A, 02B, 03 and 04. Corrective or preventive action plans reflected in Environment Monitoring Report and project implementation authority take care of such actions as per requirement Observed non-compliances are rectified through agreed corrective and preventive actions. | Since only few sites have been handed over, the IEE has been updated partially for those sites only. Once all the sites are handed over, the entire IEE will be updated further and sent for ADB’s approval. IEE will be updated further in case of change in project scope and location Continuation of application of SEMP as per approved IEE and during implementation of the project |
| Human and Financial Resources to Implement Safeguards Requirements |  |  |  |
| 10 | The Borrower shall make available, or cause the EA to make available, all necessary budgetary and human resources to fully implement the EMP required. | Under Compliance Budgetary provisions have been included in EMP Executive Engineer in-charge of Environment and Social Safeguard is in place in Project Management Unit heading Safeguard Monitoring Unit An Environment Specialist is in place within Project Management Consultancy (PMC) An Environmental Specialist of DSISC, for implementation of EMPs, is in place for regular monitoring to secure complete compliance. (Personnel detail shown in Table 1) | Training continued for Environment, health and safety officer of contractors for application of SEMP and necessary corrective action |
| Safeguards – Related Provisions in Bidding Documents and Works Contracts |  |  |  |
| 11. | The Borrower shall ensure, or cause the EA to ensure, that all bidding documents and contracts for Works contain provisions that require contractors to: (a) comply with the measures and requirements relevant to the contractor set forth in the IEE, the EMP, the RP and the IPP (to the extent they concern impacts on affected people during construction), and any corrective or preventative actions set out in a Safeguards Monitoring Report; (b) make available a budget for all such environmental measures; (c) provide the EA with a written notice of any unanticipated environmental risks or impacts that arise during construction, implementation or operation of the Project that were not considered in the IEE, the EMP, the RP or the IPP; (d) adequately record the condition of roads, agricultural land and other infrastructure prior to starting to transport materials and construction; and (e) fully reinstate pathways, other local infrastructure, and agricultural land to at least their pre-project condition upon the completion of construction. | Under compliance EMP, BOQ line items, reinstatement to pre-project conditions included in the contract agreement. (a) Approved IEE, EMP is attached in Bidding documents. In case of any change of scope, revised IEEs with EMP(s) will be prepared and corrective measures will be disclosed to the contractor and same will be reflected in the “Environment Monitoring Report”. Contractors done base line pre construction monitoring and survey of pre- works condition. Budget has been allocated for EMP application and monitoring. Pre construction documents submitted by contractors for all the packages except EM/01 and BK/01, where actual physical construction started. Partly documents submitted for BK/01 and EM/01. Also pre construction monitoring done for all the packages except EM/01. Monitoring will be done before commencement of actual physical work. (b) IEE includes budgetary provisions for implementation of EMP for all the packages. Contractors submit monitoring budget in the SEMP (c) During implementation of any project if additional impacts/risks arise due to change in scope/area that will be reflected in the revised IEEs, EMPs and Environment Monitoring Report and accordingly project Executing Contractor will inform the Construction Contractor for taking relevant corrective measures. Till date no as such situation create for packages which under implementation. (d) Haul roads have marked properly (by avoiding residential and agricultural land) before commencement of transportation of materials. (e) Pathways, and land which are affected for varying periods during implementation of the sub project restored by concerned construction Contractor before acceptance of the work. Restoration status has reflected in post construction monitoring report. Since the pipe laying work continued in villages, no as such underground infrastructure noted | In case of any change of scope, revised IEEs with EMP(s) will be prepared and corrective measures will be disclosed to the contractor It is pertinent to note here that in view of COVID-19 pandemic, work at all sites has been stopped since 3rd week of March 2020 due to nation-wide lockdown. Once, work resumes post lockdown, project health and safety plan will need to be updated to minimize risk from corona virus disease (COVID-19) at the facilities and work sites of WBDWSIP In this context, it may be noted that OHR site for Zone- 2 and 18 of Bhangar-II block has been relocated outside the East Kolkata Wetland Management Authority (EKWMA) area. At Zone-1, Bhangar II distribution of water will be done suing control valves However, NOC for pipe laying work in these locations has been obtained from EKWMA (Appendix 6A) Presently, pipeline trenches are filled up and initial road restorations are completed on the same day. Whenever incomplete restoration is observed appropriate corrective and preventive actions are taken. |
| Safeguards Monitoring and Reporting |  |  |  |
| 12 | The Borrower shall cause the EA to do the following: - submit semi-annual Safeguards Monitoring Reports to ADB and disclose relevant information from such reports to affected persons promptly upon submission; - if any unanticipated environmental and/or social risks and impacts arise during construction, implementation or operation of the Project that were not considered in the IEEs, the EMPs, promptly inform ADB of the occurrence of such risks or impacts, with detailed description of the event and proposed corrective action plan; and (c) report any breach of compliance with the measures and requirements set forth in the EMPs, promptly after becoming aware of the breach. | Under compliance (a) Last semi-annual environment monitoring report submitted for the period May 2019- October 2019. Disclosure of SEMR at ADB and project website – done Preparation of semi-annual environment monitoring report started for the period November 2019- April 2020 which to be submitted in July 2020 (b) During implementation of any project, if additional impacts/risks arise due to change in scope/area, those will be reflected in revised IEEs with EMPs and accordingly Executing Contractor (EA) will inform the ADB such change along with corrective action plan which will be reflected in the subsequent Monitoring Reports. (c) in case of any breach of compliance with the measures and requirements set forth in the EMP, EA will promptly inform ADB and suitable corrective action program will be planned/initiated. Till date no as such breach of compliance | Semi Annual Environment Monitoring Report will be submitted as per schedule IEE/ EMP will be revised in case of inclusion of additional impact and change in location/ scope for the project |
| Prohibited List of Investments |  |  |  |
| 13 | The Borrower shall ensure or cause the State to ensure that no proceeds of the Loan are used to finance any activity included in the list of prohibited investment activities provided in Appendix 5 of the SPS. | Complied There is no violation of prohibited investment activities as per ADB SPS (2009) Appendix 5. |  |
| Labour Standards |  |  |  |
| 14 | The EA shall ensure that civil works contracts under the Project follow all applicable labourer laws of the Borrower and the State, and that these further include provisions to the effect that contractors: (a) carry out HIV/AIDS awareness programs for labourer and disseminate information at worksites on risks of sexually transmitted diseases and HIV/AIDS as part of health and safety measures for those employed during construction; and (b) follow and implement all statutory provisions on labourer (including not employing or using children as labourer, equal pay for equal work), health, safety, welfare, sanitation, and working conditions. Such contracts will also include clauses for termination in case of any breach of the stated provisions by the contractors. | Under compliance Provision are included (as per EMP & BID document) to carry out HIV/AIDS awareness programs for construction contractor, application of all relevant labour laws for health and safety including child labour law and engagement of local labours (preferably from economically backward group) covering women labours. No child labour engaged in the packages Labour license and health related WC policy for workers available for all the awarded packages where construction started (Attached as Appendix 3 and 4). Submission of Labour license is required for EM/01 and renewal for package BK/01. HIV- AIDS training program conducted for package N 24 pgs/02 A and 02B (sample document attached as Appendix 11). HIV/ AIDS awareness program to be conducted shortly for other contracts Induction training continued for the awarded packages, where actual physical work started. In case of any breach of provision, necessary corrective measures as per contract clauses shall be taken. All activities including awareness program reflected in “Monitoring Report”. | HIV- AIDS training for other construction packages which awarded and where actual physical work to be started In addition, H & S training for COVID 19 pandemic to be arranged shortly |

# IV. 	COMPLIANCE STATUS WITH THE ENVIRONMENTAL MANAGEMENT PLAN

- There are 10 environment sensitive packages under implementation. Site Environment plans including site specific EMPs (SEMP) were submitted by the contractors before starting of each construction package.  SEMPs were prepared and are being prepared zone wise based on final detailed design. Appendix 7 shows sample SEMP for a zone of Bankura district work package. SEMPs for other zones (for which design completed and work started or to be started, Ref Table 3A) are available with contractor, DSISC and PIU for implementation and monitoring of application of mitigation measures. Compliance status of SEMPs are also available with DSISC and PIU and are also reflected in sample SEMP (Appendix 7). Table 7 shows package wise EMP implementation status.

Table 7:  Package wise EMP Implementation Status

| Package No | Components | Design status (Preliminary Design Stage/ Detailed Design Completed | Final IEE based on Detailed Design |  |  |  | Site specific EMP (or construction EMP) approved by Project Director (Yes/ No) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Not yet due (detailed design not yet completed) | Submitted to ADB (provide date of submission) | Disclosed on project website (provide link) | Final IEE provided to Contractors (Yes/No) |  |  |
| WBDWSIP/DWW/ N-24P/ NCB/01/2017-18 | Design, Construction and Operation of Water Treatment Plant, Reservoirs, Transmission Mains and Pumping Stations works in Haroa, Rajarhat, and Bhangar II. | DBO contract: design completed -partially. Rest undergoing (specifically transmission main pipeline) | Updated Final IEE based on upto date design prepared along with SEMP | Updated IEE submitted in December 2019 along with Log sheet | Updated IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-8) | Updated disclosed IEE provided to contractor | Prepared and approved by PIU Further updation will be done after completion of design | Nil |
| WBDWSIP/DWW/N-24P/ NCB/02A/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Haroa Block. | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Updated IEE prepared upto date design and SEMPs zone wise. | Updated IEE submitted in January 2020 along with Log sheet | Zone wise updated IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-10) | Updated disclosed IEE provided to contractor | Prepared and approved by PIU The list of SEMPs that have been approved from PIU are provided in Table 3A. | Nil |
| WBDWSIP/DWW/N-24P/ NCB/02B/2017-18 | Design and Construction of Overhead Reservoir including design, supply and laying of Water Supply Distribution Network in Bhangar II Block. | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Updated IEE prepared upto date design and SEMPs zone wise. | Updated IEE submitted in February 2020 along with Log sheet | Zone wise updated IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-11) | Updated disclosed IEE provided to contractor | Prepared and approved by PIU The list of SEMPs that have been approved by PIU are provided in Table 3A. | Nil |
| WBDWSIP/DWW/NCB/ BK/01/2017-18 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Reservoir, Transmission Main for Indpur and Taldangra block in Bankura. | For DBO contract detailed design completed - partially. Rest undergoing | Final IEE under finalization along with SEMP. Will be submitted shortly | To be submitted by July 2020 after completion of design | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-1) | Draft disclosed IEE only provided to contractor | Preparation of SEMP undergoing with development of design | Nil |
| WBDWSIP/DWW/NCB/ BK/02A/2018-19 | Design and Construction of Intermediate Pumping Station, ground storage reservoirs, overhead reservoirs, water distribution network and metering works in Indpur block., Bankura | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Updated IEE prepared upto date design and SEMPs zone wise. | Updated IEE submitted in April 2020 along with Log sheet | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-1 Zone wise updated IEE accepted by ADB, disclosure pending | Draft disclosed IEE only provided to contractor | Prepared and approved by PIU The list of SEMPs that have been approved by PIU are provided in Table 3A. | Nil |
| WBDWSIP/DWW/NCB/ BK/02B/2018-19 | Design and Construction of Intermediate Pumping Station, Secondary transmission mains, overhead reservoirs including water distribution network and metering works in Taldangra Block. | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Updated IEE under finalization. Will be submitted shortly | To be submitted by May 2020 on the basis of upto date design | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-1) | Draft disclosed IEE only provided to contractor | Prepared and approved by PIU The list of SEMPs that have been approved by PIU are provided in Table 3A. | Nil |
| WBDWSIP/DWW/NCB/ BK/03/2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Mejhia and Gangajalghati Block in Bankura | DBO contract: design completed | Updated Final IEE based on upto date design prepared along with SEMP | Updated IEE submitted on February 2020 along with Log sheet | Updated IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-9) | Updated disclosed IEE provided to contractor | Prepared and approved by PIU Updation will be required in case of any change in design | Nil |
| WBDWSIP/DWW/NCB/ BK/04/2018-19 | Design and Construction of Overhead Reservoir including Water Supply Distribution Network and Metering Works in Mejhia and Gangajalghati Blocks including Rehabilitation of Existing Schemes., Bankura | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Updated IEE prepared upto date design and SEMPs zone wise. | Updated IEE submitted in April 2020 along with Log sheet | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-1) Zone wise updated IEE accepted by ADB, disclosure pending | Draft disclosed IEE only provided to contractor | Prepared and approved by PIU The list of SEMPs that have been approved by PIU are provided in Table 3A. | Nil |
| WBDWSIP/DWW/ICB/ EM/01/2018-19 | Design, Construction and Operation-Maintenance of Raw Water Intake Well, Water Treatment Plant, Raw and Clear Water Transmission Main for Nandakumar, Chandpur, Nandigram-I and II blocks in East Medinipur. | Design is under process. Detailed design continued as per zone wise. | Final IEE will be done after finalization of design and along with approved SEMP | Expected to be submitted by August 2020 | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-7) | Draft IEE provided to the contractor | SEMP preparation is undergoing | Nil |
| WBDWSIP/DWW/ICB/ EM/02/2018-19 | Construction of Intermediate Pumping Station, Secondary transmission mains, overhead tanks including water distribution network and metering, East Mednipur | Detailed design continued zone wise. Partly completed and work ongoing (design status stated in Table 3) | Draft updated IEE under finalization | Expected to be submitted by May/June 2020 on the basis of upto date design | Draft IEE disclosed in ADB website (https://www.adb.org/projects/documents/ind-49107-006-iee-5) | Draft IEE provided to the contractor | Prepared and approved by PIU The list of SEMPs that have been approved by PIU are provided in Table 3A. | Nil |
| WBDWSIP Project website link: https://wbdwsip.org/safeguard-documents - Disclosed IEEs available |  |  |  |  |  |  |  |  |

- Implementation Arrangement

- Public Health Engineering Department (PHED) of Government of West Bengal is the executing and implementing agency of the WBDWSIP, responsible for management, coordination and execution of all activities funded under the loan. A project management unit (PMU), exclusively established in PHED, assist the PHED in implementation of WBDWSIP. PMU supported by district level Project Implementation Units (PIUs). PMU is headed by a Project Director. Each PIU is headed by a Superintending Engineer, reporting to the Project Director. PMU with the support of PIUs is responsible for planning, implementation, monitoring and supervision, and coordination of all activities under the WBDWSIP. Project Implementation Unit (PIU) for North 24 Parganas, Bankura and East Medinipur district has already been established.

- PMU is supported by Project Management Consultant (PMC) to supervise, monitor and oversee the implementation. Each PIU is supported by a Design, Supervision and Institutional Support Consultant (DSISC); there are three DSISCs supporting 3 PIUs respectively in North 24 Parganas, Bankura and East Medinipur districts. PIUs appoint construction contractors to build infrastructure.

- A Steering Committee, headed by Chief Secretary, will provide strategic guidance, and oversee the implementation of the investment project. District Steering Committee, headed by the respective District Magistrate, established for monitoring program implementation at districts level. Steering committee is already established (Appendix 14).

- Safeguard Implementation Arrangement

- Project Management Unit. A Safeguard and Gender Cell (SGC) established in PMU with the overall responsibility of ensuring compliance with ADB SPS. SGC is headed by a Head Safeguards Gender Officer (HSGO) and report to the Project Director directly. The HSGO have overall responsibility in implementation of the RF, EARF, RPs, EMPs, SEMP, GESI action plan, and appropriate monitoring and reporting responsibilities. Key environmental safeguard tasks and responsibilities at the PMU level are as follows:

  - Ensure subprojects confirms to exclusion criteria and project selection guidelines as stipulated in the EARF;

  - Approve subproject environmental category;

  - Approve IEEs; ensure that updated IEEs/EMPs reflect final project designs;

  - Ensure that EMPs are included in bidding documents and civil works contracts;

  - Ensure proper implementation of EMPs by contractors;

  - Facilitate and ensure compliance with all government rules and regulations regarding site and environmental clearances, as well as any other environmental requirements (e.g. location clearance certificates, environmental clearance certificates), as relevant;

  - Oversee public consultation and disclosure;

  - Approve quarterly EMP implementation reports;

  - Review and approve semi-annual monitoring reports prepared by PMC; and submit to ADB;

  - Oversee grievances redress process and ensure timely redress;

  - Undertake regular review of safeguards related loan covenants, and the compliance in program implementation; and

  - Organize periodic capacity building and training programs for WBDWSIP stakeholders, PHED, PMU and PIU staff on safeguards.

- The SGC supported by environmental, social and gender safeguard specialists in the PMC. Key safeguard tasks and responsibilities of Environmental Specialist of the PMC on environmental safeguards are as follows:

  - Review and finalize REA checklist and classify the project;

  - Review and confirm project selection/ design; ensure compliance with exclusion criteria and project environmental selection guidelines;

  - Review   and   finalize   IEE   reports   including   EMPs   prepared/updated   by PIUs/DSISCs;

  - Oversee public consultation and information disclosure activities; ensure timely disclosure;

  - Provide advise/support in obtaining government clearance/ approvals;

  - Review and confirm that IEEs/EMPs are included in bids and contracts;

  - Review and confirm SEMPs prepared by contractor;

  - Oversee the implementation of SEMP by contractors and ensure corrective actions, where necessary;

  - Review and approve quarterly environmental monitoring reports submitted by PIU/DSISCs;

  - Conduct site visits of project facilities and work sites to oversee implementation;

  - Prepare semi-annual environmental monitoring reports and submit to PMU SGC HSGO;

  - Oversee grievance redress process; advise on critical grievance related to environmental issues and concerns; and

  - Organize training and capacity development programs.

- Project Implementation Unit. At each PIU, an Assistant Engineer is given additional responsibilities of safeguard tasks and designated as Assistant Safeguards Officer. The Safeguards Officer oversee the safeguards implementation at PIU level, coordinate public consultations, information disclosure, regulatory clearances and approvals, RP implementation, EMP implementation and grievance redressal. Key environmental safeguard tasks and responsibilities of Safeguards Officer are as follows:

  - Coordinate public consultation and information disclosure;

  - Liaise with local offices of regulatory agencies in obtaining clearances /approvals; assist PMU for clearances obtained at state level;

  - Review and approve contractors SEMPs;

  - Oversee   day-to-day   implementation   of   SEMPs   by   contractors   including compliance with all government rules and regulations;

  - Take necessary action for obtaining rights of way;

  - Ensure continuous public consultation and awareness;

  - Coordinate grievance redress process and ensure timely actions by all parties;

  - Review monthly contractor’s SEMP Monitoring Reports;

  - Review and forward quarterly monitoring reports to PMU; and

  - Inform PMU of unanticipated impacts and formulate corrective action plan; and

  - Recommend issuance of work construction work completion certification to the contractor upon verification of satisfactory post-construction clean-up.

- The PIUs assisted by DSISC teams which includes an Environmental Specialist and a Social Safeguards Specialist. Following are the key tasks of Environmental Specialist of DSISC:

  - Assist PIU in identifying projects/components in compliance with the project exclusion criteria and selection guidelines stipulated in EARF;

  - Prepare environmental screening checklists and submit to PMU for categorization; update checklist and category as and when required to reflect project changes, and report to PMU;

  - Work closely with PIU and design teams to include environmental considerations in project location, design and technical specifications;

  - Identify statutory clearance / permissions / approvals required for subproject; assist PIU in obtaining them;

  - Assist in including standards/conditions, if any, stipulated in regulatory clearances, consents in the project design;

  - Update IEE and EMP to reflect any changes in subproject during detail design / implementation; IEE shall reflect the final project design;

  - Lead / assist PIU in public consultation in compliance with the EARF; reflect inputs from public consultation in IEEs, EMPs, and project design;

  - Advise / assist PIU in disclosing relevant information on safeguards to stakeholders, affected people etc.;

  - Assist / ensure all EMP measures related project design and location and included in the detailed designs;

  - Integrate EMP into the bid and contract documents (for DBO contracts, include full IEE including EMP in bids);

  - Advise contractor in preparation of SEMP as per the final design, prior to start of construction;

  - Ensure that all necessary clearances/permission (including those required by Contractor) are in place prior to start of construction;

  - Monitor implementation of SEMP;

  - Ensure Contractors including subcontractor’s, if any, comply with the measures set forth in the EMP;

  - Assist PIU in establishing GRM for the Project:

  - Assist PIU in grievance redress, advise the contractor on appropriate actions on grievances, ensure timely resolution and proper documentation;

  - Identify, if any, non-compliance or unanticipated impacts; initiate corrective actions, report to PMU;

  - Review and approve monthly monitoring reports submitted by Contractor; consolidate and prepare quarterly Environmental Monitoring Reports (EMR) and submit to PMU; and

  - Conduct training and capacity building activities (workshops, hands-on trainings, visits etc.,) in EMP implementation.

- Civil Works Contracts and Contractors. The contractor appoints an Environment, Health and Safety (EHS) Officer to implement EMP. The EHS Officer will update the EMP and submit a SEMP for approval of PIU.  Contractors will carry out all environmental mitigation and monitoring measures outlined in EMP, approved SEMP and their contracts. Key responsibilities of the EHS supervisor are:

  - Prepare SEMP and submit to PIU for approval prior to start of construction;

  - Conduct orientation and daily briefing sessions to workers on environment, health and safety;

  - Ensure that appropriate worker facilities are provided at the work place and labour camps as per the contractual provisions;

  - Records accidents and undertake remedial actions;

  - Implement SEMP measures and report to PIU/DSISC if any new impacts are surfaced; seek guidance from as required in EMP implementation;

  - Conduct environmental monitoring (air, noise etc.,) as per the monitoring plan

  - Ensure conduct of water quality surveillance program;

  - Prepare monthly EMP monitoring reports and submit to PIU;

  - Work closely with PIU Safeguards Officer and consultants to ensure communities are aware of project-related impacts, mitigation measures and GRM; and

  - Address any public compliance and grievances effectively and in timely manner.

- Environment Specialist and Environment support staff of DSISC visited all construction sites every week and arranged onsite training program for contractors and supervisory staff and instructed contractors for application of corrective action measures to mitigate impacts. Environment Specialist of PMC visited work sites atleast once in a month to audit application of EMP and assess shortfall if any. Table 8 shows detail of safeguard personal from contractor. Table 1 shows project safeguard Team.

Table 8: Details of Contractor’s safeguard officer for WBDWSIP

| Designation | Name and Contact Details |
| --- | --- |
| North 24 Parganas |  |
| Contractor- NCC, North 24 Pgs, Safeguard Manager | Name: Sharan Kumar Basu Phone: 7365003066 Email: sharankumarbose@gmail.com |
| Contractor- NCC, North 24 Pgs Package: N-24P/NCB/02A/2017-18, Safeguard Officer | Name: Mr. Saradindu Gain Phone: 9775459487 Email: gain.saradindu@gmail.com |
| Contractor- NCC, North 24 Pgs, Package: N-24P/NCB/02B/2017-18, Safeguard Officer | Name: Nitin Sartape Phone: 8971107720 E mail: nitinsolapur@gmail.com |
| Contractor- Furnace Fabrica (India) Ltd.- North 24 Pgs, Package: N-24P/NCB/01/2017-18, Safeguard Officer | Name: Subhankar Chattopadhya Phone: 6290016411, 9649986813 Email: shubhankar_chattopadhyay@rediffmail.com |
| Bankura |  |
| Contractor- Techno Fab Engineering Ltd. and S N Envirotech Pvt. Ltd. Jv, Bankura, Package: BK/NCB//01/2017-18, Safeguard Officer | Name: Mr. Prasanta Panda Phone: 7684097086 Email: prasantapanda7@gmail.com |
| Contractor- L & T Ltd. Package: BK/NCB/02A/2018-19, Safeguard officer | Name: Mr. Tirthraj Gupta Phone: 9939691579 Email: tirthraj@lntecc.com |
| Contractor- L & T Ltd. Package: BK/NCB/02B/2018-19, Safeguard officer | Name: Dhabaleswar Biswal Phone: 8895094563 Email: dhabaleswarbiswal@lntecc.com |
| Contractor- L & T Ltd Package: BK/NCB/03/2018-19, Safeguard officer | Name: Mr. Mukesh Kumar Phone: 8637860268 Email: k-mukesh@lntecc.com |
| Contractor- L & T Ltd. Package: BK/NCB/04/2018-19, Safeguard officer | Name: Mr. Tarak Das Phone: 9143696713 Email: tarakdas@lntecc.com |
| East Medinipur |  |
| Contractor- L & T Ltd Package: EM/ICB/01/2018-19 Safeguard officer | Safeguard officer Not Mobilized Work not yet started |
| Contractor- L & T Ltd Package: EM/ICB/02/2018-19 Safeguard officer | Name: Mr. Sujan Goswami Phone: 7837175749 Email: sujangoswami@lntecc.com |

- Figure 7 shows institutional arrangement for safeguard implementations for the project.

Figure 7: Overall Institutional Arrangement – Safeguards

- EMP implementation - Application of Mitigation measures

- Environment Specialist from DSISC and PMC carried out periodic monitoring of EMP implementation through desk review of contractor’s records as submitted monthly and site inspections. Sample site inspection report as prepared by DSISC is shown in Appendix 8. Further,  sample monthly environmental compliance monitoring with photographic evidences are presented in Appendix 8A. Package wise status of Environment Monitoring for North 24 Parganas are  shown in Table 9 (Package:N-24P/01), Table 10 (Package North 24 pgs N-24P/02A and N-24P/02B), Table 11 (Package Bankura BK/01 and BK/03) Table 12 (Package Bankura BK/02A, BK/02B and BK/04) and Table 13 (Package East Medinipur EM/02),  Contractors are regularly submitting monthly environment monitoring reports and daily and weekly checklists. Sample monthly EMP monitoring checklist is enclosed as Appendix 9.

- Preliminary work started for package BK/01 and EM/02, design and pre construction phase monitoring has been started. Survey work and final designing are being continued for package under East Medinipur PIU (EM/01). Field level monitoring as per EMP provisions will be started shortly for the package. Contractors has already complied pre construction requirements like collection of PUC certificate (Appendix 2), labour license (except EM/01) (Appendix 3), Workman Compensation Policy (Appendix 4) and also done base line monitoring (except EM/01) (Appendix 10).

Table 9: Summary of Environmental Monitoring Activities for the Package WBDWSIP/DWW/NCB/N24P/01/2017-18: BULK WATER SUPPPLY

| Impacts (List from IEE) | Mitigation Measures (List from IEE) | Parameters Monitored (As a minimum those identified in the IEE should be monitored) | Method of Monitoring | Location of Monitoring | Date of Monitoring Conducted | Name of Person Who Conducted the Monitoring | Compliance Status/ Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Design Phase |  |  |  |  |  |  |  |
| Source sustainability and efficiency | - Discontinuation of current unsafe and unsustainable groundwater sources and creating a new comprehensive surface water (river) based water supply system - Recovering wash water from treatment process to optimize the water use - Treatment and reuse of sludge from treatment process - Designing the entire system to maintain optimal flow and terminal pressure, and optimizing the overall energy usage - Reducing the incidence of water borne diseases by providing 100% population including urban poor with potable water supplies - Preparation and implementation of a water quality surveillance program including development of a laboratory as part of the project by DBO contractor to ensure that supplied water meets the drinking water standards - Development of laboratory with all necessary environment, health and safety measures and adopting international standard procedures for water quality testing - Using low-noise and energy efficient pumping systems - Installing the noise-producing pumps and motors etc., in enclosed buildings with noise reducing walls, and also maintaining adequate buffer to the nearby inhabited areas. Provision of appropriate personal protection equipment to the workers and staff | • Design philosophy • Treatment scheme • Project QA/QC plan | Document review and LOP Survey | All project locations | Before Commencement and during final design | Environment Specialist of DSISC, PIU and PMU/PMC | Detailed design under process particularly water transmission pipeline, necessary points as mentioned in IEE are considered during finalization of design |
| Chlorine handling and application risk – health and safety risk to workers and general public | - Provide the following measure at the chlorine application unit: - Chlorine neutralization pit with a lime slurry feeder - Chlorine absorption and neutralization facility - Proper ventilation, lighting, entry and exit facilities - Visible and audible alarm facilities to alert chlorine gas leak - Facility for isolation in the event of major chlorine leakage - Eye wash and shower facility - Personal protection and safety equipment for the operators in the chlorine plant (masks, oxygen cylinders, gloves, etc.,) - Provide training to the staff in safe handling and application of chlorine; this shall be included in the contract of Chlorinator supplier - Supplier of Chlorinator equipment shall provide standard operating manual for safe operation and as well as maintenance and repairs; preferably these shall be provided both in English and Bengali Languages | • Project emergency management plan • Project safety and PPE use plan • Training plan | Document review | WTP and Booster pumping site | Before Commencement and during final design | Environment Specialist of DSISC and PIU | Detail design continued. Existing authorization under “The manufacture, storage and import of hazardous chemicals rules, 1989” should be reviewed and as appropriate authorization for additional storage should be sought prior to operation. |
| Tree cutting | - Minimize removal of trees by adopting to site condition and with appropriate layout design of GLSRs - Obtain prior permission for tree cutting - Plant and maintain 5 trees for each tree that is removed | Tree felling requirement and afforestation after final design | • Checking of records • Visual inspection of sites | Project locations | Before Commencement and during final design | Environment Specialist of DSISC and PIU | As of now, tree felling requirement is not envisaged. |
| Disturbance to natural drainage | - Construction GLSR at Haroa away from the flood plain of Bidyadhari river - Integrate measures into GLSR design to avoid risk of flooding. | • Location map • Design philosophy | • Document review • Visual inspection of sites | GLSR sites | Before Commencement and during final design | Environment Specialist of DSISC, PIU and PMU/PMC | Under compliance- during final designing. GLSR sites are not yet handed over. |
| Pre-Construction Phase |  |  |  |  |  |  |  |
| Telephone lines, electric poles and wires, water lines within proposed project area | - Identify and include locations and operators of these utilities in the detailed design documents to prevent unnecessary disruption of services during construction phase; and - Require construction contractor to prepare a contingency plan to include actions to be taken in case of unintentional interruption of services. - Require contractors to prepare spoils management plan and traffic Management plan | List of affected utilities if any and operators | Observation and document checking | Specific project location | Before commencement of construction | Environment Specialist of DSISC and PIU | Under Compliance Transmission main pipe line design not yet finalized. Alignment selection continued. Spoil management and traffic management plan for pipe line laying locations will be submitted after finalization of design/ alignment |
| Conflicts with local community; disruption to traffic flow and sensitive receptors | - Prioritize areas within or nearest possible vacant space in the project location; - If it is deemed necessary to locate elsewhere, consider sites that will not promote instability and result in destruction of property, vegetation, irrigation, and drinking water supply systems. - Not to consider residential areas. - Take extreme care in selecting sites to avoid direct disposal to water body which will inconvenience the community. - For excess spoil disposal, ensure (a) site shall be selected preferably from barren, infertile lands. In case agricultural land needs to be selected, written consent from landowners (not lessees) will be obtained; (b) debris disposal site shall be at least 200 m away from surface water bodies; (c) no residential areas shall be located within 50 m downwind side of the site; and (d) site is minimum 250 m away from sensitive locations like settlements ponds/lakes or other water bodies | • List of selected project location and proposed pipeline alignment plan • Involvement of traffic dept. • Road closure planning | •Site observation • Review of documents • Grievance Register | Specific project location | Before commencement of final design and commencement of construction | Environment Specialist of DSISC and PIU | Under Compliance. WTP construction started within fixed campus. No impact on property. Pipe laying will be commenced after finalization of design and alignment |
| Extraction of materials can disrupt natural land contours and vegetation resulting in accelerated erosion, disturbance in natural drainage patterns, ponding and water logging, and water pollution. | - Obtain construction materials only from government approved quarries with prior approval of PIU; - PIU to review, and ensure that proposed quarry sources have all necessary clearances/ permissions in place prior to approval - Contractor to submit to PIU on a monthly basis documentation on material obtained from each source (quarry/ borrow pit) - Avoid creation of new borrow areas, quarries etc., for the project; if unavoidable, contractor to obtain all clearances and permissions as required under law, including Environmental Clearance prior to approval by PIU | • List of approved quarry sites and sources of materials • Construction Contractor documentation | • Checking of records • Visual inspection of sites | Project sites | Before commencement of construction | DSISC Construction Management and Environmental Safeguard Team | Complied All materials procured from licensed vendors after approval of PIU/ DSISC |
| Failure to obtain necessary consents, permits, NOCs, etc. can result to design revisions and/or stoppage of works | - Obtain all necessary consents, permits, clearance, NOCs, etc. prior to award of civil works. - Ensure that all necessary approvals for construction to be obtained by contractor are in place before start of construction - Acknowledge in writing and provide report on compliance all obtained consents, permits, clearance, NOCs, etc. - Include in detailed design drawings and documents all conditions and provisions if necessary | List of applicable legislation | Checking of documents | All project locations | Before commencement of construction | Environment Specialist of DSISC, PIU and PMU/PMC | CTE for the WTP has been obtained Consent to Establish (CTE) for Batching plant also obtained (Appendix 5A). Compliance statement for WTP CTE included in Appendix 5A Other relevant NOC will be obtained as per requirement and with progress of design |
| Health risk due to exposure to asbestos materials | - Obtain details on location of underground asbestos cement materials - Lay the new pipes carefully to avoid encountering asbestos cement pipes - If found, leave the asbestos cement pipes undisturbed in the ground. | asbestos cement materials | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied. Till date no rubbish containing asbestos cement has been found |
| Construction Phase |  |  |  |  |  |  |  |
| Irreversible impact to the environment, workers, and community | Project manager and all key workers will be required to undergo training on EMP implementation including spoils/waste management, Standard operating procedures (SOP) for construction works; occupational health and safety (OHS), core labor laws, applicable environmental laws, etc. | Induction & Awareness Trainings Toolbox Talks Safeguard Trainings | Review of Training records Site Inspections | Project Locations | - | Environment Specialist of DSISC, PIU and PMU | Complied; Site Environmental Safety training and awareness arranged on regular basis (sample attached as Appendix 11) |
| Emissions from construction vehicles, equipment, and machinery used for installation of pipelines resulting to dusts and increase in concentration of vehicle-related pollutants such as carbon monoxide, sulfur oxides, particulate matter, nitrous oxides, and hydrocarbons. | For all construction works - Comply with the Direction of West Bengal Department of Environment under the Air Act, 1981 in controlling air pollution from construction activities - Comply with the air pollution / dust control measures for construction activities stipulated by the “Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7/001/2009 dated: 10 December 2009” - Damp down the soil and any stockpiled material on site by water sprinkling; - Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; - Provide a dust screen around the construction sites at GLSR and WTP work sites - Clean wheels and undercarriage of haul trucks prior to leaving construction site/quarry - Control dust generation while unloading the loose material (particularly aggregate, soil) at the site by sprinkling water and unloading inside the barricaded area - Stabilize surface soils where loaders, support equipment and vehicles will operate by using water and maintain surface soils in a stabilized condition - Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; - Apply water and maintain soils in a visible damp or crusted condition for temporary stabilization - Apply water prior to leveling or any other earth moving activity to keep the soil moist throughout the process - Cover the soil stocked at the sites with tarpaulins - Control access to work area, prevent unnecessary movement of vehicle, public trespassing into work areas; limiting soil disturbance will minimize dust generation - Ensure that all the construction equipment, machinery is fitted with pollution control devises, which are operating correctly, and have a valid pollution under control (PUC) certificate Pipeline works - Barricade the construction area using hard barricades (of 2 m height) on both sides and provide dust/wind screen (such geo textile fabric) up to 3 m height (1m above the hard barricading) - Initiate site clearance and excavation work only after barricading of the site is done - Confine all the material, excavated soil, debris, equipment, machinery (excavators, cranes etc.,), to the barricaded area - Limit the stocking of excavated material at the site; remove the excess soil from the site immediately to the designated disposal area - Undertake the work section wise: 100 – 200 m section should be demarcated and barricaded - Conduct work sequentially - excavation, pipe laying, backfilling; conduct pipe testing section-wise (for a minimum length as possible) so that backfilling, stabilization of soil can be done. - Remove the excavated soil of first section to the disposal site; as the work progresses, sequentially, by the time second section is excavated, the first section will be ready for back filling, use the freshly excavated soil for back filling, this will avoid stocking of material, and minimize the dust. - Backfilled trench at any completed section after removal of barricading will be the main source of dust pollution. The traffic, pedestrian movement and wind will generate dust from backfilled section. Road restoration shall be undertaken immediately. | - Location of stockpiles - Complaints from sensitive receptors - Monitoring data- PM10, PM2.5 NO2, SO2, CO - Heavy equipment and machinery with air pollution control - Water sprinkling arrangement - Cover materials | Site inspection Public grievance register | Covering different locations. Air – noise monitoring: Monitoring is expected to be conducted at 5 locations. However, as the GLSR lands has not been under possession pre-construction monitoring has been conducted at WTP and Booster pumping station only. | Air – noise monitoring: Once before start of construction Yearly 3 times during construction (3-year period considered) Monitoring conducted on 22-01-2020 to 23-01-2020 | DSISC Construction Management and Environmental Safeguard Team, PIU Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied; pre-construction baseline data and during construction data has been collected results attached as Appendix 10. Emission of equipment and vehicle tested. PUC certificate obtained for Vehicle (Appendix 2) Relevant regulation under compliance. Other activities like dust suppression, covering of loose materials, dust screen will be arranged. For pipe line work, barricading, removal of earth, backfilling will be done as per SEMP |
| Mobilization of settled silt materials, and chemical contamination from fuels and lubricants during construction can Contaminate nearby surface water quality. Ponding of water in the pits /foundation excavations | - All earthworks be conducted during the dry season to prevent the problem of soil run-off during monsoon season; - Avoid stockpiling of earth fill especially during the monsoon season unless covered by tarpaulins or plastic sheets; - Prioritize re-use of excess spoils and materials in the construction works. If spoils will be disposed, only designated disposal areas shall be used; - Install temporary silt traps or sedimentation basins along the drainage leading to the water bodies; - Place storage areas for fuels and lubricants away from any drainage leading to water bodies; - Store fuel, construction chemicals etc., on an impervious floor, also avoid spillage by careful handling - Dispose any wastes generated by construction activities in designated sites; - Conduct surface quality inspection according to the Environmental Management Plan (EMP). - Create a temporary drainage channel around the work area to arrest the entry of runoff from upper areas into the work area - Pump out the water collected in the pits / excavations to a temporary sedimentation pond; dispose of only clarified water into drainage channels/streams after sedimentation in the temporary ponds - Consider safety aspects related to pit collapse due to accumulation of water | • Areas for stockpiles, storage of fuels and lubricants and waste materials • Number of silt traps installed along drainages (in slope) leading to water bodies • Entry routes of pollutant in nearby Waterbodies | Site inspection Public grievance register | All project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | DSISC Construction Management and Environmental Safeguard Team Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Silt trap arranged, fuel - lubricants yet to purchase, designated site for waste disposal under consideration. All safety aspect maintained. |
| Increase in noise level due to earth-moving and excavation equipment, and the transportation of equipment, materials, and people | - Plan activities in consultation with PIU so that activities with the greatest potential to generate noise are conducted during periods of the day which will result in least disturbance; - Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; - Minimize noise from construction equipment by using vehicle silencers, fitting jackhammers with noise-reducing mufflers, and use portable street barriers to minimize sound impact to surrounding sensitive receptor; and - Maintain maximum sound levels not exceeding 80 decibels (dBA) when measured at a distance of 10 m or more from the vehicle/s. - Identify any buildings at risk from vibration damage and avoiding any use of pneumatic drills or heavy vehicles in the vicinity - Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; - Consult local communities in advance of the work to identify and address key issues, and avoid working at sensitive times, such as religious and cultural festivals. | Day time and night time noise levels. | • Checking of records • Visual inspection of sites | Monitoring is expected to be conducted at 5 locations. However, as the GLSR lands has not been purchased yet, pre-construction monitoring has been conducted at WTP and Booster pumping station only. | Air – noise monitoring: Once before start of construction Yearly 3 times during construction (3-year period considered) Monitoring conducted on 22-01-2020 to 23-01-2020 Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team at operational site | DSISC Construction Management and Environmental Safeguard Team Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied; pre-construction baseline and during construction data has been collected. There is no as such noise generated from equipment. Stipulated condition as per SEMP is followed |
| Impacts due to excess excavated earth, excess construction materials, and solid waste such as removed concrete, wood, packaging materials, empty containers, spoils, oils, lubricants, and other similar items. | - Prepare and implement a Construction Waste Management Plan - As far as possible utilize the debris and excess soil in construction purpose, for example for raising the ground level or construction of access roads etc., - Stockpiles, lubricants, fuels, and other materials should be located away from steep slopes and water bodies; - Avoid stockpiling any excess spoils at the site for long time. Excess excavated soils should be disposed off to approved designated areas immediately; - If disposal is required, the site shall be selected preferably from barren, infertile lands; site should be located away from residential areas, forests, water bodies and any other sensitive land uses - Domestic solid wastes should be properly segregated in biodegradable and non-biodegradable for collection and disposal to designated solid waste disposal site; create a compost pit at workers’ camp sites for disposal of biodegradable waste; non-biodegradable / recyclable material shall be collected separately and sold in the local recycling material market - Residual and hazardous wastes such as oils, fuels, and lubricants shall be disposed of in disposal sites approved by local authorities/WBPCB; - Prohibit burning of construction and/or domestic waste; - Ensure that wastes are not haphazardly dumped thrown within and around the project site and adjacent areas; provide proper collection bins, and create awareness to use the dust bins. - Conduct site clearance and restoration to original condition after the completion of construction work; PIU to ensure that site is properly restored prior to issuing of construction completion certificate | • Waste Management List • Stockpile Management • Complaints from Sensitive receptors • PMU/ PIU/ DSISC to report in writing that the necessary environmental restoration work has been done | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied Excess earth Mostly used for Backfilling. Domestic solid waste collection bin arranged No as such stock piling allowed for spoil |
| Disruption of service and Damage to existing infrastructure at specified project location | - Prepare a list of affected utilities and operators if any; - Prepare a contingency plan to include actions to be done in case of unintentional interruption of service | • List of affected utilities if any and operators • Public grievance | Observation and document checking | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | Complied as Per requirement. Consultation with utility dept.as and when required; pipeline laying has not yet started |
| Loss of vegetation and tree cover | - Minimize removal of vegetation and disallow cutting of trees; - If tree-removal will be required, obtain tree-cutting permit and - Plant 5 native trees for every one that is removed. | Tree felling requirement and afforestation after final design | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | As of now, tree felling requirement is not envisaged. |
| Traffic problems and conflicts near project locations and haul road | Hauling (material, waste/debris and equipment) activities - Plan transportation routes so that heavy vehicles do not use narrow local roads, except in the immediate vicinity of delivery sites - Schedule transport and hauling activities during non-peak hours; - Locate entry and exit points in areas where there is low potential for traffic congestion; - Drive vehicles in a considerate manner - Notify affected public by public information notices, providing sign boards informing nature and duration of construction works and contact numbers for concerns / complaints. Pipeline works - (i) Confine work areas along the roads to the minimum possible extent; all the activities, including material and waste/surplus soil stocking should be confined to this area. Proper barricading should be provided; avoid - material/surplus soil stocking in congested areas – immediately removed from site/ or brought to the as and when required. - Leave spaces for access between mounds of soil to maintain access to the houses / properties - Provide pedestrian access in all the locations; provide wooden/metal planks over the open trenches at each house to maintain the access. - Inform the affected local population 1-week in advance about the work schedule - Plan and execute the work in such a way that the period of disturbance/ loss of access is minimum. - Keep the site free from all unnecessary obstructions; - Coordinate with Traffic Police for temporary road diversions, where necessary, and for provision of traffic aids if transportation activities cannot be avoided during peak hours | • Traffic Management Plan • Public grievance • Number of signages placed at subproject location | Site visit and document review | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Complied; WTP location is within the PHED campus, no traffic management is required. As of now no pipeline work has started and no grievance has been registered. During pipe laying all Traffic Management rules will be followed |
| Generation of temporary employment and increase in local revenue | - Employ local labor force as far as possible - Comply with labor laws | Employment record | Checking of records | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | Direct and indirect employment for local population |
| Occupational hazards which can arise during work | - Comply with all national, state and local core labor laws (see Appendix 7of this IEE) - Develop and implement site-specific occupational health and safety (OHS) Plan which will include measures such as: (a) excluding public from the site; (b) ensuring all workers are provided with and use personal protective equipment like helmet, gumboot, safety belt, gloves, nose musk and ear plugs; (c) OHS Training for all site personnel; (d) documented procedures to be followed for all site activities; and (e) documentation of work-related accidents; - Ensure that qualified first-aid can be provided at all times. Equipped first-aid stations shall be easily accessible throughout the site; - Provide medical insurance coverage for workers; - Secure all installations from unauthorized intrusion and accident risks; - Provide supplies of potable drinking water; - Provide clean eating areas where workers are not exposed to hazardous or noxious substances; - Provide health and safety orientation training to all new workers to ensure that they are apprised of the basic site rules of work at the site, personal protective protection, and preventing injuring to fellow workers; - Provide visitor orientation if visitors to the site can gain access to areas where hazardous conditions or substances may be present. Ensure also that visitor/s do not enter hazard areas unescorted; - Ensure the visibility of workers through their use of high visibility vests when working in or walking through heavy equipment operating areas; - Ensure moving equipment is outfitted with audible back-up alarms; - Mark and provide sign boards for hazardous areas such as energized electrical devices and lines, service rooms housing high voltage equipment, and areas for storage and disposal. Signage shall be in accordance with international standards and be well known to, and easily understood by workers, visitors, and the general public as appropriate; and - Disallow worker exposure to noise level greater than 85 dBA for a duration of more than 8 hours per day without hearing protection. The use of hearing protection shall be enforced actively. | • Site-specific Health and Safety (H&S) Plan • Equipped first-aid stations; • Medical insurance coverage for workers • Number of accidents • Supplies of potable drinking water; • Record of H&S orientation trainings • Personal protective equipment • Sign boards for hazardous areas such as energized electrical devices and lines, service rooms | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC; PIU and PMU/PMC | Complied. Site-specific Health and Safety (H&S) Plan under implementation. OHS plan submitted by contractor and approved by DSISC and PIU. That document is available with DSISC and PIU However, the existing H&S plan would be revisited to include preventive and mitigative measures with respect to COVID-19 Induction and tool box training arranged by contractor and first aid training arranged by DSISC (sample shown in Appendix 11) Use of PPEs noted mostly. Visibility vest also used. . Drinking water and first aid box available at site. . Medical Insurance arranged for the labourer (see Appendix 4). Medical tie up with health institute done (Appendix 13) but health checking of workers pending. No as such noise producing equipment available at site Accident register is maintained at site. Till date no major accident recorded. Only minor first aid cases recorded. Sample attached as Appendix 12. |
| Health risk due to exposure to asbestos materials | - Obtain details on location of underground asbestos cement materials - Lay the new pipes carefully to avoid encountering asbestos cement pipes - If found, leave the asbestos cement pipes undisturbed in the ground. | asbestos cement materials | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Till date no asbestos cement material has been found on site |
| Traffic accidents and vehicle collision with pedestrians during material and waste transportation | - Restrict construction vehicle movements to defined access roads and demarcated working areas (unless in the event of an emergency) - Enforce strict speed limit (20-30 kmph) for playing on unpaved roads, construction tracks - Night-time haulage will be by exception only, as approved by the PIU to minimize driving risk and disturbance to communities - Adopt standard and safe practices for micro tunneling - Temporary traffic control (e.g. flagmen) and signs will be provided where necessary to improve safety and provide directions - All drivers will undergo safety and training - Public access to all areas where construction works are on-going will be restricted through the use of barricading and security personnel - Warning signs, blinkers will be attached to the barricading to caution the public about the hazards associated with the works, and presence of deep excavation - The period of time when the pipeline trench is left open will be minimized through careful planning - Control dust pollution – implement dust control measures as suggested under air quality section - Maintain regularly the vehicles and use of manufacturer-approved parts to minimize potentially serious accidents caused by equipment malfunction or premature failure. - Provide road signs and flag persons to warn of on-going trenching activities. | Public grievance | Review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Work continued within fixed WTP campus. No pedestrian accident has been recorded till date No traffic control and application of traffic management plan is required. Only during transportation, entry of vehicle at construction site controlled by concerned construction supervisor |
| Temporary air and noise pollution from machine operation, water pollution from storage and use of fuels, oils, solvents, and lubricants Unsanitary and poor living conditions for workers | - Ensure that a proper compound wall is provided, and erect a wind/dust screen around - Camp site shall not be located near (100 m) water bodies, flood plains flood prone/low lying areas, or any ecologically, socially, archeologically sensitive areas - Separate the workers living areas and material storage areas clearly with a fencing and separate entry and exit - Provide proper temporary accommodation with proper materials, adequate lighting and ventilation, appropriate facilities for winters and summers; ensure conditions of livability at work camps are maintained at the highest standards possible at all times; - Consult PIU before locating project offices, sheds, and construction plants; (viii)Minimize removal of vegetation and disallow cutting of trees - Ensure conditions of livability at work camps are maintained at the highest standards possible at all times; living quarters and construction camps shall be provided with standard materials (as far as possible to use portable ready to fit-in reusable cabins with proper ventilation); thatched huts, and facilities constructed with materials like GI sheets, tarpaulins, etc., shall not be allowed as accommodation for workers - Camp shall be provided with proper drainage, there shall not be any water accumulation - Provide drinking water, water for other uses, and sanitation facilities for employees - Prohibit employees from cutting of trees for firewood; contractor should be provided proper facilities including cooking fuel (oil or gas; fire wood not allowed) - Train employees in the storage and handling of materials which can potentially cause soil contamination - Recover used oil and lubricants and reuse or remove from the site - Manage solid waste according to the following preference hierarchy: reuse, recycling and disposal to designated areas; provide a compost pit for biodegradable waste, and non-biodegradable / recyclable waste shall be collected and sold in local market - Remove all wreckage, rubbish, or temporary structures which are no longer required - At the completion of work, camp area shall be cleaned and restored to pre-project conditions, and submit report to PIU; PIU to review and approve camp clearance and closure of work site | • Public grievance • Accommodation • Water and sanitation facilities for employees • Housekeeping – regular disposal of solid waste | Site inspection and review of documents | Construction camps | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Mostly Complied. Initially labour accommodation. kitchen and toilet facilities were very poor. However, after intervention of DSISC and PMC, followed by consultation with other stakeholders these facilities have been significantly improved now. Storage and disposal of solid waste planned accordingly. Site photo in Appendix 1. |
| There are no protected properties in the subproject sites. However, in case of chance finds, contractors will be required to follow a protocol as defined in the mitigation measures. | - Consult Archaeological Survey of India (ASI) or West Bengal State Archaeology Department to obtain an expert assessment of the archaeological potential of the site. - Include state and local archaeological, cultural and historical authorities, and interest groups in consultation forums as project stakeholders so that their expertise can be made available. - In case of chance finds, works must be stopped immediately until such time chance finds are cleared by experts | Site inspection records | Site inspection and review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PMU/PMC | No chance finds to date. |
| Unsatisfactory compliance to EMP | - Appointment of (I) Environment, Health and Safety (EHS) Supervisor to ensure EMP implementation - Timely submission of monitoring reports including pictures | Appointment letter Monitoring records | Review of records | - | - | Environment Specialist of DSISC and PIU | Complied mostly. Analysis of monitoring result indicates high PM10 and PM2.5 concentration in all the monitored locations. However, exceedances are largely attributable to high background concentration Continuous training is required for safety officer on Environment aspect |
| Damage due to debris, spoils, excess construction materials | - Remove all spoils wreckage, rubbish, or temporary structures (such as buildings, shelters, and latrines) which are no longer required; and - All excavated roads shall be reinstated to original condition. - All disrupted utilities restored - All affected structures rehabilitated/compensated - The area that previously housed the construction camp is to be checked for spills of substances such as oil, paint, etc. and these shall be cleaned up. - All hardened surfaces within the construction camp area shall be ripped, all imported materials removed, and the area shall be top soiled and re-grassed using the guidelines set out in the re-vegetation specification that forms part of this document. - The contractor must arrange the cancellation of all temporary services. quest PIU to report in writing that worksites and camps have been vacated and restored to pre-project conditions before acceptance of work. | Stockpile Management Spoil Management Restoration of sites | Review of documents and site inspections | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Complied; Spoil Management Plan has been submitted for the WTP construction No pipe laying work started. Spoil management plan will be updated as a part of SEMP. |
| Operational Phase |  |  |  |  |  |  |  |
| Public health, safety and environmental impacts | - Operate as per the Operational Manual following Standard Operating Procedures as per the WTP design - Undertake preventive and periodic maintenance activities as required - Ensure periodic training to staff in WTP operation, especially in chemical handling and dosing, filter backwash, etc., - replace pumps, motors and other parts as per the operating life prescribed by manufacturer - Maintain the mechanical parts as per the maintenance plan to avoid any hazards. - Ensure that all safety apparatus at WTP including personal protection equipment are in good condition all times; and are at easily accessible and easily identifiable place; periodically check the equipment, and conduct mock drills to deal with emergency situations - Ensure that backwash recirculation system and sludge management system are operated as per the manual | Health records Facility inspection records Incident reports | Review of records | Project facilities | - | Environment specialist of DSISC; PIU and PMU/PMC | NA at the present project stage |
| Loss of water, increased demand and Inconvenience to consumers and general public | Effectiveness of leak detection and water auditing to reduce the water losses. | DCS system water balance | Review of data | Bulk Distribution network | - | Environment specialist of DSISC; PIU and PMU/PMC | NA at the present project stage |
| Health, social and economic impacts on the workers | - Provide appropriate PPE and training on its proper use and maintenance. - Use fall protection equipment when working at heights. - Maintain work areas to minimize slipping and tripping hazards. - Implement a training program for operators who work with chlorine regarding safe handling practices and emergency response procedures. - Prepare escape plans from areas where there might be a chlorine emission. - Install safety showers and eye wash stations near the chlorine equipment and other areas where hazardous chemicals are stored or used. - Prohibit eating, smoking, and drinking except in designated areas. | Facility inspection records Incident register | Review of data | Project locations, especially WTP site | - | Environment specialist of DSISC; PIU and PMU/PMC | NA at the present project stage Health & Safety training will be continued during O & M stage |
| Water pollution, and impacts on public health and environment | Sanitation and sewerage/septage facilities needs to be improved/provided in the project area to suit the increased sewage generation | Health records increase in sewage facility | Review of records | Project influenced area | - | Environment specialist of DSISC; PIU and PMU/PMC | NA at the present project stage |

Table 10: Summary of Environmental Monitoring Activities for the Package WBDWSIP/DWW/NCB/N24P/02A/2017-18: Water Supply Distribution at Haroa Block and Package WBDWSIP/DWW/NCB/N24P/02B/2017-18: Water Supply Distribution at Bhangar II Block

| Impacts | Mitigation Measures | Parameters Monitored (As a minimum those identified in the IEE should be monitored) | Method of Monitoring | Location of Monitoring | Date of Monitoring Conducted | Name of Person Who Conducted the Monitoring | Compliance Status/ Explanation Pack- N24P/02A |  | Compliance Status/ Explanation Pack- N24P/02B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design Phase |  |  |  |  |  |  |  |  |  |  |  |
| Source sustainability and efficiency | (i) Gravity distribution system: designing the entire system to maintain optimal flow and terminal pressure, and optimizing the overall energy usage (ii)Implementation of a water quality surveillance program including development of a laboratory as part of the project to ensure that supplied water meets the drinking water standards (iii)Minimizing water losses from pipelines by perfect jointing and alignments using appropriate techniques (iv) Reducing the incidence of water borne diseases by providing 100% population including urban poor with potable water supplies | - Design philosophy - Project QA/QC plan - Selection methodology for distribution network | Document review and LOP Survey | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Complied; Distribution network finalized for: Zone-1 (Amta), Zone-2 (Nebutala Abad) Zone-3 (Adampur) Zone 4 (Laugachi), Zone-5 (Tegharia), Zone-6 (Puratan Kamarganti), Zone-10 (Baganti) Zone-12 (Parchandpur), Zone-13 (Chauhata), Zone-14 (Khalisadi), Zone-15 (Jhikia), Zone-16 (Haripur) Zone-17 (Salipur), Zone-18 (Gopalpur), Zone-19 (Khoda Chandpur), Zone-21 (Haroa) |  | Complied; Design completed and Pipe laying work continued at 13 zones - 1,3,4,7 to 16 |  |
| Socio economic impact – loss fishery area | (i) Avoid using low-lying lands / ponds for construction of OHRs; alternative private lands may be explored within the vicinity; (ii)Review the applicability of West Bengal Inland Fisheries Act, 1984, whether the site falls under the definition of fisher area; obtained permission from Fisheries Department if required prior to start of construction | List of selected location for OHRs | Site survey | All OHR sites | Before commencement of final design | Environment Specialist of DSISC and PMC | Complied; No low-lying lands or ponds are being filled for construction. |  | Complied; No low-lying lands or ponds are being filled for construction. |  |
| Tree cutting | (i) Minimize removal of trees by adopting to site condition and with appropriate layout design of OHRs within the sites (ii)Avoid cutting of trees by adopting suitable alignment changes as required during laying of pipelines; (iii)In unavoidable cases, obtain prior permission for tree cutting (iv)Plant and maintain 5 trees for each tree that is removed | - Tree felling requirement – site layout plan - NOC – paper documents from line agency | Site survey and review of site layout/ pipeline alignment plan | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Complied; except Zone 18: Gopalpur no other sites requirement for tree felling has been envisaged till date. NOC obtained and tree felling done for that site. Compensatory plantation remaining. |  | Under compliance, no further tree felling envisaged yet In case of tree felling action will be taken for planation of trees |  |
| Pre-Construction Phase |  |  |  |  |  |  |  |  |  |  |  |
| Telephone lines, electric poles and wires, water lines within proposed project area | (i) Identify and include locations and operators of these utilities in the detailed design documents to prevent unnecessary disruption of services during construction phase; and (ii) Require construction contractors to prepare a contingency plan to include actions to be taken in case of unintentional interruption of services. (iii) Require contractors to prepare spoils (waste) management plan and traffic management plan | List of affected utilities if any and operators | Observation and document checking | Specific project location | Before commencement of construction | Environment Specialist of DSISC and PIU | Complied; more than 500 km of pipeline has been laid Spoil Management Plan & Traffic Management Plan available with SEMP. As prepared zone wise |  | Complied; about 450 km of pipeline has been laid Spoil Management Plan & Traffic Management Plan available with SEMP. As prepared zone wise |  |
| Conflicts with local community; disruption to traffic flow and sensitive receptors | (i) Prioritize areas within or nearest possible vacant space in the project location; (ii) If it is deemed necessary to locate elsewhere, consider sites that will not promote instability and result in destruction of property, vegetation, irrigation, and drinking water supply systems; (iii) Do not consider residential areas; (iv) Take extreme care in selecting sites to avoid direct disposal to water body which will inconvenience the community. (v) For excess spoil disposal, ensure (a) site shall be selected preferably from barren, infertile lands. In case agricultural land needs to be selected, written consent from landowners (not lessees) will be obtained; (b) debris disposal site shall be at least 200 m away from surface water bodies; (c) no residential areas shall be located within 50 m downwind side of the site; and (d) site is minimum 250 m away from sensitive locations like settlements, ponds/lakes or other water bodies. | - List of selected location for OHRs - Involvement of traffic dept. - Road closure planning | - Site observation - Review of documents - Grievance Register | Specific project location | Before commencement of final design and commencement of construction | Environment Specialist of DSISC; PIU and PMC | Complied No disruption noted. Area selected nearby vacant place No excess spoil generated. Excess earth utilized for back filling. No complete road closure expected. Partial closure noted, which mentioned in traffic management plan |  | Complied No disruption noted. Area selected nearby vacant place No excess spoil generated. Excess earth utilized for back filling, which indicates in SEMP. No complete road closure expected. Partial closure noted, which mentioned in traffic management plan |  |
| Extraction of materials can disrupt natural land contours and vegetation resulting in accelerated erosion, disturbance in natural drainage patterns, ponding and water logging, and water pollution. | (i) Obtain construction materials only from government approved quarries with prior approval of PIU; (ii) PIU to review, and ensure that proposed quarry sources have all necessary clearances/ permissions in place prior to approval (iii) Contractor to submit to PIU on a monthly basis documentation on material obtained from each source (quarry/ borrow pit) (iv) Avoid creation of new borrow areas, quarries etc., for the project; if unavoidable, contractor to obtain all clearances and permissions as required under law, including Environmental Clearance prior to approval by PIU | - List of approved quarry sites and sources of materials - Construction Contractor documentation | - Checking of records - Visual inspection of sites | Quarries and material source areas | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC at operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied; Royalty receipt checked. Approval obtained from DSISC, PIU before procurement. |  | Complied; Royalty receipt checked. Approval obtained from DSISC, PIU before procurement. |  |
| Failure to obtain necessary consents, permits, NOCs, etc. can result to design revisions and/or stoppage of works | (i) Obtain all necessary consents, permits, clearance, NOCs, etc. prior to award of civil works. (ii) Ensure that all necessary approvals for construction to be obtained by contractor are in place before start of construction (iii) Acknowledge in writing and provide report on compliance all obtained consents, permits, clearance, NOCs, etc. (iv) Include in detailed design drawings and documents all conditions and provisions if necessary | List of applicable legislation | Checking of documents | All project locations | Before commencement of construction | Environment Specialist of DSISC and PMC | Complied and to be continued as per requirement |  | NOC from EKWMA obtained for pipelaying work in EKW area i.e. Zone-1, 2 and 18. NOC from EKWMA is attached as Appendix 6A. Decided – not to construct OHR within EKW zone. Alternative sites are under finalization. |  |
| Health risk due to exposure to asbestos materials | (i) Obtain details on location of asbestos cement materials (ii) Lay the new piper carefully to avoid encountering AC pipes (ii) Leave the AC pipes undisturbed in the ground. | Asbestos cement materials | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied. Till date no rubbish containing asbestos cement has been found |  | Complied. Till date no rubbish containing asbestos cement has been found |  |
| Construction Phase |  |  |  |  |  |  |  |  |  |  |  |
| Irreversible impact to the environment, workers, and community | (i) Project manager and all key workers will be required to undergo training on EMP implementation including spoils/waste management, Standard operating procedures (SOP) for construction works; occupational health and safety (OHS), core labor laws, applicable environmental laws, etc. | Induction & Awareness Trainings Toolbox Talks Safeguard Trainings | Review of Training records Site Inspections | Project Locations | - | Environment Specialist of DSISC: PIU and PMC | Under compliance; Site Environmental Safety training and awareness arranged by contractor on regular basis (Sample attached as Appendix 11) Awareness program cum training arranged by DSISC which shown in Appendix 11 |  | Under compliance; Site Environmental Safety training and awareness arranged by contractor on regular basis (Sample attached as Appendix 11) Awareness program cum training arranged by DSISC which shown in Appendix 11 |  |
| Emissions from construction vehicles, equipment, and machinery used for installation of pipelines resulting to dusts and increase in concentration of vehicle-related pollutants such as carbon monoxide, sulfur oxides, particulate matter, nitrous oxides, and hydrocarbons. | For all construction works (i) Comply with the air pollution / dust control measures for construction activities stipulated by the “Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009” (ii) Damp down the soil and any stockpiled material on site by water sprinkling; (iii)Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; (iv) Provide a dust screen/high compound wall around the construction sites (OHRs) (v) Clean wheels and undercarriage of haul trucks prior to leaving construction site/quarry (vi) Control dust generation while unloading the loose material (particularly aggregate, soil) at the site by sprinkling water and unloading inside the barricaded area (vii) Stabilize surface soils where loaders, support equipment and vehicles will operate by using water and maintain surface soils in a stabilized condition (viii) Apply water prior to leveling or any other earth moving activity to keep the soil moist throughout the process (ix) Control access to work area, prevent unnecessary movement of vehicle, public trespassing into work areas; limiting soil disturbance will minimize dust generation (x) Ensure that all the construction equipment and machineries are fitted with pollution control devises, which are operating correctly, and have a valid pollution under control (PUC) certificate Pipeline works (i) Barricade the construction area (ii) Initiate site clearance and excavation work only after barricading of the site is done (iii)Confine all the material, excavated soil, debris, equipment, machinery (excavators, cranes etc.,), to the barricaded area (iv) Limit the stocking of excavated material at the site; remove the excess soil from the site immediately to the designated disposal area (v) Undertake the work section wise (vi)Conduct work sequentially - excavation, pipe laying, backfilling; conduct pipe testing section-wise (for a minimum length as possible) so that backfilling, stabilization of soil can be done. (vii) Remove the excavated soil of first section to the disposal site; as the work progresses, sequentially, by the time second section is excavated, the first section will be ready for back filling, use the freshly excavated soil for back filling, this will avoid stocking of material, and minimize the dust. (viii) Backfilled trench at any completed section after removal of barricading will be the main source of dust pollution. The traffic, pedestrian movement and wind will generate dust from backfilled section. Road restoration shall be undertaken immediately. | - Location of stockpiles - Complaints from sensitive receptors - Monitoring data- PM!0, PM2.5, SO2, NO2, CO - Heavy equipment and machinery with air pollution control - Water sprinkling arrangement - Cover materials | Site inspection Public grievance register | Project locations Air- monitoring as per selected sites in ref. to SEMP | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff Air- monitoring schedule as per SEMP Monitoring conducted between 2/12/2019 and 5/12/2019 for package N24P/02A and between 08/11/2019 and 30/11/2019 for package 02B | Environment Specialist of DSISC; PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Under compliance; More than 500 km pipe line laid. Water sprinkling done as per requirement. Pre- construction and during construction air quality monitoring done as per IEE (Complete result certificates available in DSISC office as back up paper and attached as Appendix 10) Most of the PUC certificate obtained for Vehicle and Equipment (Appendix 2 as sample) Other activities like dust suppression, covering of loose materials, dust screen arranged. For pipe line work, barricading, removal of earth, backfilling done but improvement required at few locations. |  | Under compliance; More than 450 km pipe line laid. Water sprinkling done as per requirement. Pre- construction and during construction air quality monitoring done as per IEE (Complete result certificates available in DSISC office as back up paper and attached as Appendix 10) Most of the PUC certificate obtained for Vehicle and Equipment (Appendix 2 as sample) Other activities like dust suppression, covering of loose materials, dust screen arranged. For pipe line work, barricading, removal of earth, backfilling done but improvement required at few locations. |  |
| Mobilization of settled silt materials, and chemical contamination from fuels and lubricants during construction can contaminate nearby surface water quality. Ponding of water in the pits / foundation excavations | (i) All earthworks be conducted during the dry season to prevent the problem of soil run-off during monsoon season; (ii) Avoid stockpiling of earth fill especially during the monsoon season unless covered by tarpaulins or plastic sheets; (iii)Prioritize re-use of excess spoils and materials in the construction works. If spoils will be disposed, only designated disposal areas shall be used; (iv) Install temporary silt traps or sedimentation basins along the drainage leading to the water bodies; (v) Place storage areas for fuels and lubricants away from any drainage leading to water bodies; (vi) Store fuel, construction chemicals etc., on an impervious floor, also avoid spillage by careful handling (vii) Dispose any wastes generated by construction activities in designated sites; and (viii) Conduct surface quality inspection according to the Environmental Management Plan (EMP). (ix) Create a temporary drainage channel around the work area to arrest the entry of runoff from upper areas into the work area (x) Pump out the water collected in the pits / excavations to a temporary sedimentation pond; dispose of only clarified water into drainage channels/streams after sedimentation in the temporary ponds (xi) Consider safety aspects related to pit collapse due to accumulation of water | - Areas for stockpiles, storage of fuels and lubricants and waste materials - Number of silt traps installed along drainages (in slope) leading to water bodies - Entry routes of pollutant in nearby Water bodies | Site inspection Public grievance register | All project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC: PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Mostly Complied Earth work conducted during dry season. No as such requirement of disposal of spoil. Excess earth utilized for backfilling. Material never disposed in the pond located within this site. Silt traps / sediment basin should be installed prior to discharge of construction wastewater from site. Fuel storage not started. Surface water quality monitoring done All safety aspect maintained |  | Mostly Complied Earth work conducted during dry season. No as such requirement of disposal of spoil. Excess earth utilized for backfilling. Material never disposed in the pond located within this site. Silt traps / sediment basin should be installed prior to discharge of construction wastewater from site. Fuel storage not started. Surface water quality monitoring done All safety aspect maintained |  |
| Increase in noise level due to earth-moving and excavation equipment, and the transportation of equipment, materials, and people | (i) Plan activities in consultation with PIU so that activities with the greatest potential to generate noise are conducted during periods of the day which will result in least disturbance; (ii) Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; (iii) Minimize noise from construction equipment by using vehicle silencers, fitting jackhammers with noise-reducing mufflers, and use portable street barriers to minimize sound impact to surrounding sensitive receptor; and (iv) Identify any buildings at risk from vibration damage and avoiding any use of pneumatic drills or heavy vehicles in the vicinity (v) Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; (vi) Consult local communities in advance of the work to identify and address key issues, and avoid working at sensitive times, such as religious and cultural festivals. | - Complaints from sensitive receptors - Use of silencers in noise-producing equipment and sound barriers - Monitoring data | - Checking of records - Visual inspection of sites | All project locations Noise- monitoring as per selected sites in ref. to SEMP | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff Noise monitoring schedule as per SEMP. Monitoring conducted between 2/12/2019 and 5/12/2019 for package N24P/02A And between 08/11/2019 and 30/11/2019 for package 02B | Environment Specialist of DSISC; PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Mostly Complied. No such noise generating problem near the project location. Pre-construction and during construction monitoring done. Monitoring will be continued as per IEE and SEMP. Complete result certificates available in DSISC office as back up paper and attached as Appendix 10. However, use of ear plugs by labourer ensured during noisy activities. Honking generally avoided at work sites. |  | Mostly Complied. No such noise generating problem near the project location. Pre-construction and during construction monitoring done. Monitoring will be continued as per IEE and SEMP. Complete result certificates available in DSISC office as back up paper and attached as Appendix 10. However, use of ear plugs by labourer ensured during noisy activities. Honking generally avoided at work sites. |  |
| Impacts due to excess excavated earth, excess construction materials, and solid waste such as removed concrete, wood, packaging materials, empty containers, spoils, oils, lubricants, and other similar items. | (i) Prepare and implement a Construction Waste Management Plan (ii) As far as possible utilize the debris and excess soil in construction purpose, for example for raising the ground level or construction of access roads etc., (iii) Stockpiles, lubricants, fuels, and other materials should be located away from steep slopes and water bodies; (iv)Avoid stockpiling any excess spoils at the site for long time. Excess excavated soils should be disposed of to approved designated areas immediately; (v) If disposal is required, the site shall be selected preferably from barren, infertile lands; site should be located away from residential areas, forests, water bodies and any other sensitive land uses (vi) Domestic solid wastes should be properly segregated in biodegradable and non-biodegradable for collection and disposal to designated solid waste disposal site; create a compost pit at workers’ camp sites for disposal of biodegradable waste; non-biodegradable / recyclable material shall be collected separately and sold in the local recycling material market (vii) Residual and hazardous wastes such as oils, fuels, and lubricants shall be disposed of in disposal sites approved by local authorities/West Bengal Pollution Control Board (WBPCB); (viii) Prohibit burning of construction and/or domestic waste; (ix) Ensure that wastes are not haphazardly dumped thrown within and around the project site and adjacent areas; provide proper collection bins, and create awareness to use the dust bins. (x) Conduct site clearance and restoration to original condition after the completion of construction work; PIU to ensure that site is properly restored prior to issuing of construction completion certificate | - Waste Management List - Stockpile Management - Complaints from Sensitive receptors - PMU/PIU/DSISC to report in writing that the necessary environmental restoration work has been done | - Checking of records - Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC: PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied Excess earth used mostly for backfilling Excess spoils are not generally stockpiled No hazardous waste and construction waste generated. |  | Complied Excess earth used mostly for backfilling Excess spoils are not generally stockpiled No hazardous waste and construction waste generated. |  |
| Disruption of service and damage to existing infrastructure at specified project location | (i) Prepare a list of affected utilities and operators if any; (ii) Prepare a contingency plan to include actions to be done in case of unintentional interruption of service | - List of affected utilities if any and operators - Public grievance | Observation And document checking | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PIU | Complied as Per requirement. Consultation with utility dept. as and when required |  | Complied as Per requirement. Consultation with utility dept. as and when required |  |
| Loss of vegetation and tree cover | (i) Minimize removal of vegetation and disallow cutting of trees, by adopting best site layout and pipeline alignments (ii) If tree-removal will be required, obtain tree-cutting permit and (iii) Plant 5 native trees for every one that is removed. | Tree felling requirement and afforestation after final design | - Checking of records - Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | As of now, tree felling will be required only at Zone 18: Gopalpur. NOC obtained and tree felling done. Compensatory plantation not yet done |  | As of now, tree felling requirement is not envisaged. |  |
| Traffic problems and conflicts near project locations and haul road | Hauling (material, waste/debris and equipment) activities (i) Plan transportation routes so that heavy vehicles do not use narrow local roads, except in the immediate vicinity of delivery sites (ii) Schedule transport and hauling activities during non-peak hours; (iii)Locate entry and exit points in areas where there is low potential for traffic congestion; (iv) Drive vehicles in a considerate manner (v) Notify affected public by public information notices, providing sign boards informing nature and duration of construction works and contact numbers for concerns/complaints. Pipeline works (i) Confine work areas along the roads to the minimum possible extent; all the activities, including material and waste/surplus soil stocking should be confined to this area. Provide barricading; avoid material/surplus soil stocking in congested areas – immediately removed from site/ or brought to the as and when required (ii) Leave spaces for access between mounds of soil to maintain access to the houses / properties (iii)Provide pedestrian access in all the locations; provide wooden/metal planks over the open trenches at each house to maintain the access. (iv) Inform the affected local population 1-week in advance about the work schedule (v) Plan and execute the work in such a way that the period of disturbance/ loss of access is minimum. (vi) Keep the site free from all unnecessary obstructions; (vii) Coordinate with Police for temporary road diversions, where necessary, and for provision of traffic aids if transportation activities cannot be avoided during peak hours | - Traffic Management Plan - Public grievance - Number of signages placed at subproject location | Site visit and document review | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC; PIU and PMC | Complied; As of now about 500 km pipeline has been laid. Barricading done- partially complied. Local people informed before start of work No transportation of pipe/ material done during pick hours, no impact on local public movement. Access maintained in most of the cases at pipe laying areas. Work plan informed to local public atleast 1 week in advance. |  | Complied; As of now about 450 km pipeline has been laid. Barricading done- partially complied. Local people informed before start of work No transportation of pipe/ material done during pick hours, no impact on local public movement. Access maintained in most of the cases at pipe laying areas. Work plan informed to local public atleast 1 week in advance. |  |
| Generation of temporary employment and increase in local revenue | (i) Employ local labor force as far as possible (ii) Comply with labor laws | Employment record | Checking of records | Project locations | - | Environment Specialist of DSISC and PMC | Direct and indirect employment for local population - noted |  | Direct and indirect employment for local population - noted |  |
| Occupational hazards which can arise during work | (i) Comply with all national, state and local core labor laws (ii) Develop and implement site-specific occupational health and safety (OHS) Plan which will include measures such as: (a) excluding public from the site; (b) ensuring all workers are provided with and use personal protective equipment like helmet, gumboot, safety belt, gloves, nose musk and ear plugs; (c) OHS Training for all site personnel; (d) documented procedures to be followed for all site activities; and (e) documentation of work-related accidents; (iii) Ensure that qualified first-aid can be provided at all times. Equipped first-aid stations shall be easily accessible throughout the site; (iv) Provide medical insurance coverage for workers; (v) Secure all installations from unauthorized intrusion and accident risks; (vi) Provide supplies of potable drinking water; (vii) Provide clean eating areas where workers are not exposed to hazardous or noxious substances; (viii) Provide health and safety orientation training to all new workers to ensure that they are apprised of the basic site rules of work at the site, personal protective protection, and preventing injuring to fellow workers; (ix) Provide visitor orientation if visitors to the site can gain access to areas where hazardous conditions or substances may be present. Ensure also that visitor/s do not enter hazard areas unescorted; (x) Ensure the visibility of workers through their use of high visibility vests when working in or walking through heavy equipment operating areas; (xi) Ensure moving equipment is outfitted with audible back-up alarms; (xii) Mark and provide sign boards for hazardous areas such as energized electrical devices and lines, service rooms housing high voltage equipment, and areas for storage and disposal. Signage shall be in accordance with international standards and be well known to, and easily understood by workers, visitors, and the general public as appropriate | - Site-specific Health and Safety (H&S) Plan - Equipped first-aid stations; - Medical insurance coverage for workers - Number of accidents - Supplies of potable drinking water; - Record of H&S orientation trainings - Personal protective equipment - Sign boards for hazardous areas such as energized electrical devices and lines, service rooms | - Checking of records - Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Site-specific Health and Safety (H&S) Plan under implementation. Copy of the approved Health and Safety plan available with DSISC., PIU H & S training done on regular basis. Sample training document is attached as Appendix 11 However, the existing H&S Plans will be updated to include the preventive and mitigative measures with respect to COVID-19 Drinking water and first aid box available at site. Partial use of PPEs- noted. - Missing PPEs particularly use of safety shoes, ear plug, goggles were noted at few sites (Haroa – Zone 1, Zone 16) Tie up letter with nearby health center in case of emergency obtained (Appendix13). Health check-up done for worker (Appendix 13) Medical Insurance arranged for the labourer (see Appendix 4). Accident/ First aid register is maintained at each site (see sample copy in Appendix 12) |  | Site-specific Health and Safety (H&S) Plan under implementation. Copy of the approved Health and Safety plan available with DSISC., PIU H & S training done on regular basis. Sample training document is attached as Appendix 11 However, the existing H&S Plans will be updated to include the preventive and mitigative measures with respect to COVID-19 Drinking water and first aid box available partially. - First aid box missing in many sites like Bhangar II – zone 12, zone 13. Partial use of PPEs- noted. - Missing PPE, particularly use of safety shoes, ear plug, goggles are serious issues at few sites e.g. (Bhangar II – Zone 13) Tie up letter with nearby health center in case of emergency obtained (Appendix 13). Health check-up done for worker (Appendix 13) Medical Insurance arranged for the labourer (see Appendix 4). Accident/ First aid register is maintained at each site (see sample copy in Appendix 12) |  |
| Health risks associated with AC pipes | (i) leave AC pipes in-situ untouched | Decommissioned AC Pipes | Site inspection | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Till date no AC pipes has been found |  | Till date no AC pipes has been found |  |
| Impact on community safety. Traffic accidents and vehicle collision with pedestrians during material and waste transportation | (i) Restrict construction vehicle movements to defined access roads and demarcated working areas (unless in the event of an emergency) (ii) Enforce strict speed limit (20-30 kph) for playing on unpaved roads, construction tracks (iii) Night-time haulage will be by exception only, as approved by the PIU to minimize driving risk and disturbance to communities (iv) Adopt standard and safe practices for micro tunneling (vi) Temporary traffic control (e.g. flagmen) and signs will be provided where necessary to improve safety and provide directions (vii) All drivers will undergo safety and training (viii) Public access to all areas where construction works are on-going will be restricted through the use of barricading and security personnel (ix) Warning signs, blinkers will be attached to the barricading to caution the public about the hazards associated with the works, and presence of deep excavation (x) The period of time when the pipeline trench is left open will be minimized through careful planning (xi) Control dust pollution – implement dust control measures as suggested under air quality section (xii) Maintain regularly the vehicles and use of manufacturer-approved parts to minimize potentially serious accidents caused by equipment malfunction or premature failure. (xiii) Provide road signs and flag persons to warn of on-going trenching activities. | Public grievance | Review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC; PIU and PMC | No pedestrian accident has been recorded till date. Pipe line laying work continued. - Barricades and caution tapes in work areas, in particular along the pipelines were found as the most ubiquitous issue for sites like Haora-Zone 1. At Zone 2, separate lane for movement of school children as well as safety coordinator should be provided during school opening and closing hours. All safety measures arranged. No trench will be kept open after pipe laying. Caution tape will be placed. |  | No pedestrian accident has been recorded till date. Pipe line laying work continued. - Barricades and caution tapes in work areas, in particular along the pipelines were found as the most ubiquitous issue for sites like Bhangar II-Zone 14 . All safety measures arranged. No trench will be kept open after pipe laying. Caution tape will be placed. |  |
| Impact on work camps and work site. Temporary air and noise pollution from machine operation, water pollution from storage and use of fuels, oils, solvents, and lubricants Unsanitary and poor living conditions for workers | (i) As far as possible located the camp site within the work sites; if any camp to be established outside these, then select a camp site away from residential areas (at least 100 m buffer shall be maintained) (ii) Avoid tree cutting for setting up camp facilities (iii) Camp site shall not be located near (100 m) water bodies, flood plains flood prone/low lying areas, or any ecologically, socially, archeologically sensitive areas (iv) Separate the workers living areas and material storage areas clearly with a fencing and separate entry and exit (v) Provide proper temporary accommodation with proper materials, adequate lighting and ventilation, appropriate facilities for winters and summers; ensure conditions of livability at work camps are maintained at the highest standards possible at all times; (vi) Consult PIU before locating project offices, sheds, and construction plants; (vii) Minimize removal of vegetation and disallow cutting of trees (viii) Ensure conditions of livability at work camps are maintained at the highest standards possible at all times; living quarters and construction camps shall be provided with standard materials (as far as possible to use portable ready to fit-in reusable cabins with proper ventilation); thatched huts, and facilities constructed with materials like GI sheets, tarpaulins, etc., shall not be allowed as accommodation for workers (ix) Camp shall be provided with proper drainage, there shall not be any water accumulation (x) Provide drinking water, water for other uses, and sanitation facilities for employees (xi) Prohibit employees from cutting of trees for firewood; contractor should be providing proper facilities including cooking fuel (oil or gas; fire wood not allowed) (xii) Train employees in the storage and handling of materials which can potentially cause soil contamination (xiii) Recover used oil and lubricants and reuse or remove from the site (xiv) Manage solid waste according to the following preference hierarchy: reuse, recycling and disposal to designated areas; provide a compost pit for biodegradable waste, and non-biodegradable / recyclable waste shall be collected and sold in local market (xv) Remove all wreckage, rubbish, or temporary structures which are no longer required (xvi) At the completion of work, camp area shall be cleaned and restored to pre-project conditions, and submit report to PIU; PIU to review and approve camp clearance and closure of work site | - Public grievance - Accommodation - Water and sanitation facilities for employees - Housekeeping – regular disposal of solid waste | Site inspection and review of documents | Construction camps | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Partially Complied However, housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking, provision of solid waste and wastewater disposal are some of the aspects requiring improvement. - Provision of basic facilities (like bed, drinking water fuel for cooking, toilets at site) to labourers in the construction camp sites at Haroa – Zone 16, Zone 18- unsatisfactory. - In Haroa Zone 16 labour camp was found within ICDS centre. It should be relocated as soon as possible. - In Haroa Zone 1 there is an issue of maintenance, as silencer leakage was found during site inspection. Beside it in this site vat sludge from storage tank and wastes from mixing machine was mixed with local pond. Soil barrier have to provide in this location. |  | Partially Complied However, housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking, provision of solid waste and wastewater disposal are some of the aspects requiring improvement. - Provision of basic facilities (like bed, drinking water fuel for cooking, toilets at site) to labourers in the construction camp sites at Bhangar II – Zone 4, 10, 13 - unsatisfactory. - Management of toilet waste at camp sites, use of pit latrines are serious concern (Bhangar II – Zone 10, 13) - Instruction has been given for improvement |  |
| Unsatisfactory compliance to EMP | (i) Appointment of (I) Environment, Health and Safety (EHS) Supervisor to ensure EMP implementation (ii) Timely submission of monitoring reports including pictures | - Appointment letter - Monitoring records | Review of records | - | - | Environment Specialist of DSISC and PMC | Safety person appointed from contractor end, but environment training is required on regular basis. Monitoring report submitted on monthly basis. Analysis of monitoring result indicates high PM 10 and PM 2.5 concentration in most of the locations. Furthermore, marginal exceedance of noise level was also observed in most of the locations. However, this exceedance is largely attributable to high background concentrations |  | Safety person appointed from contractor end, but environment training is required on regular basis. Monitoring report submitted on monthly basis. Analysis of monitoring result indicates high PM 10 and PM 2.5 concentration in most of the locations. Furthermore, marginal exceedance of noise level was also observed in most of the locations. However, this exceedance is largely attributable to high background concentrations |  |
| Damage due to debris, spoils, excess construction materials | (i) Remove all spoils wreckage, rubbish, or temporary structures (such as buildings, shelters, and latrines) which are no longer required; and (ii) All excavated roads shall be reinstated to original condition. (iii) All disrupted utilities restored (iv) All affected structures rehabilitated/compensated (v) The area that previously housed the construction camp is to be checked for spills of substances such as oil, paint, etc. and these shall be cleaned up. (vi) All hardened surfaces within the construction camp area shall be ripped, all imported materials removed, and the area shall be top soiled and re-grassed using the guidelines set out in the re-vegetation specification that forms part of this document. (vii) The contractor must arrange the cancellation of all temporary services. (viii) Request PIU to report in writing that worksites and camps have been vacated and restored to pre-project conditions before acceptance of work. | - Stockpile Management - Spoil Management - Restoration of sites | Review of documents and site inspections | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Complied; About 500 km pipe laying has been laid. Spoil Management Plan has been submitted for the pipe laying work. No utilities affected during pipe laying at village roads. All excavated road reinstated to original condition |  | Complied; About 450 km pipe laying has been laid. Spoil Management Plan has been submitted for the pipe laying work. No utilities affected during pipe laying at village roads. All excavated road reinstated to original condition |  |

Table 11: Summary of Environmental Monitoring Activities for the Package WBDWSIP/DWW/NCB/BK/01/2017-18 and WBDWSIP/DWW/NCB/BK/03/2018-19: Bulk Water Supply for Bankura

| Impacts (List from IEE) | Mitigation Measures (List from IEE) | Parameters Monitored (As a minimum those identified in the IEE should be monitored) | Method of Monitoring | Location of Monitoring | Date of Monitoring Conducted | Name of Person Who Conducted the Monitoring | Compliance Status/ Explanation for BK 01 | Compliance Status/ Explanation for BK03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design Phase |  |  |  |  |  |  |  |  |  |
| Source sustainability and efficiency | - Discontinuation of current unsafe and unsustainable groundwater sources and creating a new comprehensive surface water(river)based water supply system - Recovering wash water from treatment process to optimize the water use - Treatment and reuse of sludge from treatment process - Designing the entire system to maintain optimal flow and terminal pressure, and optimizing the overall energy usage - Reducing the incidence of water borne diseases by providing 100% population including urban poor with potable water supplies - Preparation and implementation of a water quality surveillance program including development of a laboratory as part of the project by DBO contractor to ensure that supplied water meets the drinking water standards - Development of laboratory with all necessary environment, health and safety measures and adopting international standard procedures for water quality testing - Using low-noise and energy efficient pumping systems - Installing the noise-producing pumps and motors etc.in enclosed buildings with noise reducing walls, and also maintaining adequate buffer to the nearby inhabited areas. Provision of appropriate personal protection equipment to the workers and staff | - Design philosophy - Treatment scheme - Project QA/QC plan | Document review and LOP Survey | All project locations | Before Commencement and during final design | Environment Specialist of DSISC, PIU and PMU/PMC | Detailed design under process, necessary points as mentioned in IEE are considered during finalization of design | Detailed design completed as per guideline of IEE. The water treatment process work will be continuing without disturbing the ground water source & optimizing the overall energy. Before supplying of treated drinking water, it will ensure the water test as per IS standard by testing of water parameter at Laboratory |
| Chlorine handling and application risk– health and safety risk to workers and general public | - Provide the following measure at the chlorine application unit: - Chlorine neutralization pit with a lime slurry feeder - Chlorine absorption and neutralization facility - Proper ventilation, lighting, entry and exit facilities - Visible and audible alarm facilities to alert chlorine gas leak - Facility for isolation in the event of major chlorine leakage - Eye wash and shower facility - Personal protection and safety equipment for the operators in the chlorine plant (masks, oxygen cylinders, gloves, etc.,) - Provide training to the staff in safe handling and application of chlorine; this shall be included in the contract of Chlorinator supplier - Supplier of Chlorinator equipment shall provide standard operating manual for safe operation and as well as maintenance and repairs; preferably these shall be provided both in English and Bengali Languages | - Project emergency management plan - Project safety and PPE plan - Training plan | Document review | WTP and Booster pumping site | Before Commencement and during final design | Environment Specialist of DSISC and PIU | Detailed design under process, necessary safety during storage and use of chlorine is considered during designing of WTP | Considered in design- at the chlorination room chlorine neutralization pit & leakage detector provided for identification purpose. The adequate lighting, ventilation, proper access & egress provide at chlorination room. The Eye wash, Shower, PPEs, SCADA & handling manual in local language is provide in design. |
| Tree cutting | - Minimize removal of trees by adopting to site condition and with appropriate layout design of WTP - Obtain prior permission for tree cutting - Plant and maintain 5 trees for each tree that is removed | - Tree felling requirement and - afforestation after final design | - Checking of records - Visual inspection of sites | Project locations | Before Commencement and during final design | Environment Specialist of DSISC and PIU | As of now, tree felling requirement is not envisaged at any project locations. | Total 232 number of tree felling required at WTP site. Necessary permission obtained from Forest Department (Appendix 5) Tree felling will be done |
| Disturbance to natural drainage | - Diversion of drainage system - Minimization of impact on drainage basin | - Location map - Design philosophy | - Checking of records - Visual inspection of sites | Project locations | Before Commencement and during final design | Environment Specialist of DSISC, PIU and PMU/PMC | Under compliance- during final designing | Under compliance- during implementation |
| Pre-Construction Phase |  |  |  |  |  |  |  |  |  |
| Telephone lines, electric poles and wires, water lines within proposed project area | - Identify and include locations and operators of these utilities in the detailed design documents to prevent unnecessary disruption of services during construction phase; and - Require construction contractor to prepare a contingency plan to include actions to be taken in case of unintentional interruption of services. - Require contractors to prepare spoils management plan and traffic Management plan | List of affected utilities if any and operators | - Observation and - Document checking | Specific project location | Before commencement of construction | Environment Specialist of DSISC and PIU | The overhead LT power supply line pass over WTP area. Communication made for shifting of LT line. Procedure has been initiated. Spoil and Traffic management plan submitted. | The overhead LT power supply line pass over WTP area. Communication made for shifting of LT line. Permission received. Spoil and Traffic management plan submitted. |
| Conflicts with local community; disruption to traffic flow and sensitive receptors | - Prioritize areas within or nearest possible vacant space in the project location; - If it is deemed necessary to locate elsewhere, consider sites that will not promote instability and result in destruction of property, vegetation, irrigation, and drinking water supply systems. - Not to consider residential areas. - Take extreme care in selecting sites to avoid direct disposal to water body which will inconvenience the community. - For excess spoil disposal, ensure (a) site shall be selected preferably from barren, infertile lands. In case agricultural land needs to be selected, written consent from landowners (not lessees) will be obtained; (b) debris disposal site shall be at least 200 m away from surface water bodies; (c) no residential areas shall be located within 50 m downwind side of the site; and (d) site is minimum 250 m away from sensitive locations like settlements ponds/lakes or other water bodies | - List of selected project location and proposed pipeline alignment plan - Involvement of traffic dept. - Road closure planning | - Site observation - Review of documents - Grievance Register | Specific project location | Before commencement of final design and commencement of construction | Environment Specialist of DSISC and PIU | The area selected for construction work camp is vacant government land and free from any conflict. Pipe laying will be done after finalization of design and alignment | The area selected for construction work camp is vacant government land and free from any conflict. |
| Extraction of materials can disrupt natural land contours and vegetation resulting in accelerated erosion, disturbance in natural drainage patterns, ponding and water logging, and water pollution. | - Obtain construction materials only from government approved quarries with prior approval of PIU; - PIU to review, and ensure that proposed quarry sources have all necessary clearances/ permissions in place prior to approval - Contractor to submit to PIU on a monthly basis documentation on material obtained from each source (quarry/ borrow pit) - Avoid creation of new borrow areas, quarries etc., for the project; if unavoidable, contractor to obtain all clearances and permissions as required under law, including Environmental Clearance prior to approval by PIU | • List of approved quarry sites and sources of materials • Construction Contractor documentation | - Checking of records - Visual Inspection of sites | Project sites | Before commencement of construction | DSISC Construction Management and Environmental Safeguard Team | All materials procured from licensed vendors after approval of PIU | All materials procured from licensed vendors after approval of PIU |
| Failure to obtain necessary consents, permits, NOCs, etc. can result to design revisions and/or stoppage of works | - Obtain all necessary consents, permits, clearance, NOCs, etc. prior to award of civil works. - Ensure that all necessary approvals for construction to be obtained by contractor are in place before start of construction - Acknowledge in writing and provide report on compliance all obtained consents, permits, clearance, NOCs, etc. - Include in detailed design drawings and documents all conditions and provisions if necessary | List of applicable legislation | Checking of documents | All project locations | Before commencement of construction | Environment Specialist of DSISC, PIU and PMU/PMC | All necessary approval, NOC, Clearance & permits has been initiated for safely execution of work at site. | CTE for WTP (Appendix 5A), land possession certificate (Appendix 20) received. Others necessary approval, NOC, clearance & permits has been initiated for safely execution of work at site. |
| Health risk due to exposure to asbestos materials | - Obtain details on location of underground asbestos cement materials - Lay the new pipes carefully to avoid encountering asbestos cement pipes - If found, leave the asbestos cement pipes undisturbed in the ground. | Asbestos cement materials | Site inspection | Specific project location | Daily visit by Construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Till date no asbestos cement material has been found on site | Till date no asbestos cement material has been found on site |
| Construction Phase |  |  |  |  |  |  |  |  |  |
| Irreversible impact to the environment workers, and community | - Project manager and all key workers will be required to undergo training on EMP implementation including spoils/waste management, Standard operating procedures (SOP) for construction works; occupational health and safety (OHS), core labor laws, applicable environmental laws, etc. | Induction & Awareness Trainings Toolbox Talks Safeguard Trainings | Review of Training records Site Inspections | Project Locations | - | Environment Specialist of DSISC and PMU | Site Environmental Safety training and awareness will be arranged after commencement of work. | Complied; site Environmental Safety training and awareness arranged on regular basis (sample attached as Appendix 11) |
| Emissions from construction vehicles, equipment, and machinery used for installation of pipelines resulting to dusts and increase in concentration of vehicle-related pollutants such as carbon monoxide, sulfur oxides, particulate matter, nitrous oxides, and hydrocarbons. | For all construction works - Comply with the Direction of West Bengal Department of Environment under the Air Act, 1981 in controlling air pollution from construction activities - Comply with the air pollution / dust control measures for construction activities stipulated by the “Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7/001/2009 dated: 10 December 2009” - Damp down the soil and any stockpiled material on site by water sprinkling; - Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; - Provide a dust screen around the construction sites at GLSR and WTP work sites - Clean wheels and undercarriage of haul trucks prior to leaving construction site/quarry - Control dust generation while unloading the loose material (particularly aggregate, soil) at the site by sprinkling water and unloading inside the barricaded area - Stabilize surface soils where loaders, support equipment and vehicles will operate by using water and maintain surface soils in a stabilized condition - Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; - Apply water and maintain soils in a visible damp or crusted condition for temporary stabilization - Apply water prior to leveling or any other earth moving activity to keep the soil moist throughout the process - Cover the soil stocked at the sites with tarpaulins - Control access to work area, prevent unnecessary movement of vehicle, public trespassing into work areas; limiting soil disturbance will minimize dust generation - Ensure that all the construction equipment, machinery is fitted with pollution control devises, which are operating correctly, and have a valid pollution under control (PUC) certificate Pipeline works - Barricade the construction area using hard barricades (of 2 m height) on both sides and provide dust/wind screen (such geo textile fabric) up to 3 m height (1m above the hard barricading) - Initiate site clearance and excavation work only after barricading of the site is done - Confine all the material, excavated soil, debris, equipment, machinery (excavators, cranes etc.,), to the barricaded area - Limit the stocking of excavated material at the site; remove the excess soil from the site immediately to the designated disposal area - Undertake the work section wise: 100 – 200 m section should be demarcated and barricaded - Conduct work sequentially - excavation, pipe laying, backfilling; conduct pipe testing section-wise (for a minimum length as possible) so that backfilling, stabilization of soil can be done. - Remove the excavated soil of first section to the disposal site; as the work progresses, sequentially, by the time second section is excavated, the first section will be ready for back filling, use the freshly excavated soil for back filling, this will avoid stocking of material, and minimize the dust. - Backfilled trench at any completed section after removal of barricading will be the main source of dust pollution. The traffic, pedestrian movement and wind will generate dust from backfilled section. Road restoration shall be undertaken immediately. | - Location of stockpiles - Complaints from sensitive receptors - Monitoring data- PM10, PM2.5 NO2, SO2, CO - Heavy equipment and machinery with air pollution control - Water sprinkling arrangement - Cover materials | - Site inspection - Public grievance register | Covering different locations of WTP, Intake, pumping stations as per monitoring plan. | Air – noise monitoring: Once before start of construction Yearly 3 times during construction (3-year period considered) Monitoring for BK 01 package done during earlier report period. For BK 03 monitoring done for baseline and during construction in November 2019 and February 2020 | DSISC Construction Management and Environmental Safeguard Team, PIU | Partially complied; pre-construction baseline data has been collected, except surface water monitoring result. Results attached as Appendix 10. Collection of PUC certificate under process. Relevant regulation under compliance. Other activities like dust suppression, covering of loose materials, dust screen will be arranged. For pipe line work, barricading, removal of earth, backfilling will be done as per SEMP | Complied; pre-construction baseline data and during construction monitoring data has been collected results attached as Appendix 10. Emission of equipment and vehicle tested. PUC certificate obtained for Vehicle (Appendix 2) Relevant regulation under compliance. Other activities like dust suppression, covering of loose materials, dust screen have arranged. For pipe line work, barricading, removal of earth, backfilling has done as per SEMP. |
| Mobilization of settled silt materials, and chemical contamination from fuels and lubricants during construction can Contaminate nearby surface water quality. Ponding of water in the pits /foundation excavations | - All earthworks be conducted during the dry season to prevent the problem of soil run-off during monsoon season; - Avoid stockpiling of earth fill especially during the monsoon season unless covered by tarpaulins or plastic sheets; - Prioritize re-use of excess spoils and materials in the construction works. If spoils will be disposed, only designated disposal areas shall be used; - Install temporary silt traps or sedimentation basins along the drainage leading to the waterbodies; - Place storage areas for fuels and lubricants away from any drainage leading to water bodies; - Store fuel, construction chemicals etc., on an impervious floor, also avoid spillage by careful handling - Dispose any wastes generated by construction activities in designated sites; - Conduct surface quality inspection according to the Environmental Management Plan (EMP). - Create a temporary drainage channel around the work area to arrest the entry of runoff from upper areas into the work area - Pump out the water collected in the pits / excavations to a temporary sedimentation pond; dispose of only clarified water into drainage channels/streams after sedimentation in the temporary ponds - Consider safety aspects related to pit collapse due to accumulation of water | • Areas for stockpiles, storage of fuels and lubricants and waste materials • Number of silt traps installed along drainages (in slope) leading to water bodies • Entry routes of pollutant in nearby waterbodies | Site inspection Public grievance register | All project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | DSISC Construction Management and Environmental Safeguard Team | Work not yet started. Silt trap will be arranged, designated site for waste disposal under consideration. All safety aspect will be maintained. | Work started. Silt trap arranged, designated site for waste disposal selected and approved. All safety aspects are maintained. |
| Increase in noise level due to earth-moving and excavation equipment, and the transportation of equipment, materials, and people | - Plan activities in consultation with PIU so that activities with the greatest potential to generate noise are conducted during periods of the day which will result in least disturbance; - Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; - Minimize noise from construction equipment by using vehicle silencers, fitting jackhammers with noise-reducing mufflers, and use portable street barriers to minimize sound impact to surrounding sensitive receptor; and - Maintain maximum sound levels not exceeding 80 decibels (dBA) when measured at a distance of 10 m or more from the vehicle/s. - Identify any buildings at risk from vibration damage and avoiding any use of pneumatic drills or heavy vehicles in the vicinity - Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; - Consult local communities in advance of the work to identify and address key issues, and avoid working at sensitive times, such as religious and cultural festivals. | Day time and night time noise levels. | - Checking of records - Visual inspection of sites | Covering different locations of WTP, Intake, pumping stations as per monitoring plan. | Noise monitoring: Once before start of construction Yearly 3 times during construction (3-year period considered) Monitoring for BK 01 package done during earlier report period. For BK 03 monitoring done for baseline and during construction in November 2019 and February 2020 | DSISC Construction Management and Environmental Safeguard Team | Partially complied; pre-construction baseline data has been collected except surface water monitoring result. There is no as such noise generated from equipment. Stipulated condition as per SEMP will be followed | Complied; pre-construction baseline and during construction data has been collected. There is no as such noise generated from equipment. Stipulated condition as per SEMP is followed |
| Impacts due to excess excavated earth, excess construction materials, and solid waste such as removed concrete, wood, packaging materials, empty containers, spoils, oils, lubricants, and other similar items. | - Prepare and implement a Construction Waste Management Plan - As far as possible utilize the debris and excess soil in construction purpose, for example for raising the ground level or construction of access roads etc., - Stockpiles, lubricants, fuels, and other materials should be located away from steep slopes and water bodies; - Avoid stockpiling any excess spoils at the site for long time. Excess excavated soils should be disposed off to approved designated areas immediately; - If disposal is required, the site shall be selected preferably from barren, infertile lands; site should be located away from residential areas, forests, water bodies and any other sensitive land uses - Domestic solid wastes should be properly segregated in biodegradable and non-biodegradable for collection and disposal to designated solid waste disposal site; create a compost pit at workers’ camp sites for disposal of biodegradable waste; non-biodegradable / recyclable material shall be collected separately and sold in the local recycling material market - Residual and hazardous wastes such as oils, fuels, and lubricants shall be disposed of in disposal sites approved by local authorities/WBPCB; - Prohibit burning of construction and/or domestic waste; - Ensure that wastes are not haphazardly dumped thrown within and around the project site and adjacent areas; provide proper collection bins, and create awareness to use the dust bins. - Conduct site clearance and restoration to original condition after the completion of construction work; PIU to ensure that site is properly restored prior to issuing of construction completion certificate | • Waste Management List • Stockpile Management • Complaints from Sensitive receptors • PMU/ PIU/ DSISC to report inwriting that the necessary environmental restoration work has been done | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | Excess earth will be used mostly for backfilling Domestic solid waste collection bin will be arranged Burning of waste has been strictly prohibited in site. | Complied Excess earth is being used for backfilling Domestic solid waste collection bin arranged. Burning of waste has been strictly prohibited in site. |
| Disruption of service and Damage to existing infrastructure at specified project location | - Prepare a list of affected utilities and operators if any; - Prepare a contingency plan to include actions to be done in case of unintentional interruption of service | • List of affected utilities if any and operators • Public grievance | Observation and document checking | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | Will be done as per requirement. At present, there is no any disruption of service and damage the existing infrastructure | Complied as Per requirement. At present, there is no any disruption of service and damage the existing infrastructure. |
| Loss of vegetation and tree cover | - Minimize removal of vegetation and disallow cutting of trees; - If tree-removal will be required, obtain tree-cutting permit and - Plant 5 native trees for every one that is removed. | Tree felling requirement and afforestation after final design | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU/PMC | As of now, tree felling requirement is not envisaged. | Measures taken for minimize number of tree felling. Tree felling anticipated in WTP site only. Necessary permission obtained from Forest Department (Appendix 5) |
| Traffic problems and conflicts near project locations and haul road | Hauling (material, waste/debris and equipment) activities - Plan transportation routes so that heavy vehicles do not use narrow local roads, except in the immediate vicinity of delivery sites - Schedule transport and hauling activities during non-peak hours; - Locate entry and exit points in areas where there is low potential for traffic congestion; - Drive vehicles in a considerate manner - Notify affected public by public information notices, providing sign boards informing nature and duration of construction works and contact numbers for concerns / complaints. Pipeline works - Confine work areas along the roads to the minimum possible extent; all the activities, including material and waste/surplus soil stocking should be confined to this area. Proper barricading should be provided; avoid - material/surplus soil stocking in congested areas – immediately removed from site/ or brought to the as and when required. - Leave spaces for access between mounds of soil to maintain access to the houses / properties - Provide pedestrian access in all the locations; provide wooden/metal planks over the open trenches at each house to maintain the access. - Inform the affected local population 1-week in advance about the work schedule - Plan and execute the work in such a way that the period of disturbance/ loss of access is minimum. - Keep the site free from all unnecessary obstructions; - Coordinate with Traffic Police for temporary road diversions, where necessary, and for provision of traffic aids if transportation activities cannot be avoided during peak hours | • Traffic Management Plan • Public grievance • Number of signages placed at subproject location | Site visit and document review | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | No work started. | Haul road prepared for transportation of material to WTP site. For intake point, there is only one road which is used for transportation of material and for communication of villagers and traffic. Adequate signage, safety steward is implemented during construction. Local villagers are informed in advance to avoid the inconvenience. Traffic plan has been submitted for pipe laying work. Local people are informed in advance in every case. Construction work is limited by considering the convenience of local people. |
| Generation of temporary employment and increase in local revenue | - Employ local labor force as far as possible - Comply with labor laws | Employment record | Checking of records | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC, PIU and PMU / PMC | Direct and indirect employment for local population will be ensured. | Maximum number of local people are engaged in site. |
| Occupational hazards which can arise during work | Comply with all national, state and local core labor laws (see Appendix 7of this IEE) - Develop and implement site-specific occupational health and safety (OHS) Plan which will include measures such as: (a) excluding public from the site; (b) ensuring all workers are provided with and use personal protective equipment like helmet, gumboot, safety belt, gloves, nose musk and ear plugs; (c) OHS Training for all site personnel; (d) documented procedures to be followed for all site activities; and (e) documentation of work-related accidents; - Ensure that qualified first-aid can be provided at all times. Equipped first-aid stations shall be easily accessible throughout the site; - Provide medical insurance coverage for workers; - Secure all installations from unauthorized intrusion and accident risks; - Provide supplies of potable drinking water; - Provide clean eating areas where workers are not exposed to hazardous or noxious substances; - Provide health and safety orientation training to all new workers to ensure that they are apprised of the basic site rules of work at the site, personal protective protection, and preventing injuring to fellow workers; - Provide visitor orientation if visitors to the site can gain access to areas where hazardous conditions or substances may be present. Ensure also that visitor/s do not enter hazard areas unescorted; - Ensure the visibility of workers through their use of high visibility vests when working in or walking through heavy equipment operating areas; - Ensure moving equipment is outfitted with audible back-up alarms; - Mark and provide sign boards for hazardous areas such as energized electrical devices and lines, service rooms housing high voltage equipment, and areas for storage and disposal. Signage shall be in accordance with international standards and be well known to, and easily understood by workers, visitors, and the general public as appropriate; and - Disallow worker exposure to noise level greater than 85 dBA for a duration of more than 8 hours per day without hearing protection. The use of hearing protection shall be enforced actively. | - Site-specific Health and Safety (H&S) Plan - Equipped first-aid stations; - Medical insurance coverage for workers - Number of accidents - Supplies of potable drinking water; - Record of H&S orientation trainings - Personal protective equipment - Sign boards for hazardous areas such as energized electrical devices and lines, service rooms | • Checking of records • Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC; PIU and PMU/PMC | Work not started. Site-specific Health and Safety (H&S) submitted and approved by DSISC and PIU. OHS plan submitted by contractor and approved by DSISC and PIU. Induction and tool box training will be arranged by contractor. Use of PPEs will be ensured at site. Drinking water, first aid box will be available at site. WC Policy renewal under process (validity upto 29th April 2020, Appendix 4). Medical tie up done with local hospital. No as such noise producing equipment available at site. Accident register will be maintained at site. | Complied. Site-specific Health and Safety (H&S) Plan under implementation. OHS plan submitted by contractor and approved by DSISC and PIU. That document is available with DSISC and PIU. The existing H&S plan would be revisited to include preventive and mitigative measures with respect to COVID-19. Induction and tool box training arranged by contractor (sample shown in Appendix 11) Use of PPEs noted mostly. Visibility vest also used. Drinking water, first aid box, hand sanitizer, face masks etc. are available at site. Medical Insurance arranged for the laborers (see Appendix 4). Medical tie up and health checking of workers done. No as such noise producing equipment available at site. Accident register is maintained at site. Till date no accident recorded. |
| Health risk due to exposure to asbestos materials | - Obtain details on location of underground asbestos cement materials - Lay the new pipes carefully to avoid encountering asbestos cement pipes - If found, leave the asbestos cement pipes undisturbed in the ground. | asbestos cement materials | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Till date no asbestos cement material has been found on site | Till date no asbestos cement material has been found on site |
| Traffic accidents and vehicle collision with pedestrians during material and waste transportation | - Restrict construction vehicle movements to defined access roads and demarcated working areas (unless in the event of an emergency) - Enforce strict speed limit (20-30 kmph) for playing on unpaved roads, construction tracks - Night-time haulage will be by exception only, as approved by the PIU to minimize driving risk and disturbance to communities - Adopt standard and safe practices for micro tunneling - Temporary traffic control (e.g. flagmen) and signs will be provided where necessary to improve safety and provide directions - All drivers will undergo safety and training - Public access to all areas where construction works are on-going will be restricted through the use of barricading and security personnel - Warning signs, blinkers will be attached to the barricading to caution the public about the hazards associated with the works, and presence of deep excavation - The period of time when the pipeline trench are left open will be minimized through careful planning - Control dust pollution – implement dust control measures as suggested under air quality section - Maintain regularly the vehicles and use of manufacturer-approved parts to minimize potentially serious accidents caused by equipment malfunction or premature failure. - Provide road signs and flag persons to warn of on-going trenching activities. | Public grievance | Review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | No work started. | Haul road prepared for transportation of material to WTP site. For intake point, there is only one road which is used for transportation of material and for communication of villagers and traffic. Adequate signage, safety steward is implemented during construction. Local villagers are informed in advance to avoid the inconvenience. Traffic plan has been submitted for pipe laying work. Local people are informed in advance in every case. Construction work is limited by considering the convenience of local people. |
| Temporary air and noise pollution from machine operation, water pollution from storage and use of fuels, oils, solvents, and lubricants Unsanitary and poor living conditions for workers | - Ensure that a proper compound wall is provided, and erect a wind/dust screen around - Camp site shall not be located near (100 m) water bodies, flood plains flood prone/low lying areas, or any ecologically, socially, archeologically sensitive areas - Separate the workers living areas and material storage areas clearly with a fencing and separate entry and exit - Provide proper temporary accommodation with proper materials, adequate lighting and ventilation, appropriate facilities for winters and summers; ensure conditions of livability at work camps are maintained at the highest standards possible at all times; - Consult PIU before locating project offices, sheds, and construction plants; (viii)Minimize removal of vegetation and disallow cutting of trees - Ensure conditions of livability at work camps are maintained at the highest standards possible at all times; living quarters and construction camps shall be provided with standard materials (as far as possible to use portable ready to fit-in reusable cabins with proper ventilation); thatched huts, and facilities constructed with materials like GI sheets, tarpaulins, etc., shall not be allowed as accommodation for workers - Camp shall be provided with proper drainage, there shall not be any water accumulation - Provide drinking water, water for other uses, and sanitation facilities for employees - Prohibit employees from cutting of trees for firewood; contractor should be provided proper facilities including cooking fuel (oil or gas; fire wood not allowed) - Train employees in the storage and handling of materials which can potentially cause soil contamination - Recover used oil and lubricants and reuse or remove from the site - Manage solid waste according to the following preference hierarchy: reuse, recycling and disposal to designated areas; provide a compost pit for biodegradable waste, and non-biodegradable / recyclable waste shall be collected and sold in local market - Remove all wreckage, rubbish, or temporary structures which are no longer required - At the completion of work, camp area shall be cleaned and restored to pre-project conditions, and submit report to PIU; PIU to review and approve camp clearance and closure of work site | • Public grievance • Accommodation • Water and sanitation facilities for employees • Housekeeping – regular disposal of solid waste | Site inspection and review of documents | Construction camps | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Not yet established. | Complied. Housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking, provision of solid waste and wastewater disposal are properly maintained. Safe distance is maintained at camp for protection against COVID-19. |
| There are no protected properties in the subproject sites. However, in case of chance finds, contractors will be required to follow a protocol as defined in the mitigation measures. | - Consult Archaeological Survey of India (ASI) or West Bengal State Archaeology Department to obtain an expert assessment of the archaeological potential of the site. - Include state and local archaeological, cultural and historical authorities, and interest groups in consultation forums as project stakeholders so that their expertise can be made available. - In case of chance finds, works must be stopped immediately until such time chance finds are cleared by experts | Site inspection records | Site inspection and review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PMU/PMC | No chance finds to date. | No chance finds to date. |
| Unsatisfactory compliance to EMP | - Appointment of (I) Environment, Health and Safety (EHS) Supervisor to ensure EMP implementation - Timely submission of monitoring reports including pictures | Appointment letter Monitoring records | Review of records | - | - | Environment Specialist of DSISC and PIU | Partially complied More training is required for safety officer on Environment aspect | Complied. |
| Damage due to debris, spoils, excess construction materials | - Remove all spoils wreckage, rubbish, or temporary structures (such as buildings, shelters, and latrines) which are no longer required; and - All excavated roads shall be reinstated to original condition. - All disrupted utilities restored - All affected structures rehabilitated/compensated - The area that previously housed the construction camp is to be checked for spills of substances such as oil, paint, etc. and these shall be cleaned up. - All hardened surfaces within the construction camp area shall be ripped, all imported materials removed, and the area shall be top soiled and re-grassed using the guidelines set out in the revegetation specification that forms part of this document. - The contractor must arrange the cancellation of all temporary services. quest PIU to report in writing that worksites and camps have been vacated and restored to pre-project conditions before acceptance of work. | Stockpile Management Spoil Management Restoration of sites | Review of documents and site inspections | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Safeguard Team. | Environment Specialist of DSISC and PIU | Complied; Spoil Management Plan has been submitted. No pipe laying work started. | Complied; Spoil Management Plan has been submitted as part of SEMP |

Table 12: Summary of Environmental Monitoring Activities for the Package WBDWSIP/DWW/NCB/ BK/02A/2018-19, WBDWSIP/DWW/NCB/BK/02B/2018-19, and WBDWSIP/DWW/NCB/BK/04/2018-19- Water Supply Distribution

| Impacts | Mitigation Measures | Parameters Monitored (As a minimum those identified in the IEE should be monitored) | Method of Monitoring | Location of Monitoring | Date of Monitoring Conducted | Name of Person Who Conducted the Monitoring | Compliance Status/ Explanation for BK 02A | Compliance Status/ Explanation for BK 02B | Compliance Status/ Explanation for BK 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design Phase |  |  |  |  |  |  |  |  |  |
| Source sustainability and efficiency | (i) Gravity distribution system: designing the entire system to maintain optimal flow and terminal pressure, and optimizing the overall energy usage (ii)Implementation of a water quality surveillance program including development of a laboratory as part of the project to ensure that supplied water meets the drinking water standards (iii)Minimizing water losses from pipelines by perfect jointing and alignments using appropriate techniques (iv) Reducing the incidence of water borne diseases by providing 100% population including urban poor with potable water supplies | - Design philosophy - Project QA/QC plan - Selection methodology for distribution network | Document review and LOP Survey | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Complied; Transmission main network finalized for the following zones: 1, 12, 13, 18, 19, 20 under Package 2A and 16, 20 and 21 under Package 2B. Rest under progress. | Complied; Distribution network finalized for the following zones: 1,3 and 15 Rest under progress. | Complied; Distribution network finalized for the following zones: 5 and 20. Rest under progress. |
| Socio economic impact – loss fishery area | (i) Avoid using low-lying lands / ponds for construction of OHRs; alternative private lands may be explored within the vicinity; (ii)Review the applicability of West Bengal Inland Fisheries Act, 1984, whether the site falls under the definition of fisher area; obtained permission from Fisheries Department if required prior to start of construction | List of selected location for OHRs | Site survey | All OHR sites | Before commencement of final design | Environment Specialist of DSISC and PMC | Complied; No low-lying lands or ponds are being filled for construction. | Complied; No low-lying lands or ponds are being filled for construction. | Complied; No low-lying lands or ponds are being filled for construction. |
| Tree cutting | (i) Minimize removal of trees by adopting to site condition and with appropriate layout design of OHRs within the sites (ii) Avoid cutting of trees by adopting suitable alignment changes as required during laying of pipelines; (iii)I In unavoidable cases, obtain prior permission for tree cutting (iv) Plant and maintain 5 trees for each tree that is removed | - Tree felling requirement – site layout plan - NOC – paper documents from line agency | Site survey and review of site layout/ pipeline alignment plan | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Complied; no tree felling has been envisaged till date. | Complied; no tree felling has been envisaged till date. | Complied; no tree felling has been envisaged till date. |
| Pre-Construction Phase |  |  |  |  |  |  |  |  |  |
| Telephone lines, electric poles and wires, water lines within proposed project area | (i) Identify and include locations and operators of these utilities in the detailed design documents to prevent unnecessary disruption of services during construction phase; (ii) Require construction contractors to prepare a contingency plan to include actions to be taken in case of unintentional interruption of services. (iii) Require contractors to prepare spoils (waste) management plan and traffic management plan | List of affected utilities if any and operators | Observation and document checking | Specific project location | Before commencement of construction | Environment Specialist of DSISC and PIU | Complied; about 17 km of transmission main and 3 km distribution network pipeline has been laid (sample Spoil Management Plan & Traffic Management Plan for one zone available with SEMP, Appendix 7) | Complied; about 5 km of distribution main pipeline has been laid (sample Spoil Management Plan & Traffic Management Plan for one zone available with SEMP, Appendix 7) | Complied; about 10 km of distribution main pipeline has been laid (sample Spoil Management Plan & Traffic Management Plan for one zone available with SEMP, Appendix 7) |
| Conflicts with local community; disruption to traffic flow and sensitive receptors | (i) Prioritize areas within or nearest possible vacant space in the project location; (ii) If it is deemed necessary to locate elsewhere, consider sites that will not promote instability and result in destruction of property, vegetation, irrigation, and drinking water supply systems; (iii) Do not consider residential areas; (iv) Take extreme care in selecting sites to avoid direct disposal to water body which will inconvenience the community. (v) For excess spoil disposal, ensure (a) site shall be selected preferably from barren, infertile lands. In case agricultural land needs to be selected, written consent from landowners (not lessees) will be obtained; (b) debris disposal site shall be at least 200 m away from surface water bodies; (c) no residential areas shall be located within 50 m downwind side of the site; and (d) site is minimum 250 m away from sensitive locations like settlements, ponds/lakes or other water bodies. | - List of selected location for OHRs - Involvement of traffic dept. - Road closure planning | - Site observation - Review of documents - Grievance Register | Specific project location | Before commencement of final design and commencement of construction | Environment Specialist of DSISC; PIU and PMC | Complied, No disruption noted. Area selected nearby vacant place. No excess spoil generated. Excess earth utilized for back filling. No complete road closure expected. Partial closure noted, which mentioned in traffic management plan | Complied, No disruption noted. Area selected nearby vacant place. No excess spoil generated. Excess earth utilized for back filling. No complete road closure expected. Partial closure noted, which mentioned in traffic management plan | Complied, No disruption noted. Area selected nearby vacant place. No excess spoil generated. Excess earth utilized for back filling. No complete road closure expected. Partial closure noted, which mentioned in traffic management plan |
| Extraction of materials can disrupt natural land contours and vegetation resulting in accelerated erosion, disturbance in natural drainage patterns, ponding and water logging, and water pollution. | (i) Obtain construction materials only from government approved quarries with prior approval of PIU; (ii) PIU to review, and ensure that proposed quarry sources have all necessary clearances/ permissions in place prior to approval (iii) Contractor to submit to PIU on a monthly basis documentation on material obtained from each source (quarry/ borrow pit) (iv) Avoid creation of new borrow areas, quarries etc., for the project; if unavoidable, contractor to obtain all clearances and permissions as required under law, including Environmental Clearance prior to approval by PIU | - List of approved quarry sites and sources of materials - Construction Contractor documentation | - Checking of records - Visual inspection of sites | Quarries and material source areas | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC at operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied. Approval obtained from DSISC, PIU before procurement. | Complied. Approval obtained from DSISC, PIU before procurement. | Complied. Approval obtained from DSISC, PIU before procurement. |
| Failure to obtain necessary consents, permits, NOCs, etc. can result to design revisions and/or stoppage of works | (i) Obtain all necessary consents, permits, clearance, NOCs, etc. prior to award of civil works. (ii) Ensure that all necessary approvals for construction to be obtained by contractor are in place before start of construction (iii) Acknowledge in writing and provide report on compliance all obtained consents, permits, clearance, NOCs, etc. (iv) Include in detailed design drawings and documents all conditions and provisions if necessary | List of applicable legislation | Checking of documents | All project locations | Before commencement of construction | Environment Specialist of DSISC and PMC | Complied and to be continued as per requirement | Complied and to be continued as per requirement | Complied and to be continued as per requirement |
| Health risk due to exposure to asbestos materials | (i) Obtain details on location of asbestos cement materials (ii) Lay the new piper carefully to avoid encountering AC pipes (ii) Leave the AC pipes undisturbed in the ground. | Asbestos cement materials | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC at operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied. Till date no rubbish containing asbestos cement has been found | Complied. Till date no rubbish containing asbestos cement has been found | Complied. Till date no rubbish containing asbestos cement has been found |
| Construction phase |  |  |  |  |  |  |  |  |  |
| Irreversible impact to the environment, workers, and community | (i) Project manager and all key workers will be required to undergo training on EMP implementation including spoils/waste management, Standard operating procedures (SOP) for construction works; occupational health and safety (OHS), core labor laws, applicable environmental laws, etc. | Induction & Awareness Trainings Toolbox Talks Safeguard Trainings | Review of Training records Site Inspections | Project Locations | - | Environment Specialist of DSISC: PIU and PMC | Under compliance; Site Environmental Safety training and awareness arranged by contractor on regular basis (Sample attached as Appendix 11) Safeguard orientation/ Audit program arranged by PMC (Appendix 18) | Under compliance; Site Environmental Safety training and awareness arranged by contractor on regular basis (Sample attached as Appendix 11) Safeguard orientation / Audit program arranged by PMC (Appendix 18) | Under compliance; Site Environmental Safety training and awareness arranged by contractor on regular basis (Sample attached as Appendix 11) Safeguard orientation/ Audit program arranged by PMC (Appendix 18) |
| Emissions from construction vehicles, equipment, and machinery used for installation of pipelines resulting to dusts and increase in concentration of vehicle-related pollutants such as carbon monoxide, sulfur oxides, particulate matter, nitrous oxides, and hydrocarbons. | For all construction works (i) Comply with the air pollution / dust control measures for construction activities stipulated by the “Direction of West Bengal Department of Environment under the Air Act, 1981 Direction No. EN/3170/T-IV-7 /001/2009 dated: 10 December 2009” (ii) Damp down the soil and any stockpiled material on site by water sprinkling; (iii)Use tarpaulins to cover the loose material (soil, sand, aggregate etc.,) when transported by trucks; (iv) Provide a dust screen/high compound wall around the construction sites (OHRs) (v) Clean wheels and undercarriage of haul trucks prior to leaving construction site/quarry (vi) Control dust generation while unloading the loose material (particularly aggregate, soil) at the site by sprinkling water and unloading inside the barricaded area (vii) Stabilize surface soils where loaders, support equipment and vehicles will operate by using water and maintain surface soils in a stabilized condition (viii) Apply water prior to leveling or any other earth moving activity to keep the soil moist throughout the process (ix) Control access to work area, prevent unnecessary movement of vehicle, public trespassing into work areas; limiting soil disturbance will minimize dust generation (x) Ensure that all the construction equipment and machineries are fitted with pollution control devises, which are operating correctly, and have a valid pollution under control (PUC) certificate Pipeline works (i) Barricade the construction area (ii) Initiate site clearance and excavation work only after barricading of the site is done (iii)Confine all the material, excavated soil, debris, equipment, machinery (excavators, cranes etc.,), to the barricaded area (iv) Limit the stocking of excavated material at the site; remove the excess soil from the site immediately to the designated disposal area (v) Undertake the work section wise (vi)Conduct work sequentially - excavation, pipe laying, backfilling; conduct pipe testing section-wise (for a minimum length as possible) so that backfilling, stabilization of soil can be done. (vii) Remove the excavated soil of first section to the disposal site; as the work progresses, sequentially, by the time second section is excavated, the first section will be ready for back filling, use the freshly excavated soil for back filling, this will avoid stocking of material, and minimize the dust. (viii) Backfilled trench at any completed section after removal of barricading will be the main source of dust pollution. The traffic, pedestrian movement and wind will generate dust from backfilled section. Road restoration shall be undertaken immediately. | - Location of stockpiles - Complaints from sensitive receptors - Monitoring data- PM!0, PM2.5, SO2, NO2, CO - Heavy equipment and machinery with air pollution control - Water sprinkling Arrangement Cover materials | Site inspection Public grievance register | Project locations Air- monitoring as per selected sites in ref. to SEMP | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff Air- monitoring schedule as per SEMP Base line monitoring conducted during earlier report period in July 2019 and present report period in November 2019 | Environment Specialist of DSISC; PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Under compliance; About 20 km pipe line laid. Water sprinkling done as per requirement. Pre- construction air quality monitoring done as per IEE (Complete result certificates available in DSISC office as back up paper and attached as Appendix 10) Most of the PUC certificate obtained for Vehicle and Equipment (Appendix 2) Other activities like dust suppression, covering of loose materials, dust screen arranged. For pipe line work, barricading, removal of earth, backfilling done as per SEMP. | Under compliance; About 5 km pipe line laid. Water sprinkling done as per requirement. Pre- construction air quality monitoring done as per IEE (Complete result certificates available in DSISC office as back up paper and attached as Appendix 10) Most of the PUC certificate obtained for Vehicle and Equipment (Appendix 2) Other activities like dust suppression, covering of loose materials, dust screen arranged. For pipe line work, barricading, removal of earth, backfilling done as per SEMP. | Under compliance; About 10 km pipe line laid. Water sprinkling done as per requirement. Pre- construction air quality monitoring done as per IEE (Complete result certificates available in DSISC office as back up paper and attached as Appendix 10) Most of the PUC certificate obtained for Vehicle and Equipment (Appendix 2) Other activities like dust suppression, covering of loose materials, dust screen arranged. For pipe line work, barricading, removal of earth, backfilling done as per SEMP |
| Mobilization of settled silt materials, and chemical contamination from fuels and lubricants during construction can contaminate nearby surface water quality. Ponding of water in the pits / foundation excavations | (i) All earthworks be conducted during the dry season to prevent the problem of soil run-off during monsoon season; (ii) Avoid stockpiling of earth fill especially during the monsoon season unless covered by tarpaulins or plastic sheets; (iii)Prioritize re-use of excess spoils and materials in the construction works. If spoils will be disposed, only designated disposal areas shall be used; (iv) Install temporary silt traps or sedimentation basins along the drainage leading to the water bodies; (v) Place storage areas for fuels and lubricants away from any drainage leading to water bodies; (vi) Store fuel, construction chemicals etc., on an impervious floor, also avoid spillage by careful handling (vii) Dispose any wastes generated by construction activities in designated sites; and (viii) Conduct surface quality inspection according to the Environmental Management Plan (EMP). (ix) Create a temporary drainage channel around the work area to arrest the entry of runoff from upper areas into the work area (x) Pump out the water collected in the pits / excavations to a temporary sedimentation pond; dispose of only clarified water into drainage channels/streams after sedimentation in the temporary ponds (xi) Consider safety aspects related to pit collapse due to accumulation of water | - Areas for stockpiles, storage of fuels and lubricants and waste materials - Number of silt traps installed along drainages (in slope) leading to water bodies - Entry routes of pollutant in nearby Water bodies | Site inspection and Public grievance register | All project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC: PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied Earth work conducted during dry season. No as such requirement of disposal of spoil. Excess earth utilized for backfilling. Material never disposed in the pond located within this site. Silt traps / sediment basin should be installed prior to discharge of construction wastewater from site. Fuel storage not started. Surface water quality monitoring done All safety aspect maintained | Complied Earth work conducted during dry season. No as such requirement of disposal of spoil. Excess earth utilized for backfilling. Material never disposed in the pond located within this site. Silt traps / sediment basin should be installed prior to discharge of construction wastewater from site. Fuel storage not started. Surface water quality monitoring done All safety aspect maintained | Complied Earth work conducted during dry season. No as such requirement of disposal of spoil. Excess earth utilized for backfilling. Material never disposed in the pond located within this site. Silt traps / sediment basin should be installed prior to discharge of construction wastewater from site. Fuel storage not started. Surface water quality monitoring done All safety aspect maintained |
| Increase in noise level due to earth-moving and excavation equipment, and the transportation of equipment, materials, and people | (i) Plan activities in consultation with PIU so that activities with the greatest potential to generate noise are conducted during periods of the day which will result in least disturbance; (ii) Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; (iii) Minimize noise from construction equipment by using vehicle silencers, fitting jackhammers with noise-reducing mufflers, and use portable street barriers to minimize sound impact to surrounding sensitive receptor; and (iv) Identify any buildings at risk from vibration damage and avoiding any use of pneumatic drills or heavy vehicles in the vicinity (v) Horns should not be used unless it is necessary to warn other road users or animals of the vehicle’s approach; (vi) Consult local communities in advance of the work to identify and address key issues, and avoid working at sensitive times, such as religious and cultural festivals. | - Complaints from sensitive receptors - Use of silencers in noise-producing equipment and sound barriers Monitoring data | - Checking of records - Visual inspection of sites | All project locations Noise- monitoring as per selected sites in ref. to SEMP | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff Noise monitoring schedule as per SEMP. Base line monitoring conducted during earlier report period in July 2019 and present report period in November 2019 and in March 2020 (for pack 02B) | Environment Specialist of DSISC; PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied. No such noise generating problem near the project location. Pre-construction monitoring done. Monitoring will be continued as per IEE. Complete result certificates available in DSISC office as back up paper and attached as Appendix 10. Honking generally avoided at work sites. | Complied. No such noise generating problem near the project location. Pre-construction and during construction monitoring done. Monitoring will be continued as per IEE. Complete result certificates available in DSISC office as back up paper and attached as Appendix 10. Honking generally avoided at work sites. | Complied. No such noise generating problem near the project location. Pre-construction and during construction monitoring done. Monitoring will be continued as per IEE. Complete result certificates available in DSISC office as back up paper and attached as Appendix 10. Honking generally avoided at work sites. |
| Impacts due to excess excavated earth, excess construction materials, and solid waste such as removed concrete, wood, packaging materials, empty containers, spoils, oils, lubricants, and other similar items. | (i) Prepare and implement a Construction Waste Management Plan (ii) As far as possible utilize the debris and excess soil in construction purpose, for example for raising the ground level or construction of access roads etc., (iii) Stockpiles, lubricants, fuels, and other materials should be located away from steep slopes and water bodies; (iv)Avoid stockpiling any excess spoils at the site for long time. Excess excavated soils should be disposed of to approved designated areas immediately; (v) If disposal is required, the site shall be selected preferably from barren, infertile lands; site should be located away from residential areas, forests, water bodies and any other sensitive land uses (vi) Domestic solid wastes should be properly segregated in biodegradable and non-biodegradable for collection and disposal to designated solid waste disposal site; create a compost pit at workers’ camp sites for disposal of biodegradable waste; non-biodegradable / recyclable material shall be collected separately and sold in the local recycling material market (vii) Residual and hazardous wastes such as oils, fuels, and lubricants shall be disposed of in disposal sites approved by local authorities/West Bengal Pollution Control Board (WBPCB); (viii) Prohibit burning of construction and/or domestic waste; (ix) Ensure that wastes are not haphazardly dumped thrown within and around the project site and adjacent areas; provide proper collection bins, and create awareness to use the dust bins. (x) Conduct site clearance and restoration to original condition after the completion of construction work; PIU to ensure that site is properly restored prior to issuing of construction completion certificate | - Waste Management List - Stockpile Management - Complaints from Sensitive receptors - PMU/PIU/ DSISC to report in writing that the necessary environmental restoration work has been done | - Checking of records - Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC: PIU and PMC Due to nation-wide lockdown owing to COVID-19 pandemic, site visits could not be conducted since 3rd week of March 2020 | Complied Excess earth used mostly for backfilling. Excess spoils are not generally stockpiled. No hazardous waste and construction waste generated. | Complied Excess earth used mostly for backfilling. Excess spoils are not generally stockpiled. No hazardous waste and construction waste generated. | Complied Excess earth used mostly for backfilling. Excess spoils are not generally stockpiled. No hazardous waste and construction waste generated. |
| Disruption of service and damage to existing infrastructure at specified project location | (i) Prepare a list of affected utilities and operators if any; (ii) Prepare a contingency plan to include actions to be done in case of unintentional interruption of service | - List of affected utilities if any and operator’s Public grievance | Observation And document checking | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PIU | Complied as Per requirement. Consultation done with utility dept. as and when required | Complied as Per requirement. Consultation done with utility dept. as and when required | Complied as Per requirement. Consultation done with utility dept. as and when required |
| Loss of vegetation and tree cover | (i) Minimize removal of vegetation and disallow cutting of trees, by adopting best site layout and pipeline alignments (ii) If tree-removal will be required, obtain tree-cutting permit and (iii) Plant 5 native trees for every one that is removed. | Tree felling requirement and afforestation after final design | - Checking of records - Visual inspection of sites | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | As of now, no tree felling is anticipated at work sites | As of now, no tree felling is anticipated at work sites | As of now, no tree felling is anticipated at work sites |
| Traffic problems and conflicts near project locations and haul road | Hauling (material, waste/debris and equipment) activities (i) Plan transportation routes so that heavy vehicles do not use narrow local roads, except in the immediate vicinity of delivery sites (ii) Schedule transport and hauling activities during non-peak hours; (iii)Locate entry and exit points in areas where there is low potential for traffic congestion; (iv) Drive vehicles in a considerate manner (v) Notify affected public by public information notices, providing sign boards informing nature and duration of construction works and contact numbers for concerns/complaints. Pipeline works (i) Confine work areas along the roads to the minimum possible extent; all the activities, including material and waste/surplus soil stocking should be confined to this area. Provide barricading; avoid material/surplus soil stocking in congested areas – immediately removed from site/ or brought to the as and when required (ii) Leave spaces for access between mounds of soil to maintain access to the houses / properties (iii)Provide pedestrian access in all the locations; provide wooden/metal planks over the open trenches at each house to maintain the access. (iv) Inform the affected local population 1-week in advance about the work schedule (v) Plan and execute the work in such a way that the period of disturbance/ loss of access is minimum. (vi) Keep the site free from all unnecessary obstructions; (vii) Coordinate with Police for temporary road diversions, where necessary, and for provision of traffic aids if transportation activities cannot be avoided during peak hours | - Traffic Management Plan - Public grievance - Number of signages placed at subproject location | Site visit and document review | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC; PIU and PMC | Complied; Barricading done in pipe laying areas. Local people informed before start of work No transportation of pipe/ material done during pick hours, no impact on local public movement. Access maintained in most of the cases at pipe laying areas. Work plan informed to local public atleast 1 week in advance. | Complied; Barricading done in pipe laying areas. Local people informed before start of work No transportation of pipe/ material done during pick hours, no impact on local public movement. Access maintained in most of the cases at pipe laying areas. Work plan informed to local public atleast 1 week in advance. | Complied; Barricading done in pipe laying areas. Local people informed before start of work No transportation of pipe/ material done during pick hours, no impact on local public movement. Access maintained in most of the cases at pipe laying areas. Work plan informed to local public atleast 1 week in advance. |
| Generation of temporary employment and increase in local revenue | (i) Employ local labor force as far as possible (ii) Comply with labor laws | Employment record | Checking of records | Project locations | - | Environment Specialist of DSISC and PMC | Maximum number of local people are engaged in work. | Maximum number of local people are engaged in work. | Maximum number of local people are engaged in work. |
| Occupational hazards which can arise during work | (i) Comply with all national, state and local core labor laws (ii) Develop and implement site-specific occupational health and safety (OHS) Plan which will include measures such as: (a) excluding public from the site; (b) ensuring all workers are provided with and use personal protective equipment like helmet, gumboot, safety belt, gloves, nose musk and ear plugs; (c) OHS Training for all site personnel; (d) documented procedures to be followed for all site activities; and (e) documentation of work-related accidents; (iii) Ensure that qualified first-aid can be provided at all times. Equipped first-aid stations shall be easily accessible throughout the site; (iv) Provide medical insurance coverage for workers; (v) Secure all installations from unauthorized intrusion and accident risks; (vi) Provide supplies of potable drinking water; (vii) Provide clean eating areas where workers are not exposed to hazardous or noxious substances; (viii) Provide health and safety orientation training to all new workers to ensure that they are apprised of the basic site rules of work at the site, personal protective protection, and preventing injuring to fellow workers; (ix) Provide visitor orientation if visitors to the site can gain access to areas where hazardous conditions or substances may be present. Ensure also that visitor/s do not enter hazard areas unescorted; (x) Ensure the visibility of workers through their use of high visibility vests when working in or walking through heavy equipment operating areas; (xi) Ensure moving equipment is outfitted with audible back-up alarms; (xii) Mark and provide sign boards for hazardous areas such as energized electrical devices and lines, service rooms housing high voltage equipment, and areas for storage and disposal. Signage shall be in accordance with international standards and be well known to, and easily understood by workers, visitors, and the general public as appropriate | - Site-specific Health and Safety (H&S) Plan - Equipped first-aid stations; - Medical insurance coverage for workers - Number of accidents - Supplies of potable drinking water; - Record of H&S orientation trainings - Personal protective equipment - Sign boards for hazardous areas such as energized electrical devices and lines, service rooms | - Checking of records - Visual inspection of site | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Site-specific Health and Safety (H&S) Plan under implementation Copy of the approved Health and Safety plan available with DSISC., PIU H & S training done on regular basis. Sample training document is attached as Appendix 11. Drinking water and first aid box available at site. Proper use of PPEs- noted. Tie up letter with nearby health center in case of emergency obtained (Appendix13). Health checkup done for worker (Appendix 13) Medical Insurance arranged for the labourer (see Appendix 4). Accident/ First aid register is maintained at each site No case recorded during report period | Site-specific Health and Safety (H&S) Plan under implementation. Copy of the approved Health and Safety plan available with DSISC., PIU H & S training done on regular basis. Sample training document is attached as Appendix 11. Drinking water and first aid box available at site. Proper use of PPEs- noted. Tie up letter with nearby health center in case of emergency obtained (Appendix13). Health checkup done for worker (Appendix 13) Medical Insurance arranged for the labourer (see Appendix 4). Accident/ First aid register is maintained at each site (see sample copy in Appendix 12) | Site-specific Health and Safety (H&S) Plan under implementation. Copy of the approved Health and Safety plan available with DSISC., PIU H & S training done on regular basis. Sample training document is attached as Appendix 11. Drinking water and first aid box available at site. Proper use of PPEs- noted. Tie up letter with nearby health center in case of emergency obtained (Appendix13) Health checkup done for worker (Appendix 13) Medical Insurance arranged for the labourer (see Appendix 4). Accident/ First aid register is maintained at each site. No case reported during report period. |
| Health risks associated with AC pipes | (i) leave AC pipes in-situ untouched | Decommissioned AC Pipes | Site inspection | Project locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Till date no AC pipes has been found | Till date no AC pipes has been found | Till date no AC pipes has been found |
| Impact on community safety. Traffic accidents and vehicle collision with pedestrians during material and waste transportation | (i) Restrict construction vehicle movements to defined access roads and demarcated working areas (unless in the event of an emergency) (ii) Enforce strict speed limit (20-30 kph) for playing on unpaved roads, construction tracks (iii) Night-time haulage will be by exception only, as approved by the PIU to minimize driving risk and disturbance to communities (iv) Adopt standard and safe practices for micro tunneling (vi) Temporary traffic control (e.g. flagmen) and signs will be provided where necessary to improve safety and provide directions (vii) All drivers will undergo safety and training (viii) Public access to all areas where construction works are on-going will be restricted through the use of barricading and security personnel (ix) Warning signs, blinkers will be attached to the barricading to caution the public about the hazards associated with the works, and presence of deep excavation (x) The period of time when the pipeline trench is left open will be minimized through careful planning (xi) Control dust pollution – implement dust control measures as suggested under air quality section (xii) Maintain regularly the vehicles and use of manufacturer-approved parts to minimize potentially serious accidents caused by equipment malfunction or premature failure. (xiii) Provide road signs and flag persons to warn of on-going trenching activities. | Public grievance | Review of documents | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC; PIU and PMC | No pedestrian accident has been recorded till date. Pipe line laying work continued. All safety measures arranged. Improvement of access at few locations required. No trench will be kept open after pipe laying. Caution tape / barricade placed. | No pedestrian accident has been recorded till date. Pipe line laying work continued. All safety measures arranged. Improvement of access at few locations required. No trench will be kept open after pipe laying. Caution tape/ barricade placed | No pedestrian accident has been recorded till date. Pipe line laying work continued. All safety measures arranged. Improvement of access at few locations required. No trench will be kept open after pipe laying. Caution tape / barricade placed. |
| Impact on work camps and work site. Temporary air and noise pollution from machine operation, water pollution from storage and use of fuels, oils, solvents, and lubricants Unsanitary and poor living conditions for workers | (i) As far as possible located the camp site within the work sites; if any camp to be established outside these, then select a camp site away from residential areas (at least 100 m buffer shall be maintained) (ii) Avoid tree cutting for setting up camp facilities (iii) Camp site shall not be located near (100 m) water bodies, flood plains flood prone/low lying areas, or any ecologically, socially, archeologically sensitive areas (iv) Separate the workers living areas and material storage areas clearly with a fencing and separate entry and exit (v) Provide proper temporary accommodation with proper materials, adequate lighting and ventilation, appropriate facilities for winters and summers; ensure conditions of livability at work camps are maintained at the highest standards possible at all times; (vi) Consult PIU before locating project offices, sheds, and construction plants; (vii) Minimize removal of vegetation and disallow cutting of trees (viii) Ensure conditions of livability at work camps are maintained at the highest standards possible at all times; living quarters and construction camps shall be provided with standard materials (as far as possible to use portable ready to fit-in reusable cabins with proper ventilation); thatched huts, and facilities constructed with materials like GI sheets, tarpaulins, etc., shall not be allowed as accommodation for workers (ix) Camp shall be provided with proper drainage, there shall not be any water accumulation (x) Provide drinking water, water for other uses, and sanitation facilities for employees (xi) Prohibit employees from cutting of trees for firewood; contractor should be providing proper facilities including cooking fuel (oil or gas; fire wood not allowed) (xii) Train employees in the storage and handling of materials which can potentially cause soil contamination (xiii) Recover used oil and lubricants and reuse or remove from the site (xiv) Manage solid waste according to the following preference hierarchy: reuse, recycling and disposal to designated areas; provide a compost pit for biodegradable waste, and non-biodegradable / recyclable waste shall be collected and sold in local market (xv) Remove all wreckage, rubbish, or temporary structures which are no longer required (xvi) At the completion of work, camp area shall be cleaned and restored to pre-project conditions, and submit report to PIU; PIU to review and approve camp clearance and closure of work site | - Public grievance - Accommodation Water and sanitation facilities for employees - Housekeeping – regular disposal of solid waste | Site inspection and review of documents | Construction camps | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Complied mostly. Maintenance of housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking, Provision of solid waste and wastewater disposal are noted at site. | Complied mostly. Maintenance of housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking. Provision of solid waste and wastewater disposal are noted at site. | Complied mostly. Maintenance of housekeeping, toilet facility, testing of drinking water, provision of fuel (kerosene or LPG) for cooking. Provision of solid waste and wastewater disposal are noted at site. |
| Unsatisfactory compliance to EMP | (i) Appointment of (I) Environment, Health and Safety (EHS) Supervisor to ensure EMP implementation (ii) Timely submission of monitoring reports including pictures | - Appointment letter Monitoring records | Review of records | - | - | Environment Specialist of DSISC and PMC | Safety person appointed from contractor’s end; environmental safeguard training done. Monitoring report submitted on monthly basis. | Safety person appointed from contractor’s end; environmental safeguard training done. Monitoring report submitted on monthly basis. | Safety person appointed from contractor end; environmental safeguard training done. Monitoring report submitted on monthly basis. |
| Damage due to debris, spoils, excess construction materials | (i) Remove all spoils wreckage, rubbish, or temporary structures (such as buildings, shelters, and latrines) which are no longer required; and (ii) All excavated roads shall be reinstated to original condition. (iii) All disrupted utilities restored (iv) All affected structures rehabilitated/compensated (v) The area that previously housed the construction camp is to be checked for spills of substances such as oil, paint, etc. and these shall be cleaned up. (vi) All hardened surfaces within the construction camp area shall be ripped, all imported materials removed, and the area shall be top soiled and re-grassed using the guidelines set out in the re-vegetation specification that forms part of this document. (vii) The contractor must arrange the cancellation of all temporary services. (viii) Request PIU to report in writing that worksites and camps have been vacated and restored to pre-project conditions before acceptance of work. | - Stockpile Management - Spoil Management - Restoration of sites | Review of documents and site inspections | Project Locations | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager, Visit by Environment Specialist and Support Environment staff | Environment Specialist of DSISC and PMC | Complied; Spoil Management Plan has been submitted for the pipe laying work. No utilities affected during pipe laying at village roads. All excavated road reinstated to original condition | Complied; Spoil Management Plan has been submitted for the pipe laying work. No utilities affected during pipe laying at village roads. All excavated road reinstated to original condition | Complied; Spoil Management Plan has been submitted for the pipe laying work. No utilities affected during pipe laying at village roads. All excavated road reinstated to original condition |

Table 13: Summary of Environmental Monitoring Activities for the East Medinipur Package WBDWSIP/DWW/ICB/EM/02/2018-19

| Impacts | Mitigation Measures | Parameters Monitored (As a minimum those identified in the IEE should be monitored) | Method of Monitoring | Location of Monitoring | Date of Monitoring Conducted | Name of Person Who Conducted the Monitoring | Compliance Status/ Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Design Phase |  |  |  |  |  |  |  |
| Source sustainability and efficiency | (i) Gravity distribution system: designing the entire system to maintain optimal flow and terminal pressure, and optimizing the overall energy usage (ii)Implementation of a water quality surveillance program including development of a laboratory as part of the project to ensure that supplied water meets the drinking water standards (iii)Minimizing water losses from pipelines by perfect jointing and alignments using appropriate techniques (iv) Reducing the incidence of water borne diseases by providing 100% population including urban poor with potable water supplies | - Design philosophy - Project QA/QC plan - Selection methodology for distribution network | Document review and LOP Survey | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Final design not yet completed |
| Socio economic impact – loss fishery area | (i) Avoid using low-lying lands / ponds for construction of OHRs; alternative private lands may be explored within the vicinity; (ii)Review the applicability of West Bengal Inland Fisheries Act, 1984, whether the site falls under the definition of fisher area; obtained permission from Fisheries Department if required prior to start of construction | List of selected location for OHRs | Site survey | All OHR sites | Before commencement of final design | Environment Specialist of DSISC and PMC | No ponds have been filled for construction work. |
| Tree cutting | (i) Minimize removal of trees by adopting to site condition and with appropriate layout design of OHRs within the sites (ii)Avoid cutting of trees by adopting suitable alignment changes as required during laying of pipelines; (iii)In unavoidable cases, obtain prior permission for tree cutting (iv)Plant and maintain 5 trees for each tree that is removed | - Tree felling requirement - Site layout plan - NOC – paper documents from line agency | Site survey and review of site layout/ pipeline alignment plan | All project locations | Before commencement of final design | Environment Specialist of DSISC; PIU and PMC | Under compliance, no tree felling has been envisaged yet. In case of tree felling, NOC will be taken from concern authority. Replantation will be adopted after construction as per Gov. norms. |
| Pre-Construction Phase |  |  |  |  |  |  |  |
| Telephone lines, electric poles and wires, water lines within proposed project area | (i) Identify and include locations and operators of these utilities in the detailed design documents to prevent unnecessary disruption of services during construction phase; and (ii) Require construction contractors to prepare a contingency plan to include actions to be taken in case of unintentional interruption of services. (iii) Require contractors to prepare spoils (waste) management plan and traffic management plan | - List of affected utilities if any and operators - Bid document to include requirement for a contingency plan for service interruptions (example provision of water if disruption is more than 24 hours), waste management plan and traffic management plan | - Observation And - document checking | Specific project location | Before commencement of construction | Environment Specialist of DSISC; PIU and PMC | A contingency plan has been prepaid for avoiding unnecessary disruption of services during construction. Spoil and Traffic Management plan prepared. Sample spoil management and traffic management plan included in SEMP |
| Conflicts with local community; disruption to traffic flow and sensitive receptors | (i) Prioritize areas within or nearest possible vacant space in the project location; (ii) If it is deemed necessary to locate elsewhere, consider sites that will not promote instability and result in destruction of property, vegetation, irrigation, and drinking water supply systems; (iii) Do not consider residential areas; (iv) Take extreme care in selecting sites to avoid direct disposal to water body which will inconvenience the community. (v) For excess spoil disposal, ensure (a) site shall be selected preferably from barren, infertile lands. In case agricultural land needs to be selected, written consent from landowners (not lessees) will be obtained; (b) debris disposal site shall be at least 200 m away from surface water bodies; (c) no residential areas shall be located within 50 m downwind side of the site; and (d) site is minimum 250 m away from sensitive locations like settlements, ponds/lakes or other water bodies. | - List of selected sites for construction work camps, hot mix plants, stockpile areas, storage areas, and disposal areas - Written consent of landowner/s (not lessee/s) | - Site observation - Review of documents - Grievance Register | Specific project location | Before commencement of final design and commencement of construction | Environment Specialist of DSISC; PIU and PMC | Complied No disruption noted. Area selected nearby vacant place . No complete road closure expected. |
| Extraction of materials can disrupt natural land contours and vegetation resulting in accelerated erosion, disturbance in natural drainage patterns, ponding and water logging, and water pollution. | (i) Obtain construction materials only from government approved quarries with prior approval of PIU; (ii) PIU to review, and ensure that proposed quarry sources have all necessary clearances/ permissions in place prior to approval (iii) Contractor to submit to PIU on a monthly basis documentation on material obtained from each source (quarry/ borrow pit) (iv) Avoid creation of new borrow areas, quarries etc., for the project; if unavoidable, contractor to obtain all clearances and permissions as required under law, including Environmental Clearance prior to approval by PIU | - List of approved quarry sites and sources of materials | - Checking of records - Visual inspection of sites | Quarries and material source areas | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Complied; Approval obtained from DSISC, PIU before procurement. Presently new borrow area. not required |
| Failure to obtain necessary consents, permits, NOCs, etc. can result to design revisions and/or stoppage of works | (i) Obtain all necessary consents, permits, clearance, NOCs, etc. prior to award of civil works. (ii) Ensure that all necessary approvals for construction to be obtained by contractor are in place before start of construction (iii) Acknowledge in writing and provide report on compliance all obtained consents, permits, clearance, NOCs, etc. (iv) Include in detailed design drawings and documents all conditions and provisions if necessary | List of applicable legislation | Checking of documents | All project locations | Before commencement of construction |  | Mostly Complied Tree cutting NOC will be obtained as per requirement |
| Health risk due to exposure to asbestos materials | (i) Obtain details on location of asbestos cement materials (ii) Lay the new pipes carefully to avoid encountering AC pipes (ii) Leave the AC pipes undisturbed in the ground. | Detailed construction drawings showing alignment of AC pipes | Site inspection | Specific project location | Daily visit by construction supervisor of DSISC. Weekly visit by Construction Manager and Environment Team of DSISC of operational sites. | DSISC Construction Management and Environmental Safeguard Team | Till date no rubbish containing asbestos cement has been found AC pipe is not using in this project. |
| Construction Phase |  |  |  |  |  |  |  |
| Construction Work not yet started |  |  |  |  |  |  |  |

- Site Specific Issues and Present Compliance Status

North 24 pgs

  - Provision of office and toilet facility at Site.

- It is mandatory requirement to provide an office and toilet facility at site for workers. Although in zone 15 and zone 17 (OHR) of Haroa block and Zone 4 (OHR) of Bhangar II block and in WTP site have the site office facility available but not yet established in other working sites of Haroa and Bhangar II Block. Toilet facility has been provided at all the sites but condition at zone 10 & 13 (OHR) of Bhangar II block and Zone 15 & 12 (OHR) of Haroa block are found non-hygienic

  - Provision of basic facilities at construction camps.

- The project requires proper hygienic living facility for the workers. While at most sites adequate arrangements have been made but in some sites like Zone 15, 12 & 18 (OHR) of Haroa and Zone 12 & 13 (OHR) of Bhangar II block the condition is unsatisfactory.

| - Damp unhygienic living condition at Haroa Zone 18 (OHR) Site | - Satisfactory living condition at Zone 14 (OHR) Site of Haroa Block |
| --- | --- |

  - Use of PPEs e.g. safety harness, shoes, gloves and goggle while at work.

- Although the usage of PPEs at work has improved but still in number of sites violations are observed. Violations such as non-use of safety harness while working at height may lead to major accidents

| - Bhangar Zone 4 OHR site – Working at Height without Safety Harness | - Haroa Zone 1 OHR site – Working at Height with Safety Harness |
| --- | --- |

  - Barricading and caution tapes along pipelines.

- While barricading and installation of caution tapes is a mandatory requirement during laying of pipelines, in many instances such practices are not seen at sites. However, upon constant supervision and awareness campaigning by DSISC, good practices are being seen at some sites. The photographs below depict the positive change.

| - Pipelaying at Bhangar II Block (Zone 4) | - Pipelaying at Bhangar II Block (Zone 14) |
| --- | --- |

  - Management of wastewater and pit latrines at Sites

- The pit latrines installed at Zone 10 and 13 of Bhangar – II Block may cause faecal coliform contamination to groundwater. These pit latrines should at least be replaced with systems promoted under Swachh Bharat or be removed from site.

  - Camp located within ICDS in Zone 16 of Haroa Block.

- Labour camp located inside ICDS in Zone 16  pose risk to the children in terms of social distancing, high noise and accidents. This labour camp should be immediately shifted from this location.

Bankura

- Few non complied site- specific issues and compliance status as recorded during report period is given below.

| Site specific Issues; with location name and date | Compliance Status |  |
| --- | --- | --- |
|  | Before | After |
| Spoil dumping at Saharghati OHSR (BK/02B) site. Date of Observation: 06.01.20 Compliance Date: 14.01.20 |  |  |
| Storage/ Dumping of construction material in front of school at Chandabila OHSR (BK/02B) site. Date of Observation: 24.02.20 Compliance Date: 02.03.20 |  |  |
| Poor Housekeeping near intake point BK/03 Date of Observation: 16.03.20 Compliance Date: 20.03.20 |  |  |
| Incomplete information display board at WTP Site at BK/ 03 Date of Observation: 14.01.20 Compliance Date: 20.01.20 |  |  |
| Insufficient Fire safety measures at Mohona OHSR (BK/04) site Date of Observation: 23.12.19 Compliance Date: 27.12.19 |  |  |
| Missing Emergency contact detail at Gobindadham OHSR (BK/04) Site Date of Observation: 20.01.20 Compliance Date: 22.01.20 |  |  |
| Incomplete labour rest area at Bendalaxmanpur OHSR (BK/02B) site Date of Observation: 06.01.20 Compliance Date: 21.01.20 |  |  |

- Based on the foregoing observations, findings and environmental monitoring carried out from November 2019 to April 2020, it may be concluded that the subprojects have been implemented in partial compliance of the required environmental safeguards. Overall compliance level is shown in Table 14 below.

Table 14:  Overall Compliance with Environmental Management Plan

| No. | Sub-project Name | EMP Part of Contract Documents (Y/N) | EMP Being Implemented (Y/N) | Status of Implementation (Excellent/Satisfactory / Partially Satisfactory, Below Satisfactory) | Action Proposed and Additional Measures Required |
| --- | --- | --- | --- | --- | --- |
| North 24 Pgs |  |  |  |  |  |
| 1 | Package -WBDWSIP/DWW/NCB/N24P/01/2017-18: Bulk water supply | Y | Y | Satisfactory | - PPE usage and discharge of waste water in local drain is an issue in this sub project. - One labour hutment is recommended to be relocated. |
| 2 | Package -WBDWSIP/DWW/NCB/N24P/02A/2017-18: Haroa Block | Y | Y | Partially Satisfactory | - Improvement of facilities in Construction Camps, especially sleeping arrangements, toilet facilities, provision of drinking water and fuel for cooking, solid waste and wastewater management - At Zone 2, separate lane for movement of school children as well as deploying safety coordinator during opening and closing hours for the school. - Present labour accommodation at Zone 18 is very poor and should be improved immediately. - Missing PPE, particularly use of working shoes, ear plug, goggles (Haroa – Zone 1, Zone 16). Complete use of PPEs is required - Barricades and caution tapes in work areas, in particular along the pipelines were found as the most ubiquitous issue for sites like Haora-Zone 1. Arrangement of barricade and caution tape at all pipe laying / excavated sites - In Haroa Zone 16 labour camp was found within ICDS centre. It should be relocated as soon as possible. - In Haroa Zone 1 there is an issue of maintenance, as silencer leakage was found during site inspection. Beside it in this site vat sludge from storage tank and wastes from mixing machine was mixed with local pond. Soil barrier have to provide in this location. |
| 3 | Package -WBDWSIP/DWW/NCB/N24P/02B/2017-18: Bhangar - II Block | Y | Y | Partially Satisfactory | - Provision should be made for site office and toilets at all work sites - Improvement of facilities in Construction Camps, especially sleeping arrangements, toilet facilities, provision of drinking water and fuel for cooking, solid waste and wastewater management - Management of toilet waste at camp sites, use of pit latrines are serious concern (Bhangar II – Zone 10, 13). The pit latrines installed may cause faecal coliform contamination to groundwater and should be replaced with better, approved systems. - Use of PPEs e.g. safety shoes and goggles should be ensured at all sites especially for activities like concrete braking. - Missing PPE, particularly use of safety shoes, ear plug, goggles (Bhangar II – Zone 13). Complete use of PPEs is necessary at all work sites. - Barricades and caution tapes is missing at few work areas. One such case noted at Zone 14. Instructed for complete use of barricade and caution tape - First aid box missing in many sites like Bhangar II – Zone 12 & 13. At all work sites first aid box needs to be arranged. |
| Bankura |  |  |  |  |  |
| 4 | Package - WBDWSIP/DWW/NCB/BK/01/2017-18- Bulk water supply Indpur and Taldangra | Y | Y | Partial satisfactory | - Only the preliminary activities started in the package. - Establishment of site office, construction camp, set up of Laboratory etc. not yet done. |
| 5 | Package - WBDWSIP/DWW/NCB/ BK/02A/2018-19- Water supply distribution Indpur block | Y | Y | Satisfactory | - Only pipelaying activities started with proper safety arrangement - Establishment of site office, laboratory set up not yet done. - Arrangement of proper display and grievance redressal board |
| 6 | Package -WBDWSIP/DWW/NCB/BK/02B/2018-19- Water supply distribution Taldangra block | Y | Y | Satisfactory | - Construction camp, labour hutment with all facilities done at ongoing worksites. - Laboratory set up done at Taldangra. - Arrangement of proper display and grievance redressal board |
| 7 | Package -WBDWSIP/DWW/NCB/BK/03/2018-19- Bulk water supply Mejhia and Gangajalghati Blocks | Y | Y | Satisfactory | - Construction camp, labour hutment with all facilities established at WTP and Intake locations. - Laboratory set up done at Kalgoda. |
| 8 | Package - WBDWSIP/DWW/NCB/BK/04/2018-19 Water supply distribution Mejhia and Gangajalghati Blocks | Y | Y | Satisfactory | - Construction camp, labour hutment with all facilities done at ongoing worksites. - Laboratory set up done at Rangamati. - Arrangement of proper display and grievance redressal board |
| East Medinipur |  |  |  |  |  |
| 9 | Package -WBDWSIP/DWW/ICB/EM/01/2018-19 – Bulk water supply | Y | N | Preliminary activity just started | Nil |
| 10 | Package -WBDWSIP/DWW/ICB/EM/02/2018-19- Water supply distribution Nandigram I and II blocks | Y | N | Pre – construction activity continued | - Only the pre-construction activities ongoing - Labour camp has been proposed at Monoharpur |

- Grievance Redressal Mechanism

- A common Grievance redressal mechanism (GRM) is placed to address social, environmental, or any other project and/or subproject related grievances. The GRM has been developed in consultation with stakeholders. A public awareness campaign conducted to ensure that awareness of the project and its grievance redress procedures is generated. The campaign ensured that the poor, vulnerable, and others are made aware of grievance redress procedures and entitlements according to the project entitlement matrix, and PMU and concerned PIUs responsible for addressing their grievances.

- GRM consists of the following three tiers, one tier at project level and two beyond project level, as shown in Figure 8. Each tier identifies the persons responsible for addressing grievances and provides a time limit for this.

- For the project level GRM, a Grievance Redress Cell established at PIU; the safeguards officers (Assistant Engineer and Junior Engineer of PIU) of the ESSU PIU, supported by the social safeguards specialist of DSICS is responsible for conducting periodic community meetings with affected communities to understand their concerns and help them through the process of grievance redressal including translating the complaints into Bengali or English, recording and registering grievances of non-literate affected persons and explaining the process of grievance redress mechanism.

- All expedient and minor grievances is resolved at field level; should the PIU fail to resolve any grievance within the stipulated time period, the PMU will be consulted and suggested actions by PMU taken by PIU with DSISC support, within specified time. PIU is also be responsible for follow-through for each grievance, periodic information dissemination to complainants on the status of their grievance and recording their feedback (satisfaction/dissatisfaction and suggestions). In the event that certain grievances cannot be resolved at project level, they will be referred to the District Steering Committee (DSC), which will also act as grievance redress committee (GRC), particularly in matters related to land purchase/acquisition, payment of compensation, environmental pollution etc. Any higher than district level inter-departmental coordination or grievance redress required will be referred to the state level Steering Committee.

- The GRM aims to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the project. All grievances – major or minor, is registered. In case of grievances that are immediate and urgent in the perception of the complainant, the contractor, and supervision personnel from the PIU supported by DSISC will try to successfully resolve them in consultation with the Member, Panchayat and the Gram Panchayat Pradhan. In case of larger issues, they will seek the advice and assistance of the Superintending Engineer PIU. Grievances not redressed through this process within/at the project level within stipulated time period will be referred to the DSC/GRC.

- The DSC set up to monitor project implementation in each district. In its role as a GRC, the DSC will meet every month (if there are pending, registered grievances), determine the merit of each grievance, and resolve grievances within specified time upon receiving the complaint-failing which the grievance will be addressed by the state-level steering committee. The steering committee will resolve escalated/unresolved grievances received. Grievances remaining unresolved by steering committee may be referred by affected persons to appropriate courts of law. The GRC continue to function throughout the project duration.

- An aggrieved person shall have access to the country's legal system at any stage, and accessing the country's legal system can run parallel to accessing the GRM and is not dependent on the negative outcome of the GRM.

Figure 8: Grievance Redress Mechanism

Composition of the Grievance Redress Committee

- The DSC, acting as GRC have District Magistrate (Chairperson), Superintending Engineer, PIU as Member Secretary, Additional Executive Officer, Zilla Parishad, Assistant (Social and Environmental) Safeguard Officers of the Environment and Social Safeguard Units (ESSU) of the PIU, Institutional Support and Capacity Building Officer, PIU, Block Development Officers from respective blocks, and representatives from the affected village panchayat and / or community, if any, eminent citizens, CBOs and NGOs.

- The DSC/GRC must have a minimum of two women members. In case of any indigenous people impacts in future subprojects, the DSC/GRC must have representation of the affected indigenous people community, including at least one female indigenous person, the chief of the tribe or a member of the tribal council as traditional arbitrator (to ensure that traditional grievance redress systems are integrated) and an NGO working with indigenous people groups.

- The State level Steering Committee include Chief Secretary, as chair, Principal Secretary/Additional Chief Secretary, PHED, Principal Secretary, Panchayat and Rural Development, Principal Secretary, Finance, Principal Secretary, Irrigation and Waterways Development Department, Principal Secretary, Public Works Department, Engineering in Chief, PHED, Member Secretary, and Others as invitees.

Areas of Jurisdiction

- The areas of jurisdiction of the district level GRC, headed by the District Magistrate, will be (i) all locations or sites within the district where subproject facilities are proposed or being implemented, or (ii) their areas of influence within the district. The state-level steering committee will have jurisdictional authority across the state (i.e., areas of influence of subproject facilities beyond district boundaries, if any).

Consultation Arrangements

- Consultations include regular group meetings and discussions by the social safeguard personnel of DSISC and PIUs with affected persons, at least twice during resettlement plan preparation. During the first year of project implementation, such meetings take place on a quarterly basis, while in subsequent years they will be held at least twice a year. The consultation arrangements thus envisaged are intended to address both general and/or specific individual grievances through a participatory approach. The consultative process is meant to be flexible to provide timely mitigation of grievances of the affected persons.

Recordkeeping

- Records of all grievances received, including contact details of complainant, date the complaint was received, nature of grievance, agreed corrective actions and the date these were affected and final outcome will be kept by PIU (with the support of DSISC) and submitted to PMU

Information Dissemination Methods of the Grievance Redress Mechanism

- The PIU, assisted by DSISC responsible for information dissemination to affected persons on grievance redressal procedure. Gram Panchayat/coverage area/affected area-wide public awareness campaigns ensure that awareness on grievance redress procedures is generated through the consultation and participation plan. Public awareness campaign will be conducted to ensure that awareness on the project and its grievance redress procedures is generated. The PIU safeguard officers (environment and social) assisted by DSISC safeguards specialists with information/collateral/awareness material etc. and in conducting project awareness campaigns. The campaign ensures that the poor, vulnerable and others are made aware of grievance redress procedures and entitlements per agreed entitlement matrix including whom to contact and when, where/ how to register grievance, various stages of grievance redress process, time likely to be taken for redressal of minor and major grievances, etc. Grievances received and responses provided be documented and reported back to the affected persons. The number of grievances recorded and resolved and the outcomes will be displayed/disclosed in the PMU and PIU offices, Gram Panchayat/concerned local panchayat notice boards and on the web, as well as reported in the semi-annual environmental and social monitoring reports to be submitted to ADB.

- Periodic review and documentation of lessons learned. The PMU safeguards and gender cell periodically review the functioning of the GRM and record information on the effectiveness of the mechanism, especially on the PIU’s ability to prevent and address grievances.

- Costs. All costs involved in resolving the complaints (meetings, consultations, communication and reporting/information dissemination) be borne by the PMU. Cost estimates for grievance redress are included in resettlement cost estimates.

- Till report period State level and district level steering committee have been established. Also, PMU, PIU and Gram panchyat level GRC notification done for North 24 pgs, Bankura and East Medinipur. All Notifications related to Grievance Redressal Committee are given in Appendix 14.  Till report period DSC meeting was held at Bankura. Appendix 14 shows minutes of meeting. Grievance Registration Form in local language as proposed for the project is also presented in Appendix 15. Sample filled format is also shown in Appendix 15.

- Grievance Redressal Register is maintained at each of the package working sites.

- Complaints Received during the Reporting Period from North 24 pgs packages are summarized below

Table 15: Summary of Grievance Received during November 2019 to April 2020- North 24 pgs

| Package name | Total No of Complaints/ enquiry Recorded | Nature of the Complaint | Issue Resolved/ Not Resolved | Minimum Time Taken to Resolve the Issue | Maximum Time Taken to resolve the Issue |
| --- | --- | --- | --- | --- | --- |
| 02A: Haroa | 25 | Communication Problems, Access Issues, Service Issues, Suggestions | Yes | Around 80% issues are solved Within Same Day | Maximum 11 Days are taken |
| 02B: Bhangar II | 9 | Service Issues, Personal Issues, Suggestions | Yes | Within Same Day | Maximum 7 days are taken |

- Details of complaints received during report period and resolved status attached as Appendix 16.  Complaints are mostly related to,

- Generation of dust and requirement of dust suppression

- Filling up tranches at earliest

- Requirement of placement of caution board

- Requirement of removal of excess soil from the road

- Proper storage of construction materials

- Non availability of access due to improper storage of construction materials

- Non availability of caution tape, barricade at work site

- Non coverage of water supply scheme at few locations

- Issues are resolved mostly by site supervisor engineer and safety officer of contractor within same day of complaints received.

- Appendix 16 attached minutes of the meeting for EIdgah (religious place) related issues at Zone-17 OHR site under Bakjuri GP.

- Till report period no grievances are received for Bankura.

- Training, Workshop, Public consultation and Focus group discussion

- As per approved IEEs, consultations and disclosure will be a continuous process throughout project implementation involving public consultations and focus group discussions. Informal consultations were carried out with local people, pedestrian, etc.  Field level public consultation as conducted by contractor is attached as Appendix 17.

- The indicative schedule for consultations and disclosure is presented in Table 16.

Table 16: Indicative Schedule for Consultations

| Type of Consultation/ Disclosure | Target Date | Location | Target Participants | Responsible Person and Source of Funds |
| --- | --- | --- | --- | --- |
| Local level consultation | Weekly – to be continued | At all construction locations | General public, shop keepers, pedestrian population | Construction supervisor, Environment & safety officer of contractor Project budget – continuous process |
| Consultation – safety issues, implementation of EMP | During June 2020 to October 2020 and continued | At WBDWSIP office and project site office | Supervisor Engineer, PIU Engineer, all safety and environment staff of contractors | Construction Manager, Environment specialist of DSISC and PMC |

- Field level training program on safety and environment has been arranged for contractors, supervisors by DSISC’s Environment Specialist on regular basis.

- There are series of informal discussions by the DSISC engineering Consultants with PIU mainly on understanding current situation and optimum design to be adopted in order to attain the safeguard objectives.

- Summary of trainings, workshops, public consultation and FGD arranged in all the project districts are shown below.

Table 17A: Summary of Details of Consultations during November 2019 – April 2020- North 24 pgs

| Package Name and Site Location | Date of Training /Consultation/FGD | Type of Consultation | Total No of Participant | No. of Female Participant | Details of the meeting | Meeting Conductor |
| --- | --- | --- | --- | --- | --- | --- |
| Package No. WBDWSIP/ DWW/ NCB/ N-24P/ 02A; Haroa |  |  |  |  |  |  |
| Z-18 Gopalpur OHR | 06/01/2020 | Workshop | 9 | - | Workshop on Pile Excavation | Contractor |
| Z-04 Pipeline | 22/11/2019 | Training | 13 | - | First-Aid Training | DSISC |
| Z-06 Pipeline | 21/01/2020 | Training | 10 | - | HIV/AIDS Awareness Programme | DSISC |
| Z-14 Pipeline | 27/11/2019 | Training | 11 | - | HIV/AIDS Awareness Programme | DSISC |
| Z-18 Gopalpur | 17/01/2020 | Focused Group Discussion | 10 | 1 | Environmental Safeguard: EMP Implementation, Health and Safety Training | DSISC |
| Z-06 Radhanagar | 24/12/2019 | Public Meeting | 10 | - | Benefit and Impact of the Project, GRM information | DSISC |
| Z-02 Gopalpur-I | 13/01/2020 | Public Consultation | 20 | 13 | Grievance Redressal | DSISC |
| Z-3,19,21 Haroa GP | 13/01/2020 | Public Consultation | 29 | 26 | Project Beneficiation | DSISC |
| Z-10 Baganhati | 17/04/2020 | Public Meeting | 11 | 3 | Project Beneficiation | DSISC |
| Z-06 Puratan kamarghanti | 21/01/2020 | Public Consultation | 13 | 2 | Grievance Redressal and Project beneficiation | DSISC |
| 02A/Z-19 Khorda Chandpur | 18/03/2020 | Public Consultation | 10 | 4 | Project Beneficiation | DSISC |
| Package No. WBDWSIP/ DWW/ NCB/ N-24P/ 02B; Bhangar II |  |  |  |  |  |  |
| Z-07 Pipeline | 30/01/2020 | Induction Training/ Workshop | 9 | - | Trench Excavation/ JCB handling | Contractor |
| Z-04 OHR-Site Swastyangachhi | 12/11/2019 | Tool box Training | 8 | - | Fire Fighting and Safety Belt Training Programme | Contractor |
| Z-16 Pipeline | 21/11/2019 | Training | 10 | - | First Aid Training Programme | DSISC |
| Z-14 Pipeline | 27/11/2019 | Seminar | 12 | - | HIV & AIDS Awareness Programme | DSISC and Contractor |
| Z-16 Uttar Rajapur | 26/12/2019 | Public Meeting | 10 | 1 | Labour Safety, Grievance Redressal, | DSISC |
| Z-16 Bijoyganj School | 14/01/2020 | Public Consultation | 47 | 43 | Components of Project, Grievance Redressal, Project Beneficiation | NGO (Horaizen) DSISC |
| Z-13,15,16 Chaltaberia GP | 14/01/2020 | Public Consultation | 35 | 20 | Grievance Redressal, Project utility | NGO (Horaizen) DSISC |
| Package No. WBDWSIP/ DWW/ NCB/ N-24P/ 01; WTP, Rajarhat |  |  |  |  |  |  |
| WTP Rajarhat | 09/03/2020 | Tool box Training | 8 | 1 | Potential Hazard Identification and Control Measures Implementation | Contractor and DSISC |

Table 17B: Summary of Consultation/ FGD for Bankura Packages

| Package Name | Date | Place | Zone Name | Type of meeting/ consultation | Block | No of participants | % of female participants | Topics of discussion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BK/02A | 7-11-19 | Brahmandiha GP office | Gottoriya (Zone 05) | Public awareness and discussion on possible impact from the project and mitigation measures | Indpur | 40 | 62.5 | - Objective of the project – necessity of safe drinking water to every household - Project facilities - Public participation - Water conservation and sustainable use of water resources - Anticipated impact of the project during construction phase - Safety issues of local residents during construction work - Grievance redress system, registration process of grievance - Present facilities, complains regarding available infrastructure |
|  | 7-11-19 | Brajarajpur GP office | Gunnath (Zone 16) |  | Indpur | 26 | 3.8 |  |
|  | 24-01-20 | Kurusthalia | Jugibad (Zone 17) |  | Indpur | 43 | 41.9 |  |
|  | 24-01-20 | Kharbari | Moukuri (Zone 12) |  | Indpur | 17 | 0 |  |
| BK/02B | 13-01-20 | Pakurdiha Village | Hatapara (Zone 14) |  | Taldangra | 30 | 33.3 |  |
|  | 23-01-20 | Astasol | Amdangra (Zone 01) |  | Taldangra | 24 | 66.7 |  |
|  | 23-01-20 | Lagardanga | Dhengasimul (Zone 24) |  | Taldangra | 34 | 52.9 |  |
| BK/04 | 12-02-20 | Rajbandh and Radhakrisnapur | Ramkanali (Zone 17) |  | Gangajalghati | 36 | 72.2 |  |

- At East Medinipur project site during this report period, FGD has been conducted at following 4 places. It is recorded that 38% female participants attended those discussion. Summary shows in the Table below

Table 17C: Focus Group discussion at East Medinipur

| Sl no | Location | Date of Consultation | Zones covered under GP | Total no of Participants | No of Female Participants | Issue Discussed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Bhekutia | 08.02.2020 | Zone 10, 11,12 | 18 | 6 | - Discussed about several issues like waste management and plastic waste management - Brief about this project. Local people are so happy for this project as they believe it will help them to lead a healthy life. And they are willing to help us in every possible way. |
| 2. | Amratala Purba Sing Polly | 06-02-2020 | Overall | 19 | 9 | - The community wants to have water supply connections under this project. The common hand pump is not adequate especially during summer months. - The quality of water is not so good. Iron concentration is so high in ground water. |
| 3. | Samsabad | 24-01-2020 | Zone 5 | 24 | 10 | - Water quality not so good as iron content is very high and but according to local people case of water borne diseases are less in number. - Generation of dust due to construction activities. - Generation of noise due to heavy machinery activities. - Proper place for storage of waste |
| 4. | Birulia | 01-11-2019 | Zone 3 (N2) | 39 | 13 | - Water scarcity is not reported. However, salinity in water increases during the summer month which a common problem in the area. - Local people have agreed to cooperate and help with contractor as well as Department during construction period - They are ready to using water meter for controlling waste of water |

Table 17D: Orientation Training Programme (For EM01 and EM02)

| Sl no | Location | No of Participants | Date | Discussion |
| --- | --- | --- | --- | --- |
| 1. | Jambari | Total No-14 Female No.-00 | 21st Feb,2020 | - Program has been conducted by Environment Specialist of DSISC in presence of Safety Officer and labours of contractor. - Instruction to contractor to arrange pollution under control (PUC) certificate for material carrying vehicles, equipment and machinery – Timely Renewal of PUC - Setting up labour camp with proper sanitation condition, solid waste management facility and sufficient drinking water supply. - No employment of child Labour below 18 yrs. age - First aid boxes with prescribed items only at all work sites/camp site offices |
| 2. | PIU, East Medinipur Conference Hall | Total No.-23 Female No-00 | 28th January,2020 | - Orientation and Audit program has been arranged by PMC with DSISC and Contractor for both the packages. - Timely submission of SEMR (Semi-annual Environmental Monitoring Plan) - Brief discussion on SEMP and submission of SEMP as per zone wise before start of any - Construction work and importance of SEMP. - IEE will be updation as per zone wise development of SEMP |

- Appendix 11 indicates training components on environment, Health and safety issues, including awareness programme for HIV/ AIDS as conducted by North 24 pgs DSISC at project sites. Sample internal training documents as submitted by contractors are included in this appendix. Till report period official training is pending for Bankura DSISC, only field level instruction has been provided to contractors.

- During the report period Environment and Social safeguard related Audit has been conducted by PMC at North 24 pgs PMU/ PMC office, East Medinipur PIU office and Bankura PIU office. PIU staffs, contractor’s project manager, safety officers, DSISC staff attended that Audit Program. Review meeting cum orientation program on environment safeguard for DSISC, PIU and contractors are also conducted on regular basis. Appendix 18 shows note on Environment Audit and minutes of Safeguard Orientation Program. Focus Group discussion (FGD) on awareness of the project, social, environmental and safety issues have been highlighted. Appendix 19 shows list of FGD conducted and sample note on FGD for all the project districts.

# V. 	APPROACH AND METHODOLOGY FOR ENVIRONMENTAL MONITORING OF THE PROJECT

- For effective monitoring, selected environmental parameters have been identified as indicators which will be qualitatively and quantitatively measured and compared over a period of time in order to assess/ensure the compliance of Environment Management Plan (EMP). The environmental performance indicators are physical, biological and social characteristics identified as most important in affecting the environment at critical locations all along the subproject locations. The parameters identified as performance indicators are:

- Air, noise and water quality

- Compliance to EMP

- Compliance to local/state/national environmental regulations

- Field level monitoring comprises monitoring of environmental parameters like air quality, noise level and water quality. These are monitored for understanding base line conditions at project locations and during construction monitoring for understanding level of impact on environment (in respect to those parameters) from project activity during implementation of the project.  Monitoring is conducted as per monitoring plan of approved IEE.

- Field level monitoring also carried out during pre-construction, construction and operation phases for understanding degree of impacts and mitigation measures. Corrective action plan and target date for effective implementation of mitigation measures planned accordingly. Site Environment Management Plan is the base document for implementation of EMP and application of corrective measures. This field level monitoring is continuous process and reported through weekly, monthly checklist.

- Monitoring of applicability of local/state/national environmental regulations in respect to project activity and locations is also required for smooth progress of the project. For that site verification and desk review is essential. Starting from pre-construction to construction – operation phases screening of work areas, work components under the national, state and local statutory rules and regulations is necessary.

# VI.	 MONITORING OF ENVIRONMENTAL IMPACTS ON PROJECT SURROUNDINGS

A.	Brief discussion on the basis for monitoring

- In addition to desk reviews and site inspections, monitoring of selected environmental parameters have been conducted during the reporting period. The frequencies of the environmental monitoring activities are commensurate to the type and significance of the impacts. Monitoring of ambient air quality, noise level, soil quality and water quality has been conducted to establish baseline of environmental qualities in the project area and during construction impact on environment.

- North 24 pgs District

- In accordance with the IEE & EMP, the contractors are required to undertake environmental monitoring as per below table.

Table 18A: Environmental Monitoring Requirement (Package N 24 pgs/ 01)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 5 locations (WTP, Booster pumping station, 2 GLSRs, 1 pipe line) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 5 locations (WTP, Booster pumping station, 2 GLSRs, 1 pipe line) | Day time and night time noise levels | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | 2 locations (Bidyadhari River and Kestopur Canal) | pH, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | Once before start of construction half yearly during construction (3-year construction period considered) |

Table 18B: Environmental Monitoring Requirement (Package N 24 pgs/02A – Haroa and 02B- Bhangar II)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 5 locations (Selection during implementation to represent the overall project area) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 5 locations (same as air quality monitoring) | Day time and night time noise levels (24 hours) | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | 2 locations (to be selected during implementation and as per site condition) | pH, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | Once before start of construction Half yearly during construction (3 - year construction period considered) |

- Bankura District

- Environmental Monitoring Plan of IEE is the basis for environmental monitoring before start of the work and during implementation of the project.

Table 19A: Environmental Monitoring Requirement (Package BK/01)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 2 locations (WTP and GLSR- IBPS sites) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 4 locations (Intake, WTP and GLSR-IBPS sites) | Day time and night time noise levels | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Reservoir water quality | One location (Reservoir intake point) | pH, TDS, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | (i) Once before start of construction (ii) Monthly (yearly 12 times) (3-year construction period considered) |

Table 19B: Environmental Monitoring Requirement (Package BK/02A and BK/02B)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 5 locations (Selection during implementation to represent the overall project area) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 10 locations (same as air quality monitoring) | Day time and night time noise levels (24 hours) | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | 5 locations (Selection during implementation to represent the overall project area) | pH, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Soil quality | 5 locations (including, construction camps, workers camps) | pH, Elect. Conductivity (at 250C), Moisture (at 1050C), Texture (silt, clay, sand), Calcium (as CaO), Magnesium (as Mg), Permeability, Nitrogen (as N), Sodium (as Na), Phosphate (as PO4), Potassium (as K), Organic Matter, oil and grease | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ground Water Quality | 5 locations (including workers camp site & Construction camp/ storage yards) | As per IS10,500: 2012 | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |

Table 19C: Environmental Monitoring Requirement (Package BK/03)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 2 location (WTP &Intake sites) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 4 locations (Intake, WTP & 2 sensitive sites like hospital /school) | Day time and night time noise levels | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Barrage water quality | One location (Reservoir intake point) | pH, TDS, Oil & grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | (i) Once before start of construction (ii)Monthly (yearly 12 times) (3 years construction period considered) |

Table 19D: Environmental Monitoring Requirement (Package BK/04)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 10 locations (Selection during implementation to represent the overall project area) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 20 locations (same as air quality monitoring) | Day time and night time noise levels (24 hours) | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | 10 locations (to be selected during implementation) | pH, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Soil quality | 10 locations (including, construction camps, workers camps) | pH, Elect. Conductivity (at 250C), Moisture (at 1050C), Texture (silt, clay, sand), Calcium (as CaO), Magnesium (as Mg), Permeability, Nitrogen (as N), Sodium (as Na), Phosphate (as PO4), Potassium (as K), Organic Matter, oil and grease | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ground Water Quality | 10 locations (including workers camp site & Construction camp/ storage yards) | As per IS10,500: 2012 | (i) Once before start of construction. (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |

- East Medinipur District

- Environmental Monitoring Plan of IEE is the basis for environmental monitoring before start of the work and during implementation of the project.

Table 20 A: Environmental Monitoring Requirement (Package EM/01)

| Monitoring field | Monitoring location | Monitoring parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 5 locations (Intake, WTP, Transmission mains route) | - PM10, PM2.5 NO2, SO2, CO | - Once before start of construction. - Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 5 locations (Intake, WTP, Transmission mains route) | - Day time and night time noise levels | - (i) Once before start of construction - (ii) Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | One location (River intake) | - pH, TDS, Oil & grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity and Salinity | - Once before start of construction - Monthly (yearly 12 times) (2-year construction period considered) - Salinity of river water should be checked at 2-3 points downstream of intake, may at an interval of 1 km, 2 km |

Table 20 B: Environmental Monitoring Requirement (Package EM/02)

| Monitoring Field | Monitoring Location | Monitoring Parameters | Frequency |
| --- | --- | --- | --- |
| Ambient air quality | 10 locations (Reservoirs, IBPS and pipe line) | PM10, PM2.5 NO2, SO2, CO | (i) Once before start of construction. (ii)Yearly3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ambient noise | 20 locations (Reservoirs, IBPS and pipe line) | Day time and night time noise levels (24 hours) | (i) Once before start of construction. (ii)Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Surface water quality | 10 locations (Reservoirs, IBPS and pipe line) | pH, Oil and grease, Cl, F, NO3, TC, FC, Hardness, Turbidity BOD, COD, DO, Total Alkalinity | (i) Once before start of construction. (ii)Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Soil quality | 10 locations (Reservoirs, IBPS and pipe line) | pH, Elect. Conductivity (at 250C), Moisture (at 1050C), Texture (silt, clay, sand), Calcium (as CaO), Magnesium (as Mg), Permeability, Nitrogen (as N), Sodium (as Na), Phosphate (as PO4), Potassium (as K), Organic Matter, oil and grease | (i) Once before start of construction. (ii)Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |
| Ground Water Quality | 10 locations (Reservoirs, IBPS and pipe line) | As per IS10,500: 2012 | (i) Once before start of construction. (ii)Yearly 3 times (for seasons: pre-monsoon, post-monsoon and winter) during construction (3-years period considered) |

B.	Type and location of Environmental Parameters Monitored

- As detailed in above tables, for all the districts, air and noise monitoring should be conducted for yearly 3 times except monsoon while surface water quality should be conducted at every half-yearly (North 24 pgs), yearly 3 times (Bankura and East Medinipur packages) and monthly (Barrage and reservoir of Bankura Packages and river water at intake site of East Medinipur). The monitoring parameters are also stated in Table above.

- In order to determine new monitoring locations, following matrices were developed for each package.

- Based on outcome of above exercise air quality, noise level and surface and ground water quality monitoring locations were selected for packages.

- Monitoring Locations of North 24 pgs Packages. Monitoring locations are shown with Co-ordinate in Table below and in Figures followed.

Table 21A: Ambient Air Monitoring Station Locations during November 2019 to April 2020- Pre-construction and construction phase

| Sl. No | Monitoring Location | Co-ordinates | Construction phase |
| --- | --- | --- | --- |
|  | Package N 24 Pgs/02A: Haroa Distribution |  |  |
| 1. | Near Kulti Uposasthya Kendra (Zone -5) | 22°31'28.60"N, 88°41'11.90"E | During construction |
| 2. | Near Nebutala Abada (Zone-2) | 22°35’44.30”N, 88°45’3.80”E | During construction |
| 3. | Near Haroa Market (Zone-21) | 22°36’15.30”N, 88°40’40.70”E | During construction |
| 4. | Shalipur (Zone-17) | 22°36'10.50"N, 88°38'46.60"E | During construction |
| 5. | Khardah Chandpur (Zone 19) | 22°37'00.00"N, 88°41'34.00"E | Pre- construction |
|  | Package N 24 Pgs/02B: Bhangar II Distribution |  |  |
| 1. | Jawapur (Zone 16) | 22°30'59.43"N, 88°35'10.63"E | Pre- construction |
| 2. | Pitapukuriya (Zone 14) | 22°33'17.80"N 88°33'5.69"E | Pre- construction |
| 3. | Swastayan Gacchi Near School (Zone 4) | 22°34'24.40"N, 88°33'20.20"E | During construction |
| 4. | Near Bonkochua (Zone 12) | 22°32'29.90"N, 88°36'14.50"E | During construction |
| 5. | Near Paschim Majarhat Primary School (Zone 10) | 22°32'17.00"N, 88°34'58.00"E | During construction |
|  | Package N 24 Pgs/01: Bulk Water Supply * |  |  |
| 1. | WTP | 22°35'13.00"N, 88°26'49.00"E | During construction |
| 2. | Koromyog Ashram | 22°35'04.00"N, 88°28'17.00"E | Pre-construction |

(* Note: Base line monitoring remaining for 2 GLSRs location of pack N 24pgs/01. Monitoring to be conducted after handover of lands to contractor)

Table 21B: Surface Water Sampling Locations - North 24 Pgs- During construction monitoring

| Sl. No | Monitoring Location |
| --- | --- |
| Package N 24 pgs/02A : Haroa Distribution |  |
| 1. | Pond water of Nebutala Abada zone 2 |
| 2. | Pond water in zone 13 |
| Package N 24 pgs/02B : Bhangar II Distribution |  |
| 1. | Pond water of Zone 12 (Bonkochua) |
| 2. | Pond water of Zone 4 (Swastayan Gacchi) |
| Package N 24 pgs/01 : WTP and Bulk Distribution |  |
| 1. | WTP, New Town, Rajarhat |

*Note: Water quality data is monitored once in 6 months. However, due to COVID-19 situation monitoring could not be conducted (For package 02A and 02B – Monitoring done during October 2019 and for package 01 – Monitoring done during November 2020)

Table 21C: Ambient Noise Monitoring Station Details during November 2019 to April 2020- Pre construction and construction phase

| Sl. No | Monitoring Location | Co-ordinates | Construction phase |
| --- | --- | --- | --- |
|  | Package N 24 Pgs/02A: Haroa Distribution |  |  |
| 1. | Near Kulti Uposasthya Kendra (Zone -5) | 22°31'28.60"N, 88°41'11.90"E | During construction |
| 2. | Near Nebutala Abad (Zone-2) | 22°35'44.30"N, 88°45'3.80"E | During construction |
| 3. | Near Haroa Market (Zone-21) | 22°36'15.30"N, 88°40'40.70"E | During construction |
| 4 | Near Majanpur High School, Shalipur (Zone-17) | 22°36'10.50"N, 88°38'46.60"E | During construction |
| 5 | Khardah Chandpur (Zone 19) | 22°37'0"N, 88°41'34"E | Pre- construction |
|  | Package N 24 Pgs/02B: Bhangar II Distribution |  |  |
| 1. | Jawapur (Zone 16) | 22°30'59.43"N, 88°35'10.63"E | Pre- construction |
| 2. | Pitapukuriya (Zone 14) | 22°33'17.80"N 88°33'5.69"E | Pre- construction |
| 3. | SwastayanGacchi Near School (Zone 4) | 22°34'24.40"N, 88°33'20.20"E | During construction |
| 4. | Near Bonkochua (Zone 12) | 22°32'29.90"N, 88°36'14.50"E | During construction |
| 5. | Near Paschim Majarhat Primary School (Zone 10) | 22°32'17.00"N, 88°34'58.00"E | During construction |
|  | Package N 24 Pgs/01: Bulk Water Supply |  |  |
| 1. | WTP | 22°35'13.00"N, 88°26'49.00"E | During construction |
| 2. | Koromyog Ashram | 22°35'04.00"N, 88°28'17.00"E | Pre-construction |

- The monitoring locations for the projects were marked on google earth and presented below.

Figure 9A: Ambient Air Quality, Noise Level and Surface Water Monitoring Stations in Haroa Block (Package N 24 Pgs/02A)

Figure 9B: Ambient Air Quality, Noise Level and Surface Water Monitoring Stations in Bhangar-II Block (Package N 24 Pgs/02B)

Figure 9C: Ambient Air Quality and Noise Level Monitoring Stations in Bulk water distribution (Package N 24 Pgs/01)

- Monitoring Locations of Bankura Packages. For Bankura base line monitoring has been conducted and for one package during construction monitoring has also done. Below tables show detail of monitoring stations, where monitoring conducted during report period.

Table 22 A: Ambient Air Monitoring Station Locations – Base line and During Construction Monitoring – Bankura

| Sl. No | Monitoring Location | Co-ordinates | Time period |
| --- | --- | --- | --- |
| Package BK/01 |  |  |  |
| 1. | Gobindapur- GLSR site* | 23.0720010 N, 86.9054820 E | Baseline data (June, 2019) |
| 2. | Loyadihi- WTP site* | 22.9967590 N, 86.7680160 E | Baseline data (June, 2019) |
| Package BK/02A |  |  |  |
| 1. | Raghunathpur- GLSR site* | 23.1534150 N, 86.8614200 E | Baseline data (July, 2019) |
| 2 | Gobindapur | 23.0711733, 86.9029033 | Baseline data (November, 19) |
| 3 | Choukighata | 23.164684, 86.841567 | Baseline data (November, 19) |
| 4 | Nayekhir | 23.163465, 86.936905 | Baseline data (November, 19) |
| 5 | Gunnath | 23.066843, 86.959747 | Baseline data (November, 19) |
| Package BK/02B |  |  |  |
| 1. | Saharghati - OHSR Site* | 23.020360 N, 87.1203190 E | Baseline data (July, 2019) |
| 2 | Chandabila (Satmouli) | 22.986502, 87.267074 | Baseline data (November, 19) |
| 3 | Rajpur (Gamarbani) | 22.984100, 87.296600 | Baseline data (November, 19) |
| 4 | Harmasra (Jemua) | 23.021542, 87.007521 | Baseline data (November, 19) |
| 5 | Shibdanga More | 23.045233, 87.090370 | Baseline data (November, 19) |
| Package BK/03 |  |  |  |
| 1 | Basudevpur/Radhamadhabpur- WTP site* | 23.4831300 N, 87.1584810 E | Baseline data (July, 2019) |
| 2 | Intake point, Natungram | 23.470261, 87.291954 | Baseline data (November, 19) |
| 3 | WTP area, Radhamadhabpur | 23.482980, 87. 190751 | During Construction (1st), (February, 20) |
| 4 | Intake point, Natungram | 23.281491, 87.173111 | During Construction (1st), (February, 20) |
| Package BK/04 |  |  |  |
| 1 | Desuriya - near Deuli OHSR Site, near primary school* | 23.4508610 N, 87.0990520 E | Baseline data (July, 2019) |
| 2 | Bankadaha | 23.423460, 87.160522 | Baseline data (November, 19) |
| 3 | Gangajalghati | 23.410513, 87.119293 | Baseline data (November, 19) |
| 4 | Deuli | 23.451023, 87.10522 | Baseline data (November, 19) |
| 5 | Machrakend | 23.451023, 87.099174 | Baseline data (November, 19) |
| 6 | Ranipur | 23.453812, 87.112511 | Baseline data (November, 19) |
| 7 | Barsal Srirampur | 23.489218, 87.186918 | Baseline data (November, 19) |
| 8 | Durlavpur | 23.473228, 87.131668 | Baseline data (November, 19) |
| 9 | Chhotonabagram | 23.457315, 87.021661 | Baseline data (November, 19) |
| 10 | Mohona | 23.570732, 87.031478 | Baseline data (November, 19) |

* Done during earlier report period as base line

Table 22B: Ambient Noise Sampling Locations - Base line and During Construction Monitoring – Bankura

| Sl. No | Monitoring Location | Co-ordinates | Time period |
| --- | --- | --- | --- |
| Package BK/01 |  |  |  |
| 1. | Gobindapur- GLSR site* | 23.0720010 N, 86.9054820 E | Baseline data (June, 2019) |
| 2. | Loyadihi- WTP site* | 22.9967590 N, 86.7680160 E | Baseline data (June, 2019) |
| 3 | Kumar Bohal - Intake point* | 22.9893200 N, 86.7692970 E | Baseline data (June, 2019) |
| Package BK/02A |  |  |  |
| 1. | Raghunathpur- GLSR site* | 23.1534150 N, 86.8614200 E | Baseline data (July, 2019) |
| 2 | Gobindapur | 23.0711733, 86.9029033 | Baseline data (November, 19) |
| 3 | Choukighata | 23.164684, 86.841567 | Baseline data (November, 19) |
| 4 | Nayekhir | 23.163465, 86.936905 | Baseline data (November, 19) |
| 5 | Gunnath | 23.066843, 86.959747 | Baseline data (November, 19) |
| Package BK/02B |  |  |  |
| 1. | Saharghati - OHSR Site* | 23.020360 N, 87.1203190 E | Baseline data (July, 2019) |
| 2 | Chandabila (Satmouli) | 22.986502, 87.267074 | Baseline data (December, 19) |
| 3 | Rajpur (Gamarbani) | 22.984100, 87.296600 | Baseline data (December, 19) |
| 4 | Harmasra (Jemua) | 23.021542, 87.007521 | Baseline data (December, 19) |
| 5 | Shibdanga More | 23.045233, 87.090370 | Baseline data (December, 19) |
| 6 | Amdangra | 22.94556, 87.28223 | Baseline data (March, 20) |
| 7 | Bindalaxmanpur | 23.05583, 87.09639 | Baseline data (March, 20) |
| 8 | Dhobajor | 22.99111, 87.148051 | Baseline data (March, 20) |
| 9 | Koniyara | 22.91222, 87.22722 | Baseline data (March, 20) |
| 10 | Manikmara | 23.028333, 87.149722 | Baseline data (March, 20) |
| Package BK/03 |  |  |  |
| 1 | Basudevpur/ Radhamadhabpur- WTP site* | 23.4831300 N, 87.1584810 E | Baseline data (July, 2019) |
| 2 | Intake point, Natungram | 23.470707, 87.292366 | Baseline data (November, 19) |
| 3 | School, near intake point | 23.473259, 87.157944 | Baseline data (November, 19) |
| 4 | Hospital, near WTP area | 23.470192, 87.186150 | Baseline data (November, 19) |
| 5 | WTP area, Basudevpur | 23.482981, 87.190731 | During Construction (1st), (February, 20) |
| 6 | Intake point, Natungram | 23.481521, 87.273012 | During Construction (1st), (February, 20) |
| 7 | Sri Chandanpur Primary School, Nritanandyapur, Kolgora | 23.282051, 87.110951 | During Construction (1st), (February, 20) |
| 8 | Nityanandapur, Kolgora, near PHC | 23.281511, 87.113001 | During Construction (1st), (February, 20) |
| Package BK/04 |  |  |  |
| 1 | Desuriya - near Deuli primary school* | 23.4508610 N, 87.0990520 E | Baseline data (July, 2019) |
| 2 | Gangajalghati | 23.411161, 87.119560 | Baseline data (November, 19) |
| 3 | Bankadaha | 23.428774, 87.157349 | Baseline data (November, 19) |
| 4 | Banasuriya | 23.469541, 87.082397 | Baseline data (November, 19) |
| 5 | Purbator | 23.531193, 87.185936 | Baseline data (November, 19) |
| 6 | Benabad | 23.510986, 87.083267 | Baseline data (November, 19) |
| 7 | Mejhia | 23.538759, 87.128411 | Baseline data (November, 19) |
| 8 | Lachhmanpur | 23.443277, 87.032471 | Baseline data (November, 19) |
| 9 | Mohoana | 23.563515, 87.032387 | Baseline data (November, 19) |
| 10 | Chhotonabagram | 23.448661, 87.016037 | Baseline data (November, 19) |
| 11 | Durlabhpur | 23.473913, 87.134328 | Baseline data (November, 19) |
| 12 | Barsal Srirampur | 23.866427, 87.209724 | Baseline data (November, 19) |
| 13 | Ranipur | 23.494473, 87.120308 | Baseline data (November, 19) |
| 14 | Mochrakend | 23.511042, 87.143799 | Baseline data (November, 19) |
| 15 | Deuli | 23.458918, 87.107208 | Baseline data (November, 19) |
| 16 | Raghunathpur | 23.343790, 87.073509 | Baseline data (November, 19) |
| 17 | Keduadihi (Kapista) | 23.398541, 87.129729 | Baseline data (November, 19) |
| 18 | Sarangapur | 23.437952, 87.180771 | Baseline data (November, 19) |
| 19 | Lakhiyara | 23.365717, 87.074257 | Baseline data (November, 19) |
| 20 | Subaira | 23.468479, 87.169662 | Baseline data (November, 19) |

* Done during earlier report period as base line

Table 22C: Ground water Sampling Locations – Base line Monitoring – Bankura

| Sl. No. | Monitoring Location | Co-ordinates | Time period |
| --- | --- | --- | --- |
| Package BK/02A |  |  |  |
| 1. | Raghunathpur* | 23.1534150 N, 86.8614200 E | Baseline data (June, 2019) |
| 2 | Gobindapur | 23.071308, 86.903008 | Baseline data (November, 19) |
| 3 | Choukighata | 23.231645, 86.841555 | Baseline data (November, 19) |
| 4 | Nayekhir | 23.162579, 86.914909 | Baseline data (November, 19) |
| 5 | Gunnath | 23.066843, 86.959747 | Baseline data (November, 19) |
| Package 02B |  |  |  |
| 1. | Saharghati* | 23.020360 N, 87.1203190 E | Baseline data (June, 2019) |
| 2 | Harmasra | 23.0202730N, 87.0056400E | Baseline data (November, 19) |
| 3 | Rajpur | 22.9682740N, 87.2847030E | Baseline data (November, 19) |
| 4 | Shibdanga More | 23.0448040 N, 87.0935810E | Baseline data (November, 19) |
| 5 | Chandabila | 22.9878130N, 87.2672370E | Baseline data (November, 19) |
| Package BK/04 |  |  |  |
| 1 | Desuriya - near Deuli OHSR Site* | 23.4508610 N, 87.0990520 E | Baseline data (July, 2019) |
| 2 | Chousalk (Basudevpur) | 23.472799, 87.153735 | Baseline data (December, 19) |
| 3 | Machaparulia | 23.343850, 87.096364 | Baseline data (December, 19) |
| 4 | Ranipur | 23.495304, 87.119343 | Baseline data (December, 19) |
| 5 | Gangajalghati | 23.409656, 87.116737 | Baseline data (December, 19) |
| 6 | Bankadaha | 23.427255, 87.164658 | Baseline data (December, 19) |
| 7 | Gobindadham | 23.372236, 87.112066 | Baseline data (December, 19) |
| 8 | Mochrakend | 23.512523, 87.145526 | Baseline data (December, 19) |
| 9 | Mohona | 23.566812, 87.028013 | Baseline data (December, 19) |
| 10 | Barsal Srirampur | 23.489218, 87.186918 | Baseline data (December, 19) |

* Done during earlier report period as base line

Table 22D: Surface water Sampling Locations – Base line and During Construction Monitoring – Bankura

| Sl. No. | Monitoring Location | Co-ordinates | Time period |
| --- | --- | --- | --- |
| Package BK/01 |  |  |  |
|  | Sampling done – Analyses under process (Work not yet started) |  |  |
| Package BK/02A |  |  |  |
| 1 | Chaukighata | 23.2316450N, 86.8415550E | Baseline data (November, 19) |
| 2 | Gobindapur | 23.0713080N, 86.9030080E | Baseline data (November, 19) |
| 3 | Gunnath | 23.0668430N, 86.9597470E | Baseline data (November, 19) |
| 4 | Nayekhir | 23.1625790 N, 86.9149090E | Baseline data (November, 19) |
| 5 | Raghunathpur | 23.1525800N, 86.8665710E | Baseline data (November, 19) |
| Package BK/02B |  |  |  |
| 1 | Chandabila | 22.9878130N, 87.2672370E | Baseline data (November, 19) |
| 2 | Patrabandh | 23.0194230N, 87.1216210E | Baseline data (November, 19) |
| 3 | Harmasra | 23.0202730N, 87.0056400E | Baseline data (November, 19) |
| 4 | Rajpur | 22.9682740N, 87.2847030E | Baseline data (November, 19) |
| 5 | Shibdanga More | 23.0448040 N, 87.0935810E | Baseline data (November, 19) |
| Package BK/03* |  |  |  |
| 1 | Intake point (Durgapur barrage) | 23.4707160N, 87.2924040E | Baseline data (November, 19) |
| 2 | Intake point (Durgapur barrage) | 23.481561, 87.271311 | During Construction (1st), (February, 20) |
| Package BK/04 |  |  |  |
| 1 | Mohana | 23.567759 N, 87.029091 E | Baseline data (December, 2019) |
| 2 | Machaparulia | 23.342296, 87.096078 | Baseline data (December, 19) |
| 3 | Chusalk(Basudevpur) | 23.473389, 87.151700 | Baseline data (December, 19) |
| 4 | Gangajalghati | 23.410938, 87.11528 | Baseline data (December, 19) |
| 5 | Gobindadham | 23.358759, 87.109888 | Baseline data (December, 19) |
| 6 | Ranipur | 23.494473, 87.120308 | Baseline data (December, 19) |
| 7 | Bankadaha | 23.426739, 87.162643 | Baseline data (December, 19) |
| 8 | Deuli | 23.451704, 87.099098 | Baseline data (December,19) |
| 9 | Mochrakend | 23.513701, 87.145111 | Baseline data (December, 19) |
| 10 | Barasal | 23.482309, 87.166748 | Baseline data (December, 19) |

(*Due to COVID 19 pandemic lockdown monitoring not done from March 2020. As per EMP monthly monitoring will be continued)

Table 22E: Soil Sampling Locations – Base line Monitoring – Bankura

| Sl. No. | Monitoring Location | Co-ordinates | Time period |
| --- | --- | --- | --- |
| Package BK/02A |  |  |  |
| 1 | Belasuly shyamsundarpur | 23.067222, 87.054165 | Baseline data (February, 2020) |
| 2 | Bholarkhap | 23.135277, 86.93667 | Baseline data (February, 20) |
| 3 | Goalakpur | 23.101941, 86.916389 | Baseline data (February, 20) |
| 4 | Moukuri | 23.15056, 86.98556 | Baseline data (February, 20) |
| 5 | Tunamara | 23.096111, 86.89389 | Baseline data (February, 20) |
| Package BK/02B |  |  |  |
| 1 | Amdangra | 22.94556, 87.28223 | Baseline data (March, 20) |
| 2 | Bindalaxmanpur | 23.05583, 87.09639 | Baseline data (March, 20) |
| 3 | Dhobajor | 22.99111, 87.148051 | Baseline data (March, 20) |
| 4 | Koniyara | 22.91222, 87.22722 | Baseline data (March, 20) |
| 5 | Manikmara | 23.028333, 87.149722 | Baseline data (March, 20) |
| Package BK/04 |  |  |  |
| 1 | Ranipur | 23.4944730N, 87.1203080 E | Baseline data (January, 20) |
| 2 | Barsal-srirampur | 23.4768660N, 87.2156370E | Baseline data (January, 20) |
| 3 | Mohona | 23.5634750N, 87.0323640E | Baseline data (January, 20) |
| 4 | Mochrakend | 23.5129780N, 87.1456610E | Baseline data (January, 20) |
| 5 | Gobindadham | 23.3588470N, 87.1098420E | Baseline data (January, 20) |
| 6 | Bankadaha | 23.4287970N, 87.1573110E | Baseline data (January, 20) |
| 7 | Chausalk-Basudevpur | 23.4740550N, 87.1496770 E | Baseline data (January, 20) |
| 8 | Gangajalghati | 23.4109520N, 87.1195310E | Baseline data (January, 20) |
| 9 | Deuli | 23.4494290N, 87.0966030E | Baseline data (January, 20) |
| 10 | Machaparulia | 23.3425440N, 87.0959810E | Baseline data (January, 20) |

- The monitoring locations for the projects were marked on google earth and presented below.

Figure 10A: Monitoring stations for Package BK/01- Bankura

Figure 10B: Monitoring stations for Package BK/02A- Bankura

Figure 10C: Monitoring stations for Package BK/02B- Bankura

Figure 10D: Monitoring stations for Package BK/03- Bankura

Figure 10 E: Monitoring stations for Package BK/04- Bankura

- Monitoring locations of East Medinipur:  Out of two packages of East Medinipur, i.e. EM/01 and EM/02, the baseline monitoring work of EM/02 has been completed and base line monitoring for package EM/01 will be started very soon.

Table 23A: Ambient Air Monitoring Station Locations- Pre – construction

| Sl. No | Monitoring Location | Co-ordinates | Monitoring period |
| --- | --- | --- | --- |
|  | Package EM/02 |  |  |
| 1 | Manoharpur | 22.0503170N,87.91310868E | 08.01.2020 |
| 2 | Nanigram CT | 22.02251411 N, 87.97307104 E | 09.01.2020 |
| 3 | Parbatipur | 22.01461594 N, 88.02221562 E | 09.01.2020 |
| 4 | Haripur | 22.0254522 N, 87.9516447 E | 10.01.2020 |
| 5 | Manuchak Jalpai | 22.0254522N, 87.9516447 E | 11.01.2020 |
| 6 | Gar chakraberia | 21.97035714 N, 88.00255399 E | 11.01.2020 |
| 7 | Kanakpur | 22.0856484N, 87.94174180E | 12.01.2020 |
| 8 | Kamalpur | 21.97626399N, 87.91178199E | 13.01.2020 |
| 9 | Jambari | 21.96175899N, 87.93780099E | 13.01.2020 |
| 10 | Birulia | 21.98457060N, 87.85906609E | 08.01.2020 |

Table 23B: Ground Water Sampling location- Pre construction

| Sl. No | Monitoring Location | Co-ordinates | Period |
| --- | --- | --- | --- |
|  | Package EM/02 |  |  |
| 1 | Manoharpur | 22.0503170N,87.91310868E | 26.12.2019 |
| 2 | Nanigram CT | 22.02251411 N, 87.97307104 E | 09.01.2020 |
| 3 | Parbatipur | 22.01461594 N, 88.02221562 E | 10.01.2020 |
| 4 | Haripur | 22.0254522 N, 87.9516447 E | 26.12.2019 |
| 5 | Manuchak Jalpai | 22.0254522N, 87.9516447 E | 11.01.2020 |
| 6 | Gar chakraberia | 21.97035714 N, 88.00255399 E | 12.01.2020 |
| 7 | Kanakpur | 22.0856484N, 87.94174180E | 13.01.2020 |
| 8 | Kamalpur | 21.97626399N, 87.91178199E | 12.01.2020 |
| 9 | Jambari | 21.96175899N, 87.93780099E | 11.01.2020 |
| 10 | Birulia | 21.98457060N, 87.85906609E | 26.12.2019 |

Table 23C: Surface Water Sampling location- Pre construction

| Sl. No | Monitoring Location | Co-ordinates | Period |
| --- | --- | --- | --- |
|  | Package EM/02 |  |  |
| 1 | Manoharpur | 22.0503170N,87.91310868E | 26.12.2019 |
| 2 | Nanigram CT | 22.02251411 N, 87.97307104 E | 09.01.2020 |
| 3 | Parbatipur | 22.01461594 N, 88.02221562 E | 10.01.2020 |
| 4 | Haripur | 22.0254522 N, 87.9516447 E | 26.12.2019 |
| 5 | Manuchak Jalpai | 22.0254522N, 87.9516447 E | 11.01.2020 |
| 6 | Gar chakraberia | 21.97035714 N, 88.00255399 E | 12.01.2020 |
| 7 | Kanakpur | 22.0856484N, 87.94174180E | 13.01.2020 |
| 8 | Kamalpur | 21.97626399N, 87.91178199E | 12.01.2020 |
| 9 | Jambari | 21.96175899N, 87.93780099E | 11.01.2020 |
| 10 | Birulia | 21.98457060N, 87.85906609E | 26.12.2019 |

Table 23D: Soil Sampling Locations – Pre- construction

| Sl. No | Monitoring Location | Co-ordinates | Period |
| --- | --- | --- | --- |
|  | Package EM/02 |  |  |
| 1 | Manoharpur | 22.0503170N,87.91310868E | 26.12.2019 |
| 2 | Nanigram CT | 22.02251411 N, 87.97307104 E | 09.01.2020 |
| 3 | Parbatipur | 22.01461594 N, 88.02221562 E | 10.01.2020 |
| 4 | Haripur | 22.0254522 N, 87.9516447 E | 26.12.2019 |
| 5 | Manuchak Jalpai | 22.0254522N, 87.9516447 E | 11.01.2020 |
| 6 | Gar chakraberia | 21.97035714 N, 88.00255399 E | 12.01.2020 |
| 7 | Kanakpur | 22.0856484N, 87.94174180E | 12.01.2020 |
| 8 | Kamalpur | 21.97626399N, 87.91178199E | 12.01.2020 |
| 9 | Jambari | 21.96175899N, 87.93780099E | 11.01.2020 |
| 10 | Birulia | 21.98457060N, 87.85906609E | 26.12.2019 |

Table 23E: Noise Sampling Locations- Pre- construction

| Sl. No | Monitoring Location | Co-ordinates | Period |
| --- | --- | --- | --- |
|  | Package EM/02 |  |  |
| 1 | Manoharpur | 22.0503170N,87.91310868E | 09.01.2020 |
| 2 | Nanigram CT | 22.02251411 N, 87.97307104 E | 09.01.2020 |
| 3 | Parbatipur | 22.01461594 N, 88.02221562 E | 09.01.2020 |
| 4 | Haripur | 22.0254522 N, 87.9516447 E | 11.01.2020 |
| 5 | Manuchak Jalpai | 22.0254522N, 87.9516447 E | 11.01.2020 |
| 6 | Gar chakraberia | 21.97035714 N, 88.00255399 E | 12.01.2020 |
| 7 | Kanakpur | 22.0856484N, 87.94174180E | 12.01.2020 |
| 8 | Kamalpur | 21.97626399N, 87.91178199E | 13.01.2020 |
| 9 | Jambari | 21.96175899N, 87.93780099E | 13.01.2020 |
| 10 | Birulia | 21.98457060N, 87.85906609E | 08.01.2020 |
| 11 | Gangra | 21.91828561N, 87.98308572E | 14.01.2020 |
| 12 | Amgechia | 21.97416273N, 87.96632052E | 14.01.2020 |
| 13 | Gokulnagar | 21.94799054N, 87.95879200E | 14.01.2020 |
| 14 | Naynan | 21.98991086N,87.93808325E | 15.01.2020 |
| 15 | Dinabandhupur | 22.04204448N, 88.00729970E | 15.01.2020 |
| 16 | Purushottampur | 22.00700039N, 87.9899406E | 15.01.2020 |
| 17 | Banasri Gauri | 22.00576897N, 87.91884838E | 16.01.2020 |
| 18 | Gholpukuria | 22.01558547N, 87.88140867E | 16.01.2020 |
| 19 | Narasinghapur Jalpai | 22.06273183N, 87.9563248E | 16.01.2020 |
| 20 | Gopalchak | 22.08573227N, 87.9794286E | 16.01.2020 |

- The monitoring locations for the package were marked on google earth and presented below.

Figure 11: Air, Noise, Water and Soil quality Monitoring sites for Package East Medinipur, EM 02

C	Method of monitoring and Equipment used

- Ambient Air Quality Monitoring Equipment and Methodology. Respirable Dust Samplers and Fine Dust Samplers (Model no.-APM-860/APM-154) have been used to collect samples for PM (10) and PM (2.5) in ambient air at a flow rate of 1.2 m3/min. The ambient air was sucked through the cyclone and filter paper by a blower. Samples of gases were drawn at a flow rate of 0.5 liters per minute and were analysed in the laboratory. Ambient air quality monitoring methodology is given in table below.

Table 24: Methodology for the ambient air monitoring

| Sl. No | Parameters | Measurement Method | As per |
| --- | --- | --- | --- |
| 1. | PM10 | Gravimetric | IS:5182 Part-23, RA-2017 |
| 2. | PM2.5 | Gravimetric | USEPA CFR-40, Part-50 |
| 3. | SO2 | Colorimetric (EPA modified West & Gaeke Method) | IS:5182 Part-2, 2001, RA-2017 |
| 4. | NOX | Colorimetric (Arsenite modified Jacobs & Hochheiser Method) | IS:5182 Part-6, 2006, RA-2017 |
| 5. | CO | Non-Dispersive Infra-Red (NDIR) Spectroscopy Technique | IS:5182 Part-10, 1999, RA-2014 |

- Methodology of Surface and Ground Water Monitoring. In order to establish baseline conditions, grab samples were collected from water source and were analysed for various parameters as per the procedures laid down in the BIS. Samples were collected as per IS- 2488 (Part I-V). Surface water samples were analysed as per surface water quality criteria by Central Pollution Control Board (CPCB) for class B. Ground water samples were also analysed as per method provided by CPCB. Wet chemical, Atomic Absorption Spectrophotometer and UV/VIS Spectrophotometer were used for analysis of water samples according to the necessity.

- Methodology of Noise Monitoring. Pulsar Data logger Noise Meter (Model 30) was used to monitor the noise levels. Noise level was monitored continuously for 24 hours with one-hour interval. Noise level was measured in the form of Leq dB(A), - Day and Night

D.	Monitoring results and analysis of results

- Monitoring results and analysis of results are presented below for sub projects at North 24 Pgs, Bankura and East Medinipur.

- Environmental Monitoring Results of North 24 Pgs and observations

Table 25: Ambient Air Quality Monitoring Results- North 24 pgs

| Site no. | Date of Testing | Site Location | Construction phase | Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PM10 (μg/m3) | PM2.5 (μg/m3) | CO (μg/m3) | SO2 (μg/m3) | NO2 (μg/m3) |
| Contract Package WBDWSIP/DWW/NCB/N24P/02A/2017-18 |  |  |  |  |  |  |  |  |
| Zone-2 | 2-3.12.2019 | Nebutala Abada | During construction | 171 | 87 | 791 | 12.4 | 46.6 |
| Zone-5 | 3-4.12.2019 | Kulti Uposasthya Kendra | During construction | 135 | 73 | 735 | 9.2 | 36.0 |
| Zone-19 | 3-4.12.2019 | Khardah Chandpur | Pre- construction | 141 | 80 | 769 | 10.1 | 38.1 |
| Zone-17 | 2-3.12.2019 | Shalipur | During construction | 167 | 93 | 1025 | 11.6 | 42.5 |
| Zone-21 | 4-5.12.2019 | Haroa Market | During construction | 197 | 116 | 1081 | 13.3 | 40.3 |
| Contract Package WBDWSIP/DWW/NCB/N24P/02B/2017-18 |  |  |  |  |  |  |  |  |
| Zone-16 | 29-30.11.2019 | Jawapur | Pre- construction | 109 | 61 | 583.4 | 9.2 | 34.5 |
| Zone-14 | 11-12.11.2019 | Pitapukuriya | Pre- construction | 125 | 72 | 686.4 | 9.7 | 37.4 |
| Zone-12 | 29-30.11.2019 | Bankochua | During construction | 131 | 79 | 697.8 | 8.3 | 32.1 |
| Zone-10 | 29-30.11.2019 | Shanpukuria | During construction | 139 | 81 | 594.9 | 9.0 | 33.8 |
| Zone-4 | 08-09.11.2019 | Swastayan Gacchi | During construction | 116 | 68 | 652.1 | 8.9 | 36.8 |
| Contract Package: WBDWSIP/DWW/NCB/N24P/01/2017-18 |  |  |  |  |  |  |  |  |
|  | 22-23.1.2020 | Water treatment plant | During construction | 140 | 74 | 458 | 10.6 | 40.0 |
|  | 22-23.1.2020 | Kormoyog Ashram | Pre- construction | 167 | 86 | 572 | 11.8 | 44.1 |
| National Ambient Air Quality Standard (NAAQS) |  |  |  | 100 | 60 | 2000 | 80 | 80 |
| WHO standard |  |  |  | 100 | 50 | - | 50* | 40* |

*Interim target 2

Note: National level Central Pollution Control Board’s (Govt. of India) guideline/ protocol used for sampling, For analysis. INDIAN STANDARD IS 5182 and USEPA CFR 40 (International protocol) used for analysis

Table 26: Surface Water quality monitoring Result – North 24 Pgs- During construction

| Site no. | Date of sampling | Site Location | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | pH | Turbidity (NTU) | Cl- (mg/l) | F (mg/l) | No3-(mg/l) | COD (mg/l) | BOD (mg/l) | DO (mg/l) | Total alkalinity(mg/l) | Hardness(mg/l) | Oil & Grease(mg/l) | Total coliform (MPN/ 100 ml) | Faecal coliform (MPN/100ml) |
|  | Contract Package WBDWSIP/DWW/NCB/N24P/02A/2017-18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Zone-3 | 17.10.2019* | Pond water of Nebutala Abada | 8.06 | 4.8 | 684 | 0.76 | <0.5 | 156.8 | 44.8 | 5.9 | 164 | 378 | 2.6 | 2.8 | BDL |
| Zone-13 | 17.10.2019* | Pond water in zone 13 | 7.48 | 9.4 | 86.44 | 0.36 | <0.5 | 20 | 5.6 | 4.6 | 192 | 226 | 6.2 | 9.4 | 3.6 |
| Contract Package WBDWSIP/DWW/NCB/N24P/02B/2017-18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Zone-4 | 17.10.2019* | Pond water of Swastayan Gacchi | 7.62 | 6.2 | 86.4 | <0.1 | <0.5 | 58.8 | 15.9 | 4.6 | 236 | 228 | 4.2 | 4.6 | BDL |
| Zone-12 | 17.10.2019* | Pond water of Bonkochua | 7.59 | 9.8 | 56.34 | <0.10 | <0.5 | 39.2 | 10.3 | 5.2 | 198 | 220 | 4.6 | 4.2 | 6.8 |
| Surface water quality criteria By CPCB (class B outdoor bathing) |  |  | 6.5-8.5 | --- | --- | 1.5 | --- | --- | 3 | 5 | --- | --- | --- | 500 | --- |

*Note: Water quality data is monitored once in 6 months. However, due to COVID-19 situation monitoring could not be conducted. Hence, last monitored data is presented here

Table 27: Raw Water quality monitoring Result – North 24 Pgs

Sampling Location: WTP, New Town	Sampling Date: 20.11.2019

Sample Description: Raw Water

- Biological Analysis Results

| Sl. No. | Characteristic | Norms as per IS 10500:2012 | Test Method | Results |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Total Coliform Bacteria/100ml | Not detectable | IS1622-1981, Reff. 2003 | 38 |  |
| 2 | E Coli/100ml | Not detectable | IS1622-1981, Reff. 2003 | Not detectable |  |

B) Chemical Analysis Results

| Sl. No. | Test Parameter | Test Method | As per Drinking water standard : IS:10500, 2012 |  | Results |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Desirable Limit | Permissible Limit |  |
| 1 | Colour in Hazen Units | IS 3025 (Part 4) 1983 (RA 2012) | 5 | 15 | <1.0 |
| 2 | Odour | IS 3025 (Part 4) 1983 (RA 2012) | Agreeable | Agreeable | Agreeable |
| 3 | Taste | IS 3025 (Part 7&8) 1984 (RA 2002) | Agreeable | Agreeable | Agreeable |
| 4 | pH Value at 250C | IS 3025 (Part 11)-1984; Rffm: 2006 | 6.5-8.5 | No Relaxation | 7.97 |
| 5 | Turbidity in NTU | IS 3025 (Part 14)-1984; Rffm: 2006 | 1 | 5 | <1.0 |
| 6 | Total Dissolved Solids (as TDS) in mg/l | IS 3025 (Part 16)-1984; RA 2012 | 500 | 2000 | 568 |
| 7 | Calcium (as Ca) in mg/l | IS 3025 (Part 40)-1991; Rffm 2009 | 75 | 200 | 88.4 |
| 8 | Chloride (as Cl) in mg/l | IS 3025 (Part 32)-1988; Rffm 2009 | 250 | 1000 | 74.6 |
| 9 | Iron (as Fe) in mg/l | IS 3025 (Part 53)-1988; Rffm 2009 | 0.3 | No Relaxation | 0.18 |
| 10 | Magnesium (as Mg) in mg/l | IS 3025 (Part 46)-1994; Rffm 2009 | 30 | 100 | 15.12 |
| 11 | Sulphate (as SO4) in mg/l | IS 3025 (Part 24)-1986; Rffm 2009 | 200 | 400 | 26.9 |
| 12 | Total Hardness (as CaCO3) in mg/l | IS 3025 (Part 21)-1983; Rffm 2003 | 200 | 600 | 284 |
| 13 | Alkalinity (as CaCO3) in mg/l | IS 3025 (Part 23)-1986, RA 2014 | 200 | 600 | 292 |

Table 28: Noise Level Monitoring data- North 24 Pgs

| Site No | Date of Testing | Site Location | Construction phase | Leq dB(A) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Day Time | Night Time |
| Contract Package WBDWSIP/DWW/NCB/N24P/02A/2017-18 |  |  |  |  |  |
| Zone-19 | 3-4.12.2019 | Khardah Chandpur | Pre- construction | 62.1 | 54.7 |
| Zone-17 | 2-3.12.2019 | Majanpur High School, Shalipur | During construction | 61.7. | 56.4 |
| Zone-21 | 4-5.12.2019 | Haroa Market | During construction | 65.4 | 58.6 |
| Zone-2 | 2-3.12.2019 | Nebutala Abada | During construction | 60.8 | 52.4 |
| Zone-5 | 3-4.12.2019 | Theghoria | During construction | 62.7 | 55.3 |
| Contract Package WBDWSIP/DWW/NCB/N24P/02B/2017-18 |  |  |  |  |  |
| Zone-16 | 30.11-01.12.2019 | Jawapur | Pre- construction | 55.9 | 48.2 |
| Zone-10 | 29-30.11.2019 | Paschim Majarhat Primary School, Shanpukuria | During construction | 57.8 | 50.4 |
| Zone-12 | 29-30.11.2019 | Bankochua | During construction | 58.6 | 50.9 |
| Zone-14 | 11-12.11.2019 | Pitapukuriya | Pre- construction | 59.3 | 51.8 |
| Zone-4 | 08-09.11.2019 | Swastayan Gacchi School | During construction | 57.3 | 50.6 |
|  |  | Contract Package: WBDWSIP/DWW/NCB/N24P/01/2017-18 |  |  |  |
|  | 22-23.1.2020 | Water treatment Plant | During construction | 65.4 | 54.5 |
|  | 22-23.1.2020 | Kormoyog Ashram | Pre- construction | 67.2 | 57.5 |
| Code/Category | Leq Day Time dB(A) - Standard |  |  | Leq Night Time dB(A) - Standard |  |
| A/Industrial | 75 |  |  | 70 |  |
| B/Commercial | 65 |  |  | 55 |  |
| C/Residential | 55 |  |  | 45 |  |
| D/Ecological- Sensitive | 50 |  |  | 40 |  |
|  |  | Note: National level Central Pollution Control Board’s, CPCB’s (Govt. of India) guideline/ protocol used for sampling |  |  |  |

Comparative Analysis of Monitoring Results

- The monitoring results obtained from same locations over time have been compared in the charts presented below.

Air Quality

- Of the 5 locations where ambient air quality has been monitored, 4 locations (i.e. Zone 2, 5, 17 and 21) in Haroa Block and 3 locations (i.e. Zone 4, 10 and 12) in Bhangar II Block are common with previous monitoring campaigns. Ambient air quality has been consistently monitored at WTP, New Town under Subproject 2. Hence, trend analysis over time is presented below for these locations only.

| - Comparison of Air Quality Parameters for Haroa Block |  |
| --- | --- |
| - Comparison of Air Quality Parameters for Bhangar II Block |  |

- Comparison of Air Quality Parameters for WTP

- Comparison of Surface Water Quality Parameters for Haroa Block*

- *Note: Since the water quality monitoring is conducted once in 6 months, therefore, the comparison has been made between Q2 2019 and Q4 2019. Water quality monitoring in Bhangar II Block was conducted at different locations hence, such comparison could not be made.

| Comparison of Noise Level in Haroa Block |  |  |  |
| --- | --- | --- | --- |
| Comparison of Noise Level in Bhangar II Block |  |  |  |

- For all the subprojects, PM10 and PM2.5 results during constructions are found to be exceeding the limits prescribed in National Ambient Air Quality Standard as well as WHO. Although, CO, SO2 and NO2 concentration levels –typically emitted due to combustion of fossil fuels – were all found within the prescribed National standard. Concentration of NO2 is just below the WHO standard. Therefore, it can be inferred that the increased levels of PM10 and PM2.5 are due to high background concentration.

- Analysis of trends for PM10 and PM2.5 at the monitored locations reveal consistently higher concentration during the last 6 months compared to preceding 6 months at Haroa Block whereas such increase was observed at Zone 10 and 12 of Bhangar II Block. However, the trend was found to be reversed at WTP.  The increase observed in Haroa block may be partially attributable to project activities. Mitigation measures like water sprinkling should be strictly followed at those sites.

- Contractors have been advised regularly to take necessary action on dust suppression by sprinkling of water whenever required.

- Water quality levels were found to be well within their respective limits in all the monitored samples, except BOD level. High BOD may be due to organic load from local activity/ use. Based on the results obtained it can be concluded that release of wastewater, mostly having biological loading has caused contamination of these water bodies and rendered these sources unsuitable for outdoor bathing.

- Analysis of trend in Haroa Block shows increase in BOD level over time, especially in Zone 2 where project work has commenced during the last 6 months. No such comparison is available for others packages.

- In case of noise level, while the limits are mostly exceeded when the monitoring is conducted in Package N 24pgs/02A and N 24pgs/02B near schools. healthcare facilities and residential areas, however, in other locations (commercial/ industrial) the noise levels were found to be within the stipulated limits during day time. However, no exceedance was observed in the package N 24pgs/01.

- Analysis of trend in noise level shows a consistent increase in Haroa Block over the last 6 months in comparison to previous 6- month period whereas trend was found to be reversed in Bhangar II Block while no such variation was observed in WTP.

- Noise producing activity should be limited near schools and healthcare facilities during working hours. Accordingly, instruction has been given to contractors.

- During Construction, air quality, noise level and water quality monitoring will be continued as per Environment Management and Monitoring Plan. All monitoring expenses will be borne by contractors from their project Health safety monitoring budget

- All original test certificates are available in DSISC/PIU office as back-up and result scanned copy attached as Appendix 10.

- Environmental Monitoring Results of Bankura Packages and observations

- Environmental monitoring results and observations of different packages under Bankura District are given below.

Table 29: Ambient Air Quality Monitoring Results- Bankura

| Site no. | Date of Testing | Site Location |  | Parameters |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PM10 (μg/m3) | PM2.5 (μg/m3) | CO (mg/m3) | SO2 (μg/m3) | NO2 (μg/m3) |
| Contract Package: WBDWSIP/DWW/NCB/BK/01/2017-18 |  |  |  |  |  |  |  |
| 1 | 04.06.2019 (Baseline Data)* | WTP Site- Lohadihi | 109 | 58 | 0.366 | 6.0 | 23.5 |
| 2 | 04.06.2019 (Baseline Data)* | GLSR site- Gobindapur | 118 | 64 | 0.389 | 5.7 | 26.0 |
| Contract Package: WBDWSIP/DWW/NCB/BK/02A/2018-19 |  |  |  |  |  |  |  |
| 1 | 02-03.07.19 (Baseline Data)* | GLSR Site- Raghunathpur | 54 | 31 | 0.185 | 5.0 | 16.4 |
| 2 | November, 19 (Baseline Data) | Gobindapur | 88 | 26 | 0.860 | 8.5 | 26.3 |
| 3 |  | Choukighata | 71 | 19 | 0.650 | 14.3 | 27.2 |
| 4 |  | Nayekhir | 93 | 27 | 0.420 | 5.3 | 21.5 |
| 5 |  | Gunnath | 65 | 18 | 0.530 | 8.6 | 30.8 |
| Contract Package: WBDWSIP/DWW/NCB/BK/02B/2018-19 |  |  |  |  |  |  |  |
| 1 | 02-03.07.19 (Baseline Data)* | OHSR Site- Saharghati | 48 | 26 | 0.168 | 4.5 | 15.8 |
| 2 | November, 19 (Baseline Data) | Chandabila (Satmouli) | 92 | 37 | 0.420 | 7.2 | 29.9 |
| 3 |  | Rajpur (Gamarbani) | 71 | 23 | 0.450 | 14 | 33 |
| 4 |  | Harmasra (Jemua) | 87 | 12 | 0.690 | 11.5 | 24.5 |
| 5 |  | Shibdanga More | 63 | 18 | 0.460 | 10.6 | 31.1 |
| Contract Package: WBDWSIP/DWW/NCB/BK/03/2018-19 |  |  |  |  |  |  |  |
| 1 | 01-02.07.19 (Baseline data)* | WTP Site Basudevpur/ Radhamadhabpur | 57 | 31 | 0.18 | 5.3 | 20.4 |
| 2 | November, 19 (Baseline data) | Intake point, Natungram | 76 | 18 | 0.620 | 14.7 | 25.6 |
| 3 | February, 20 [During Construction (1st)] | WTP area, Radhamadhabpur | 146.19 | 70.34 | 0.735 | 8.4 | 39.80 |
| 4 |  | Intake point, Natungram | 110.41 | 52.23 | 0.672 | 8.06 | 39.18 |
| Contract Package: WBDWSIP/DWW/NCB/BK/04/2018-19 |  |  |  |  |  |  |  |
| 1 | 01-02.07.19 (Baseline Data)* | OHSR site -Deruli- Desuriya primary school | 62 | 34 | 0.198 | 5.7 | 21.1 |
| 2 | November, 19 (Baseline data) | Bankadaha | 97 | 42 | 0.640 | 15 | 19.4 |
| 3 |  | Gangajalghati | 164 | 74 | 0.450 | 12.1 | 22.8 |
| 4 |  | Deuli | 94 | 35 | 0.560 | 11.1 | 21.8 |
| 5 |  | Machrakend | 188 | 65 | 0.430 | 15 | 19.4 |
| 6 |  | Ranipur | 235 | 76 | 0.370 | 12.3 | 18.8 |
| 7 |  | Barsal Srirampur | 98 | 24 | 0.410 | 8.7 | 21.8 |
| 8 |  | Durlabhpur | 64 | 30 | 0.460 | 16.3 | 17.2 |
| 9 |  | Chhotonabagram | 85 | 25 | 0.420 | 12.3 | 17.1 |
| 10 |  | Mohona | 173 | 79 | 0.600 | 13.2 | 19.1 |
| National Ambient Air Quality Standard |  |  | 100 | 60 | 2 | 80 | 80 |
| WHO standard |  |  | 100 | 50 | - | 50* | 40* |

* Done during earlier report period as base line

Table 30: Noise Level Monitoring Data- Bankura

| Site No | Date of Testing | Site Location | Leq dB(A) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Day Time | Night Time |
| Contract Package: WBDWSIP/DWW/NCB/BK/01/2017-18 |  |  |  |  |
| 1 | 04.06.2019 (Baseline Data)* | WTP Site-Lohadihi | 53.2 | 44.8 |
| 2 | 04.06.2019 (Baseline Data)* | GLSR Site- Gobindapur | 60.1 | 52.9 |
| 3 | 04.06.2019 (Baseline Data)* | Intake- Kumar Bahal | 51.1 | 44.7 |
| Contract Package: WBDWSIP/DWW/NCB/BK/02A/2018-19 |  |  |  |  |
| 1 | 02.07.2019 (Baseline Data)* | GLSR Site- Raghunathpur | 41.6 | 35.3 |
| 2 | November, 19 (Baseline data) | Gobindapur | 59.4 | 48.6 |
| 3 |  | Choukighata | 56.4 | 39.0 |
| 4 |  | Nayekhir | 57.8 | 42.5 |
| 5 |  | Gunnath | 56.9 | 41.5 |
| Contract Package: WBDWSIP/DWW/NCB/BK/02B/2018-19 |  |  |  |  |
| 1 | 02.07.2019* (Baseline data) | OHSR Site- Saharghati Saharghati (Patrabandh)-Revised Dated Dec, 2019 | 57.6 43.8 | 64.8 37.4 |
| 2 | December, 19 (Baseline data) | Chandabila (Satmouli) | 58.4 | 40.6 |
| 3 |  | Rajpur (Gamarbani) | 51.8 | 42.1 |
| 4 |  | Harmasra (Jemua) | 54.0 | 32.6 |
| 5 |  | Shibdanga More | 41.1 | 35.5 |
| 6 | March, 20 (Baseline data) | Amdangra | 59.6 | 44.0 |
| 7 |  | Bindalaxmanpur | 61.0 | 51.0 |
| 8 |  | Dhobajor | 63.7 | 45.5 |
| 9 |  | Koniyara | 60.6 | 44.9 |
| 10 |  | Manikmara | 59.3 | 46.6 |
| Contract Package: WBDWSIP/DWW/NCB/BK/03/2018-19 |  |  |  |  |
| 1 | 01.07.2019* (Baseline data) | WTP Site Basudevpur/ Radhamadhabpur | 51.6 | 40.7 |
| 2 | November, 19 (Baseline data) | Intake point, Natungram | 43.6 | 34.5 |
| 3 |  | School, near intake point | 58.4 | 52.0 |
| 4 |  | Hospital, near WTP area | 53.4 | 48.7 |
| 5 | February, 20 [During Construction (1st)] | WTP area, Basudevpur | 60.8 | 46.0 |
| 6 |  | Intake point, Natungram | 56.7 | 43.4 |
| 7 |  | Sri Chandanpur Primary School, Nritanandyapur, Kolgora | 57.7 | 42.7 |
| 8 |  | Nityanandapur, Kolgora, near PHC | 54.5 | 43.0 |
| Contract Package: WBDWSIP/DWW/NCB/BK/04/2018-19 |  |  |  |  |
| 1 | 01.07.2019 (Baseline data)* | OHSR site -Deruli- Desuriya primary school | 45.1 | 36.1 |
| 2 | November, 19 (Baseline data) | Gangajalghati | 43.9 | 35.9 |
| 3 |  | Bankadaha | 42.5 | 35.3 |
| 4 |  | Banasuriya | 36.6 | 36.3 |
| 5 |  | Purbator | 36.0 | 35.5 |
| 6 |  | Benabad | 36.1 | 36.0 |
| 7 |  | Mejhia | 42.5 | 36.0 |
| 8 |  | Lachhmanpur | 37.2 | 36.1 |
| 9 |  | Mohoana | 46.7 | 41.0 |
| 10 |  | Chhotonabagram | 57.8 | 50.0 |
| 11 |  | Durlabhpur | 46.1 | 36.0 |
| 12 |  | Barsal Srirampur | 47.4 | 36.2 |
| 13 |  | Ranipur | 46.4 | 36.5 |
| 14 |  | Mochrakend | 45.5 | 37.1 |
| 15 |  | Deuli | 46.2 | 36.2 |
| 16 |  | Raghunathpur | 36.1 | 35.3 |
| 17 |  | Keduadihi (Kapista) | 36.2 | 35.9 |
| 18 |  | Sarangapur | 37.3 | 36.6 |
| 19 |  | Lakhiyara | 36.4 | 35.9 |
| 20 |  | Subaira | 37.4 | 35.5 |
| Code/Category | Leq Day Time dB(A) - Standard |  | Leq Night Time dB(A) - Standard |  |
| A/Industrial | 75 |  | 70 |  |
| B/Commercial | 65 |  | 55 |  |
| C/Residential | 55 |  | 45 |  |
| D/Ecological- Sensitive | 50 |  | 40 |  |
| Note: National level Central Pollution Control Board’s, CPCB’s (Govt. of India) guideline/ protocol used for sampling |  |  |  |  |

* Done during earlier report period as base line

Table 31: Ground Water Quality Monitoring Data for BK/02A- Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02A/2018-19 |  |  |  |  | National Standards for Drinking Water-BIS 10500-2012 Acceptable limit (Maximum permissible limit) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |  |
|  |  |  | Location: Raghunathpur Date of sampling 27.06.2019 | Location: Gobindapur Date of sampling 30.11.19 | Location: Choukighata Date of sampling 30.11.19 | Location: Nayekhir Date ofsampling 30.11.19 | Location: Gunnath Date of sampling 30.11.19 |  |
| 1. | Turbidity | NTU | <1.0 | 0.14 | 0.15 | 0.20 | 0.22 | 1 (5) |
| 2. | pH | - | 7.72 | 7.4 | 7.4 | 7.5 | 7.6 | 6.5-8.5 |
| 3. | Colour | Hazan Unit | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | 5 (15) |
| 4. | Taste and Odor | - | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable |
| 5. | TDS | Mg/l | 420 | 560 | 1214 | 1178 | 244 | 500 (2000) |
| 6. | Iron | Mg/l | <0.05 | 0.002 | 0.021 | 0.321 | 0.236 | 0.3 |
| 7. | Manganese | Mg/l | <0.05 | 0.043 | 0.088 | 0.022 | 0.041 | 0.1 (0.3) |
| 8. | Arsenic | Mg/l | <0.01 | <0.001 | 0.002 | 0.002 | <0.001 | 0.01 (0.05) |
| 9. | Cadmium | Mg/l | <0.002 | <0.001 | <0.001 | <0.001 | <0.001 | 0.003 |
| 10. | Chromium | Mg/l | <0.02 | 0.001 | <0.001 | <0.001 | <0.001 | 0.05 |
| 11. | Cyanide | Mg/l | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.05 |
| 12. | Fluoride | Mg/l | 0.41 | <0.05 | <0.05 | <0.05 | <0.05 | 1 (1.5) |
| 13. | Lead | Mg/l | <0.005 | <0.001 | <0.001 | <0.001 | <0.001 | 0.01 |
| 14. | Ammonia | Mg/l | <0.5 | <0.05 | <0.05 | <0.05 | <0.05 | 0.5 |
| 15. | Chloride | Mg/l | 105.3 | 28.08 | 229.30 | 219.93 | 40.71 | 250 (1000) |
| 16. | Sulphate | Mg/l | 25.73 | 6.24 | 51.42 | 45.27 | 8.65 | 200 (400) |
| 17. | Nitrate | Mg/l | 3.37 | <1.0 | <1.0 | <1.0 | <1.0 | 45 |
| 18. | Copper | Mg/l | <0.04 | <0.001 | <0.001 | <0.001 | <0.001 | 0.05 (1.5) |
| 19. | Total Hardness | Mg/l | 364 | 310 | 460 | 450 | 106 | 200 (600) |
| 20. | Calcium | Mg/l | 97.6 | 88.18 | 136.27 | 132.26 | 32.06 | 75 (200) |
| 21. | Zinc | Mg/l | 0.48 | 0.009 | 0.019 | 0.007 | 0.007 | 5 (15) |
| 22. | Mercury | Mg/l | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | 0.001 |
| 23. | Aluminium | Mg/l | <0.02 | <0.001 | <0.001 | <0.001 | <0.001 | 0.1 (0.3) |
| 24. | Residual Chlorine | Mg/l | 0.2 | <0.05 | <0.001 | <0.05 | <0.05 | 0.2 |
| 25. | E-Coli | MPN/100 ml | - | ND | ND | ND | ND | Must not be detectable in 100 ml ND sample |
| 26. | Total Coliform | MPN/ 100 ml | - | ND | ND | ND | ND | Must not be detectable in 100 ml sample |

Table 32: Ground Water Quality Monitoring Data for BK/02B- Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02B/2018-19 |  |  |  |  | National Standards for Drinking Water-BIS 10500-2012 Acceptable limit (Maximum permissible limit) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |  |
|  |  |  | Location: Saharghati(Patrabandh) Date of sampling 27.06.2019 | Location: Harmasra Date of sampling 30.11.19 | Location: Rajpur Date of sampling 30.11.19 | Location: Shibdanga More Date of sampling 30.11.19 | Location: Chandabila Date of sampling 30.11.19 |  |
| 1. | Turbidity | NTU | <1.0 | 0.15 | 0.16 | 0.18 | 0.22 | 1 (5) |
| 2. | pH | - | 7.43 | 7.3 | 7.4 | 7.4 | 7.5 | 6.5-8.5 |
| 3. | Colour | Hazan Unit | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | 5 (15) |
| 4. | Taste and Odor | - | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable |
| 5. | TDS | Mg/l | 142 | 54 | 146 | 66 | 130 | 500 (2000) |
| 6. | Iron | Mg/l | <0.05 | 0.038 | 0.106 | 0.038 | 0.037 | 0.3 |
| 7. | Manganese | Mg/l | <0.05 | 0.011 | 0.021 | 0.014 | 0.052 | 0.1 (0.3) |
| 8. | Arsenic | Mg/l | <0.01 | <0.001 | <0.001 | <0.001 | <0.001 | 0.01 (0.05) |
| 9. | Cadmium | Mg/l | <0.002 | <0.001 | <0.001 | <0.001 | <0.001 | 0.003 |
| 10. | Chromium | Mg/l | <0.02 | 0.002 | <0.001 | <0.001 | <0.001 | 0.05 |
| 11. | Cyanide | Mg/l | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.05 |
| 12. | Fluoride | Mg/l | 0.13 | <0.05 | <0.05 | <0.05 | <0.05 | 1 (1.5) |
| 13. | Lead | Mg/l | <0.005 | <0.001 | <0.001 | < 0.001 | <0.001 | 0.01 |
| 14. | Ammonia | Mg/l | <0.5 | <0.05 | <0.05 | <0.05 | <0.05 | 0.5 |
| 15. | Chloride | Mg/l | 9.57 | 9.36 | 18.72 | 9.36 | 9.36 | 250 (1000) |
| 16. | Sulphate | Mg/l | 8.11 | 2.14 | 4.16 | 2.17 | 2.32 | 200 (400) |
| 17. | Nitrate | Mg/l | 0.51 | <1.0 | <1.0 | <1.0 | <0.05 | 45 |
| 18. | Copper | Mg/l | <0.04 | <0.001 | <0.001 | <0.001 | <0.001 | 0.05 (1.5) |
| 19. | Total Hardness | Mg/l | 56 | 22 | 66 | 30 | 64 | 200 (600) |
| 20. | Calcium | Mg/l | 19.2 | 9.36 | 20.04 | 8.02 | 16.83 | 75 (200) |
| 21. | Zinc | Mg/l | 0.48 | 0.039 | 0.119 | 0.028 | 0.028 | 5 (15) |
| 22. | Mercury | Mg/l | <0.001 | <0.001 | <0.001 | <0.001 | <0.001 | 0.001 |
| 23. | Aluminium | Mg/l | <0.02 | 0.020 | 0.004 | <0.001 | <0.001 | 0.1 (0.3) |
| 24. | Residual Chlorine | Mg/l | 0.2 | <0.05 | <0.05 | <0.05 | <0.05 | 0.2 |
| 25. | E-Coli | MPN/100 ml | - | ND | ND | ND | ND | Must not be detectable in 100 ml sample |
| 26. | Total Coliform | MPN/ 100 ml | - | ND | ND | ND | ND | Must not be detectable in 100 ml sample |

Table 33: Ground Water Quality Monitoring Data for BK/04- Bankura

| Sl.No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/04/2018-19 |  |  |  |  |  |  |  |  |  | National Standards for Drinking Water-BIS 10500-2012 Acceptable limit (Maximum permissible limit) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) [July, 19 & November, 19] |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Location: Desuriya- Deuli 02.07.2019 | Location: Chausal 23.12.2019 | Location: Machaparulia 23.12.2019 | Location: Ranipur 23.12.2019 | Location: Gangajal-ghati 23.12.2019 | Location: Bankadaha 23.12.2019 | Location: Gobindadham 23.12.2019 | Location: Mochrakend 23.12.2019 | Location: Mohona 23.12.2019 | Location: Barsalsrirampur 23.12.2019 |  |
| 1. | Turbidity | NTU | <1.0 | 2.95 | 0.55 | 1.97 | 3.85 | 6.16 | 1.38 | 1.01 | 5.90 | 6.05 | 1 (5) |
| 2. | pH | - | 6.78 | 7.46 | 7.48 | 7.5 | 7.52 | 7.61 | 7.52 | 7.49 | 7.58 | 7.53 | 6.5-8.5 |
| 3. | Colour | Hazan Unit | <1.0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 5 (15) |
| 4. | Taste and Odor | - | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable | Agreeable |
| 5. | TDS | Mg/l | 1796 | 189 | 473 | 526 | 772 | 1453 | 538 | 983 | 1176 | 1140 | 500 (2000) |
| 6. | Iron | Mg/l | 1.10 | 0.1 | BDL | BDL | 0.06 | 0.13 | 0.08 | 0.11 | 0.13 | 0.12 | 0.3 |
| 7. | Manganese | Mg/l | <0.05 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.1 (0.3) |
| 8. | Arsenic | Mg/l | <0.01 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.01 (0.05) |
| 9. | Cadmium | Mg/l | <0.002 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.003 |
| 10. | Chromium | Mg/l | <0.02 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.05 |
| 11. | Cyanide | Mg/l | <0.01 | - | - | - | - | - | - | - | - | - | 0.05 |
| 12. | Fluoride | Mg/l | <0.1 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 1 (1.5) |
| 13. | Lead | Mg/l | <0.005 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.01 |
| 14. | Ammonia | Mg/l | <0.5 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.5 |
| 15. | Chloride | Mg/l | 380.6 | 13.82 | 50.69 | 65.99 | 75.12 | 76.51 | 56.78 | 78.95 | 96.51 | 80.42 | 250 (1000) |
| 16. | Sulphate | Mg/l | 110.1 | 9.23 | 27.92 | 38.15 | 47.98 | 46.96 | 38.32 | 52.72 | 45.38 | 43.40 | 200 (400) |
| 17. | Nitrate | Mg/l | 3.24 | 4 | 3.80 | 4.20 | 5.40 | 5.60 | 4.40 | 4.40 | 6.0 | 5.50 | 45 |
| 18. | Copper | Mg/l | <0.04 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.05 (1.5) |
| 19. | Total Hardness | Mg/l | 915.2 | 161.5 | 180.5 | 418 | 503.50 | 750.50 | 266 | 940.50 | 1073.50 | 551 | 200 (600) |
| 20. | Calcium | Mg/l | 258 | 18.44 | 14.75 | 33.19 | 106.93 | 165.93 | 14.75 | 143.81 | 169.62 | 132.74 | 75 (200) |
| 21. | Zinc | Mg/l | 0.42 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 5 (15) |
| 22. | Mercury | Mg/l | <0.001 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.001 |
| 23. | Aluminium | Mg/l | <0.02 | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 0.1 (0.3) |
| 24. | Residual Chlorine | Mg/l | 0.2 | Nil | Nil | Nil | Nil | Nil | Nil | Nil | Nil | Nil | 0.2 |
| 25. | E-Coli | MPN/100 ml | - | ND | ND | ND | ND | ND | ND | ND | ND | ND | Must not be detectable in 100 ml sample |
| 26. | Total Coliform | MPN/ 100 ml | - | ND | ND | ND | ND | ND | ND | ND | ND | ND | Must not be detectable in 100 ml sample |

Table 34: Surface Water Quality Monitoring data –PKG 02A, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02A/2018-19 |  |  |  |  | Surface Water Quality Standards (as per IS: 2296) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |  |  |  |
|  |  |  | Location: Raghunathpur Date of sampling: 30.11.19 | Location: Gobindapur Date of sampling: 30.11.19 | Location: Choukighata Date of sampling: 30.11.19 | Location: Nayekhir Date of Sampling: 30.11.19 | Location: Gunnath Date of sampling: 30.11.19 | Category-B | Category-D | Category-E |
| 1. | pH (max) | (min-6.5) | 7.7 | 7.6 | 7.4 | 7.6 | 7.5 | 8.5 | 8.5 | 8.5 |
| 2. | DO (minimum) | Mg/l | 6.60 | 6.81 | 6.50 | 6.60 | 6.20 | 5 | 4 | -- |
| 3. | BOD (3D, 270 C) | Mg/l | 2.5 | 3.0 | 2.1 | 3.1 | 3.2 | 3 | -- | -- |
| 4. | O & G | Mg/l | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | -- | 0.1 | -- |
| 5. | Total Hardness as CaCO3 | Mg/l | 70 | 60 | 42 | 62 | 82 | -- | -- | --- |
| 6. | Chloride as Cl | Mg/l | -- | -- | -- | -- | -- | -- | -- | 600 |
| 7. | Nitrate as NO3 | Mg/l | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | -- | -- | -- |
| 8. | Fluorides as F | Mg/l | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | 1.5 | -- | -- |
| 9. | Total Coliforms | MPN/100 ml | ND | ND | ND | ND | ND | 500 | -- | -- |
| 10. | Fecal Coliforms | MPN/100 ml | ND | ND | ND | ND | ND | --- | -- | -- |
| 11. | Turbidity | NTU | 0.92 | 0.86 | 1.4 | 1.75 | 2.58 | --- | --- | --- |
| 12. | COD | Mg/l | 10.40 | 12.48 | 8.32 | 12.48 | 14.56 | --- | --- | -- |
| 13. | Total Alkalinity | Mg/l | 60 | 86 | 36 | 80 | 104 | --- | --- | --- |

Table 35: Surface Water Quality Monitoring data –PKG 02B, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02B/2018-19 |  |  |  |  | Surface Water Quality Standards (as per IS: 2296) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |  |  |  |
|  |  |  | Location: Harmasra Date of sampling: 30.11.19 | Location: Partabandh Date of sampling: 30.11.19 | Location: Shibdanga More Date of sampling: 30.11.19 | Location: Chandabila Date of Sampling: 30.11.19 | Location: Rajpur Date of sampling: 30.11.19 | Category-B | Category-D | Category-E |
| 1. | pH (max) | (min-6.5) | 7.6 | 7.5 | 7.5 | 7.6 | 7.7 | 8.5 | 8.5 | 8.5 |
| 2. | DO (minimum) | Mg/l | 6.40 | 6.60 | 6.60 | 6.81 | 6.50 | 5 | 4 | -- |
| 3. | BOD (3D, 270 C) | Mg/l | 2.6 | 2.8 | 2.3 | 2.2 | 2.4 | 3 | -- | -- |
| 4. | O & G | Mg/l | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | -- | 0.1 | -- |
| 5. | Total Hardness as CaCO3 | Mg/l | 70 | 50 | 50 | 30 | 42 | -- | -- | --- |
| 6. | Chloride as Cl | Mg/l | -- | -- | -- | --- | -- | -- | -- | 600 |
| 7. | Nitrate as NO3 | Mg/l | <1.0 | <1.0 | <1.0 | <1.0 | <1.0 | -- | -- | -- |
| 8. | Fluorides as F | Mg/l | <0.05 | <0.05 | <0.05 | <0.05 | <0.05 | 1.5 | -- | -- |
| 9. | Total Coliforms | MPN/100 ml | ND | ND | ND | ND | ND | 500 | -- | -- |
| 10. | Fecal Coliforms | MPN/100 ml | ND | ND | ND | ND | ND | --- | -- | -- |
| 11. | Turbidity | NTU | 0.7 | 0.62 | 1.62 | 2.96 | 2.45 | --- | --- | --- |
| 12. | COD | Mg/l | 11.44 | 11.44 | 9.36 | 8.32 | 9.36 | --- | --- | -- |
| 13. | Total Alkalinity | Mg/l | 80 | 80 | 60 | 30 | 36 | --- | --- | --- |

Table 36 Intake Water (Durgapur Barrage- Damodar River) Quality Monitoring data –PKG 03, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/03/2018-19 |  | Surface Water Quality Standards (as per IS: 2296) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) | Results (During Construction- 1st) |  |  |  |  |  |
|  |  |  | Location: Intake Site (Damodar River) Date of sampling- 12.12.19 | Location: Intake Site (Damodar River) Date of sampling- 10.02.20 | Category- A | Category-B | Category- C | Category-D | Category-E |
| 1. | pH (max) | (min-6.5) | 7.6 | 7.24 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 |
| 2. | DO (minimum) | Mg/l | 6.50 | 3.65 | 6 | 5 | 4 | 4 | -- |
| 3. | BOD (3D, 270 C) | Mg/l | 2.4 | 6.0 | 2 | 3 | 3 | -- | -- |
| 4. | O & G | Mg/l | <1.0 | <3.0 | -- | -- | 0.1 | 0.1 | -- |
| 5. | Total Hardness as CaCO3 | Mg/l | 100 | 76 | 300 | -- | -- | -- | --- |
| 6. | Chloride as Cl | Mg/l | -- | 54.96 | 250 | -- | 600 | -- | 600 |
| 7. | Nitrate as NO3 | Mg/l | <1.0 | 7.4 | 20 | -- | 50 | -- | -- |
| 8. | Fluorides as F | Mg/l | <0.05 | BDL | 1.5 | 1.5 | 1.5 | -- | -- |
| 9. | Total Coliforms | MPN/100 ml | ND | -- | 50 | 500 | 5000 | -- | -- |
| 10. | Fecal Coliforms | MPN/100 ml | ND | -- | -- | --- | -- | -- | -- |
| 11. | Turbidity | NTU | 2.34 | 2.24 | -- | --- | -- | --- | --- |
| 12. | COD | Mg/l | 12.48 | 49.6 | -- | --- | -- | --- | -- |
| 13. | Total Alkalinity | Mg/l | 116 | 70 | -- | --- | -- | --- | --- |

(Due to COVID 19 pandemic Lockdown – monitoring not done from March 2020)

Table 37: Surface Water Quality Monitoring data –PKG 04, Bankura

| Parameter | Unit | Mohana 23.12.19 | Machaparulia 23.12.19 | Basudevpur 23.12.19 | G.Ghati 23.12.19 | Gobindadham 23.12.19 | Ranipur 23.12.19 | Bankadaha 23.12.19 | Deuli 23.12.19 | Mochrakend 23.12.19 | Barasal 23.12.19 | Standard as per IS 2296:1982 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH |  | 6.63 | 6.92 | 7.04 | 6.83 | 7.02 | 6.86 | 7.44 | 7.17 | 6.55 | 6.51 | 6.5-8.5 |
| Turbidity | mg/l | 2.3 | 2.92 | 2.69 | 38.9 | 28.5 | 40.6 | 59 | 5.68 | 22.1 | 8.29 |  |
| Hardness | mg/l | 123.5 | 209 | 114 | 114 | 114 | 85.5 | 75 | 85 | 104 | 57 |  |
| Alkalinity | mg/l | 30 | 190 | 100 | 80 | 120 | 90 | 130 | 120 | 70 | 40 |  |
| Cl | mg/l | 46.8 | 87.56 | 36.87 | 18.43 | 23.04 | 36.87 | 55.3 | 50.69 | 59.91 | 27.65 | 600 |
| NO3 | mg/l | 8.14 | 8.98 | 9.1 | 8.64 | 8.64 | 8.64 | 8.64 | 9.2 | 7.4 | 6.24 | 50 |
| Total Coliform (MPN/ 100 ml) |  | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | 500 |
| Faecal Coliform (MPN/100 ml) |  | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | BDL | Nil |

(Testing continued for other chemical parameters)

NOTE:

- Category A-Drinking water without conventional treatment but after disinfection.

- Category B- Water for outdoor bathing

- Category C-Drinking water with conventional treatment followed by disinfection

- Category D- Water for fish culture and wild life propagation

- Category E- Water for irrigation, industrial cooling and controlled waste disposal

Table 38: Soil Quality Monitoring data –PKG 02A, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02A/2018-19 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |
|  |  |  | Location: Belasuly- shyamsundarpur Date of sampling: 17.02.20 | Location: Golakpur Date of sampling: 17.02.20 | Location: Tunamara Date of sampling: 17.02.20 | Location: Bholarkhap Date of Sampling: 17.02.20 | Location: Moukuri Date of sampling: 17.02.20 |
| 1. | pH | -- | 6.75 | 6.27 | 7.51 | 6.75 | 7.50 |
| 2. | Elec. Conductivity | µS/cm | 37.80 | 22.50 | 41.70 | 37.80 | 23.10 |
| 3. | Moisture | % | 2.15 | 6.94 | 2.35 | 2.15 | 7.89 |

(Testing continued for other chemical parameters)

Table 39 Soil Quality Monitoring data –PKG 02B, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/02B/2018-19 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |
|  |  |  | Location: Bindalaxmanpur Date of sampling: 16.03.20 | Location: Amdangra Date of sampling: 16.03.20 | Location: Dhobajora Date of sampling: 16.03.20 | Location: Koniyara Date of Sampling: 16.03.20 | Location: Manikmara Date of sampling: 16.03.20 |
| 1. | pH | -- | 4.61 | 4.46 | 6.17 | 5.15 | 4.74 |
| 2. | Elec. Conductivity | µS/cm | 97.4 | 212 | 62.6 | 134.9 | 84.5 |
| 3. | Moisture | % | 8.03 | 6.22 | 8.83 | 7.08 | 9.96 |
| 4. | Permeability | cm/S | 5.6 | 6.1 | 5.1 | 5.7 | 5 |
| 5. | Texture | % | Sand- 77 Clay-14 Silt- 9 | Sand-77 Clay-17 Silt-6 | Sand-75 Clay-20 Silt-5 | Sand- 72 Clay-12 Silt-16 | Sand-75 Clay-16 Silt-9 |
| 6. | Calcium as Ca | mg/kg | -- | -- | -- | -- | -- |
| 7. | Magnesium as Mg | mg/kg | 12.62 | 19.07 | 36.5 | 10.7 | 33 |
| 8. | Nitrogen as N | Kg/Hectare | 222 | 261 | 282 | 296 | 218 |
| 9. | Potassium as K | mg/kg | 8.78 | 25.88 | 25.62 | 6.02 | 20.91 |
| 10. | Sodium as Na | mg/kg | 5.48 | 9.53 | 7.04 | 7.35 | 6.04 |
| 11. | Phosphate as PO4 | mg/kg | 82.21 | 217.7 | 51.36 | 71.78 | 61.26 |
| 12. | Organic Matter | % | 1.15 | 0.98 | 1.0 | 1.37 | 0.71 |
| 13. | Oil & Grease | mg/kg | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 |

Table 40: Soil Quality Monitoring data –PKG 04, Bankura

| Sl. No. | Parameters | Unit | Contract Package: WBDWSIP/DWW/NCB/BK/04/2018-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Results (Baseline Data) |  |  |  |  |  |  |  |  |  |
|  |  |  | Location: Ranipur Date of sampling: 23.12.19 | Location: Barsal- srirampur Date of sampling: 23.12.19 | Location: Mohona Date of sampling: 23.12.19 | Location: Mochrakend Date ofSampling: 23.12.19 | Location: Gobindadham Date of sampling: 23.12.19 | Location: Bankadaha Date of Sampling: 23.12.19 | Location: Chausalk Date of Sampling: 23.12.19 | Location: Gangajalghati Date of Sampling: 23.12.19 | Location: Deuli Date of Sampling: 23.12.19 | Location: Machaparuluia Date of Sampling: 23.12.19 |
| 1. | pH | -- | 7.12 | 6.97 | 7.15 | 7.22 | 6.9 | 6.9 | 6.65 | 7.02 | 7.06 | 6.94 |
| 2. | Elec. Conductivity | µS/cm | 429 | 418 | 445 | 469 | 430 | 430 | 412 | 456 | 402 | 428 |
| 3. | Moisture | % | 10.2 | 9 | 10.6 | 9.8 | 8.8 | 8.8 | 9.2 | 12 | 9.8 | 11 |
| 4. | Permeability | cm/S | 10-9 to 10-6 | 10-9 to 10-6 | 10-9 to 10-6 | 10-9 to 10-6 | 10-5 to 10-3 | 10-5 to 10-3 | 10-3 to 1 | 10-5 to 10-3 | 10-9 to 10-6 | 10-5 to 10-3 |
| 5. | Texture | -- | Sandy Loam | Sandy | Sandy Loam | Sandy Loam | Sandy clay loam | Sandy clay loam | Clay loam | Sandy clay loam | Sandy Loam | Sandy clay loam |
| 6. | Calcium as Ca | mg/kg | 96.18 | 77.84 | 91.63 | 88.92 | 74.16 | 74.16 | 74.16 | 88.56 | 88.68 | 72.24 |
| 7. | Magnesium as Mg | mg/kg | 35.46 | 27.61 | 34 | 32.26 | 28 | 28 | 28 | 31.24 | 34.04 | 32.28 |
| 8. | Nitrogen as N | Kg/hectare | 472.1 | 437.82 | 485.16 | 470.18 | 464.22 | 464.22 | 450.22 | 435.14 | 446.52 | 480.5 |
| 9. | Potassium as K | mg/kg | 74.8 | 79.16 | 88.44 | 80.49 | 74.96 | 74.96 | 71.18 | 80.36 | 68.32 | 74.52 |
| 10. | Sodium as Na | mg/kg | 41.39 | 44.26 | 50.02 | 47.65 | 42.29 | 42.29 | 40.26 | 45.62 | 38.15 | 41.98 |
| 11. | Phosphorous as P | mg/kg | 0.81 | 0.9 | 1.26 | 1.11 | 0.89 | 0.89 | 0.77 | 1.08 | 0.66 | 0.78 |
| 12. | Organic Matter | % | 4.8 | 4.6 | 5.2 | 5 | 4.6 | 4.6 | 4.4 | 5.2 | 4.2 | 4.8 |
| 13. | Oil & Grease | mg/kg | 0.87 | 0.64 | 0.8 | 0.92 | 0.6 | 0.6 | 0.65 | 1.02 | 0.94 | 1.16 |

Analyses of Results

- For the sub-project packages, PM10 and PM2.5 base line concentrations are found to be higher in some places in BK/01 (Both WTP and GLSR, as monitoring done during earlier report period) and BK/04 (Gangajalghati, Mochrakend, Ranipur and Mohona) than the prescribed limit set by National Ambient Air Quality Standard as well as WHO. In all those cases, the project sites are located near vehicular road, which might be the cause of increasing dust particles during day time. However, CO, SO2 and NO2 concentration levels are within limits in those places.

- The concentration of PM10 and PM2.5 increased in WTP and intake location in BK/03 while comparing to baseline condition (Figure 12 A, B, C & D) might be due to dust generation from construction activities. Contractors are instructed to take necessary mitigation measures for dust suppression in and around the project locations.

| Figure 12 A: Air Quality Result comparison for BK/03 | Figure 12 B: Air Quality Result comparison for BK/03 |
| --- | --- |
| Figure 12 C: Air Quality Result comparison for BK/03 | Figure 12 D: Air Quality Result comparison for BK/03 |

- Base line noise levels at most of the locations were within the limit considering the standard set for mixed zone (commercial zone). However, it is slightly higher during day time considering standard set for residential areas.  Noise level at sensitive area of BK/03 package marginally above the limit. In all those cases, the project sites are located near vehicular road, which might be the cause of increasing noise level during day time. Mitigation measures are applied during construction time, as per site condition. Comparison of noise level monitoring data for package BK/03 is given below (Figure 13).

Figure 13:Comparison of Noise Level monitoring data for BK/03

- In all cases, surface water quality results were within prescribed limit and can be classified as Category B, i.e. suitable for outdoor bathing.

- Drinking water parameters were within the prescribed limit, except samples as collected from some locations under package BK/02A and BK/04. At those location concentrations of Iron, TDS, Hardness, Chloride and Calcium were high and sometime above the standard.

- In all cases, soil quality results were within prescribed limit. In most places sandy loamy to clay loamy soils were found.

- Results of some parameters under baseline monitoring survey are pending for surface water quality in BK/01 and BK/04 and for soil quality in BK/2A. Results will be reflected in next SEMR.

- During Construction, air quality, noise level, soil and water quality monitoring will be continued as per Environment Management and Monitoring Plan. All monitoring expenses will be borne by contractors from their project Health safety monitoring budget

- All original test certificates are available in DSISC office as back-up and result scanned copy (except soil) attached as Appendix 10.

- Environmental Monitoring Results of East Medinipur Package and observations

- Base line monitoring for air quality, noise level, surface and has been completed for EM/02 package. Monitoring for package EM/01 will be done after finalization of design and before commencement of construction work. All original test certificates are available in DSISC office as back-up and result scanned copy (except soil) attached as Appendix 10.

Table 41: Ambient Air Monitoring Data (Package- EM/02)

| Sl. No. | Station Code | Village Name | Study Area | Date | Parameters (Government Standards) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PM10 (μg/m3) | PM2.5 (μg/m3) | CO (mg/m3) | SO2 (μg/m3) | NO2 (μg/m3) |
|  | AQ1 | Garchakraberia Near Primary School | Sensitive | 11.01.2020 | 63.45 | 31.12 | 0.469 | 7.65 | 36.42 |
|  | AQ2 | Jambari Near OHR | Rural Residential | 13.01.2020 | 72.57 | 35.87 | 0.476 | 8.34 | 37.44 |
|  | AQ3 | Manuchak Jalpai | Rural Residential | 11.01.2020 | 72.18 | 35.4 | 0.516 | 8.12 | 37.16 |
|  | AQ4 | Parbatipur,Near OHR | Rural Residential | 09.01.2020 | 63.58 | 30.68 | 0.422 | 7.48 | 36.50 |
|  | AQ5 | Nandigram CT | Rural Residential | 09.01.2020 | 79.8 | 38.52 | 0.614 | 8.92 | 38.66 |
|  | AQ6 | Haripur, Near OHR | Rural Residential | 10.01.2020 | 67.95 | 33.62 | 0.485 | 7.92 | 37.22 |
|  | AQ7 | Kamalpur, Near OHR | Rural Residential | 13.01.2020 | 55.14 | 26.72 | 0.368 | 7.02 | 35.94 |
|  | AQ8 | Birulia | Rural Residential | 08.01.2020 | 66.06 | 31.59 | 0.428 | 7.81 | 36.84 |
|  | AQ9 | Monoharpur, Near Primary School | Sensitive | 08.01.2020 | 70.35 | 34.18 | 0.503 | 8.06 | 37.42 |
|  | AQ10 | Kanakpur, Near OHR | Rural Residential | 12.01.2020 | 74.68 | 36.49 | 0.526 | 8.04 | 37.82 |
| National Ambient Air Quality Standard |  |  |  |  | 100 | 60 | 2 | 80 | 80 |
| WHO Standard |  |  |  |  | 100 | 50 | - | 50 | - |

Table 42 Noise Monitoring Data (Package- EM/02)

| Sl. No. | Station Code | Village Name |  | Study Area | Date | Leq dB (A) Day Time | Leq dB (A) Night Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | N1 | Garchakraberian Near Primary School |  | Silence Zone | 12.01.2020 | 57.4 | 43.1 |
| 2 | N2 | Jambari, Near OHR |  | Residential | 13.01.2020 | 51.6 | 41.0 |
| 3 | N3 | Manuchak Jalpai |  | Residential | 11.01.2020 | 47.8 | 37.0 |
| 4 | N4 | Parbatipur, Near OHR |  | Residential | 09.01.2020 | 52.4 | 38.2 |
| 5 | N5 | Nandigram CT |  | Residential | 09.01.2020 | 56.9 | 46.1 |
| 6 | N6 | Haripur, Near OHR |  | Residential | 11.01.2020 | 51.7 | 41.3 |
| 7 | N7 | Kamalpur, Near OHR |  | Residential | 13.01.2020 | 53.5 | 41.6 |
| 8 | N8 | Birulia |  | Residential | 08.01.2020 | 51 | 42.2 |
| 9 | N9 | Monoharpur, Near Primary School |  | Silence Zone | 09.01.2020 | 51.9 | 40.2 |
| 10 | N10 | Kanakpur, Near OHR |  | Residential | 12.01.2020 | 35.3 | 32.4 |
| 11 | N11 | Gangra, Near OHR |  | Residential | 14.01.2020 | 52.8 | 46.1 |
| 12 | N12 | Amgechia, Near OHR |  | Residential | 14.01.2020 | 50.7 | 39.3 |
| 13 | N13 | Gokulnagar,near OHR |  | Residential | 14.01.2020 | 55.2 | 43.6 |
| 14 | N14 | Naynan |  | Residential | 15.01.2020 | 52.1 | 42.1 |
| 15 | N15 | Dinabandhupur |  | Residential | 15.01.2020 | 54.3 | 39.5 |
| 16 | N16 | Purushottompur |  | Residential | 15.01.2020 | 47.2 | 36.9 |
| 17 | N17 | Banasri Garui |  | Residential | 16.01.2020 | 49.6 | 39.3 |
| 18 | N18 | Gholpukuria |  | Residential | 16.01.2020 | 51.0 | 41.9 |
| 19 | N19 | Narasinghapur Jalpia |  | Residential | 16.01.2020 | 53.6 | 39.0 |
| 20 | N20 | Gopalchak |  | Residential | 16.01.2020 | 53.2 | 41.6 |
| Code/Category |  | Leq Day Time dB(A) - Standard |  |  | Leq Night Time dB(A) - Standard |  |  |
| A/Industrial |  | 75 |  |  | 70 |  |  |
| B/Commercial |  | 65 |  |  | 55 |  |  |
| C/Residential |  | 55 |  |  | 45 |  |  |
| D/Ecological- Sensitive |  | 50 |  |  | 40 |  |  |
|  |  |  | Note: National level Central Pollution Control Board’s, CPCB’s (Govt. of India) guideline/ protocol used for sampling |  |  |  |  |

Table 43: Surface Water Monitoring Data (Package EM 02)

| Location Name | Date | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | pH (25 ºC) | Turbidity (NTU) | Chloride as Cl in mg/l | Fluoride as F in mg/l | Nitrate as NO3 in mg/l | COD in mg/l | BOD in mg/l | DO in mg/l | Total alkalinity as CaCO3 in mg/l | Hardness as CaCO3 in mg/l | Oil & Grease in mg/l | Total Coliform MPN/100ml | Faecal Coliform MPN/100ml |
| Manoharpur | 26.12.2019 | 6.79 | 4.4 | 294.94 | BDL | 13 | 89.28 | 10 | 7.94 | 130 | 389.5 | 8.6 | <1,<10,<100 | <1,<10,<100 |
| Haripur | 26.12.2019 | 6.61 | 3.18 | 46.08 | BDL | 9.72 | 88.2 | 11 | 8.1 | 90 | 133 | 11.6 | <1,<10,<100 | <1,<10,<100 |
| Nandigram CT | 09.01.2020 | 7.2 | 14.21 | 414.77 | BDL | 16.2 | 97.2 | 12 | 5.67 | 170 | 432.4 | 8.6 | <1,<10,<100 | <1,<10,<100 |
| Parbatipur | 10.01.2020 | 7.18 | 14.36 | 368.68 | BDL | 12.6 | 58.32 | 7 | 5.75 | 160 | 441.6 | 8.8 | <1,<10,<100 | <1,<10,<100 |
| Manuchak Jalpai | 11.01.2020 | 7.23 | 8.39 | 152.08 | BDL | 13.5 | 1047.2 | 11 | 7.05 | 780 | 193.2 | 10 | <1,<10,<100 | <1,<10,<100 |
| Jambari | 11.01.2020 | 7.12 | 3.32 | 198.17 | BDL | 14.88 | 66.64 | 8 | 7.94 | 150 | 184 | 9.6 | <1,<10,<100 | <1,<10,<100 |
| Garchakraberia | 12.01.2020 | 7.33 | 3.54 | 267.29 | BDL | 16.2 | 1078 | 12 | 7.62 | 80 | 156.4 | 9.4 | <1,<10,<100 | <1,<10,<100 |
| Kamalpur | 12.01.2020 | 7.16 | 7.32 | 55.3 | BDL | 12.8 | 98.0 | 11 | 7.13 | 160 | 101.2 | 9.2 | <1,<10,<100 | <1,<10,<100 |
| Kanakpur | 13.01.2020 | 6.97 | 8.94 | 783.45 | BDL | 18.6 | 116.64 | 12 | 7.86 | 100 | 101.2 | 9 | <1,<10,<100 | <1,<10,<100 |
| Birulia | 26.12.2019 | 6.85 | 4.08 | 188.95 | BDL | 11.74 | 79.36 | 9 | 7.45 | 100 | 256.5 | 12.4 | <1,<10,<100 | <1,<10,<100 |
| Desirable Limit as per IS:2296:1982 |  | 6.5 to 8.5 | -- | -- | 2 | -- | 250 | 30 | 4 | 2100 | 300 | 10 | 500 | -- |

Table 44: Ground Water Quality (Package EM 02)

| Location Name | Date | Parameter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | pH (25 ºC) | Turbidity (NTU) | Conductivity µs/ cm | TDS in mg/l | Total Hardness as CaCO3 in mg/l | Chloride as Cl in mg/l | Sulfate as SO4 in mg/l | Nitrate as NO3 in mg/l | Iron as Fe in mg/l | Copper As Cu in mg/l | Lead as Pb in mg/l | Arsenic as As in mg/l | Mercury as Hg in mg/l |
| Manoharpur | 26.12.2019 | 7.53 | 4.31 | 219.4 | 147 | 616.4 | 198.16 | 58.43 | 6.7 | 0.12 | BDL | BDL | BDL | BDL |
| Haripur | 26.12.2019 | 7.35 | 2.98 | 186.57 | 125 | 386.4 | 129.04 | 69.73 | 7.8 | 0.076 | BDL | BDL | BDL | BDL |
| Nandigram CT | 09.01.2020 | 7.49 | 3.58 | 1016.42 | 681 | 404.8 | 230.43 | 81.15 | 9.2 | BDL | BDL | BDL | BDL | BDL |
| Parbatipur | 10.01.2020 | 7.5 | 3.54 | 1.67.19 | 683 | 414 | 276.51 | 80.02 | 8.6 | BDL | BDL | BDL | BDL | BDL |
| Manuchak Jalpai | 11.01.2020 | 7.52 | 1.26 | 1547.76 | 1037 | 349.6 | 368.68 | 46.57 | 10.2 | 0.1 | BDL | BDL | BDL | BDL |
| Jambari | 11.01.2020 | 7.53 | 1.48 | 974.63 | 653 | 276 | 230.43 | 44.99 | 9.7 | 0.12 | BDL | BDL | BDL | BDL |
| Garchakraberia | 12.01.2020 | 7.46 | 1.12 | 879.1 | 589 | 248.4 | 184.34 | 43.41 | 9.4 | 0.11 | BDL | BDL | BDL | BDL |
| Kamalpur | 12.01.2020 | 7.51 | 1.14 | 902.98 | 605 | 239.2 | 184.34 | 61.94 | 10 | 0.08 | BDL | BDL | BDL | BDL |
| Kanakpur | 13.01.2020 | 7.54 | 1.35 | 1180.6 | 791 | 506 | 322.6 | 42.28 | 10.2 | BDL | BDL | BDL | BDL | BDL |
| Birulia | 26.12.2019 | 7.38 | 15.91 | 207.46 | 139 | 665 | 382 | 71.76 | 7.4 | 0.14 | BDL | BDL | BDL | BDL |
| Acceptable Limit |  | 6.5 to 8.5 | 1 N.T.U | -- | 500 | 200 | 250 | 200 | 45 | 0.3 | 0.05 | 0.01 | 0.01 | 0.001 |
| Permissible limit in the absence of alternate source |  | No Relaxation | 5 | -- | 2000 | 600 | 1000 | 400 | No relaxation | 1.0 | 1.5 | No relaxation | 0.05 | No relaxation |

Table 45: Soil Quality Data (Package EM 02)

| No. | Parameters | Result | Unit |
| --- | --- | --- | --- |
| Nandigram II- Manoharpur (Zone- 6) Date: 26.12.2019 |  |  |  |
| 1 | pH (at 18 ºC) Value (1:2) | 6.89 | -- |
| 2 | Elect. Conductivity (1:5) | 384 | µs/ cm |
| 3 | Moisture | 7.52 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 66.42 | mg/kg |
| 6 | Magnesium as Mg | 25.4 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 5.8 | % |
| 9 | Oil & Grease | 0.42 | mg/kg |
| 10 | Phosphorous as P | 1.89 | mg/kg |
| 11 | Sodium as Na | 38.16 | mg/kg |
| 12 | Potassium as K | 66.78 | mg/kg |
| 13 | Nitrogen as N | 402.25 | mg/kg |
| Nandigram I – Haripur (Zone- 19) Date: 26.12.2019 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.24 | -- |
| 2 | Elect. Conductivity (1:5) | 498.0 | µs/ cm |
| 3 | Moisture | 9.11 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 74.84 | mg/kg |
| 6 | Magnesium as Mg | 28.12 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 6.4 | % |
| 9 | Oil & Grease | 0.5 | mg/kg |
| 10 | Phosphorous as P | 2.09 | mg/kg |
| 11 | Sodium as Na | 45.63 | mg/kg |
| 12 | Potassium as K | 75.14 | mg/kg |
| 13 | Nitrogen as N | 462.76 | mg/kg |
| Nanigram I- Nandigram CT (Zone- 18) Date: 09.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.17 | -- |
| 2 | Elect. Conductivity (1:5) | 460 | µs/ cm |
| 3 | Moisture | 8.02 | % |
| 4 | Texture | Loam |  |
| 5 | Calcium as Ca | 62.82 | mg/kg |
| 6 | Magnesium as Mg | 23.77 | mg/kg |
| 7 | Permeability | 10-6 to 10-3 | cm/s |
| 8 | Organic Matter | 7 | % |
| 9 | Oil & Grease | 0.63 | mg/kg |
| 10 | Phosphorous as P | 2.36 | mg/kg |
| 11 | Sodium as Na | 47.08 | mg/kg |
| 12 | Potassium as K | 78.19 | mg/kg |
| 13 | Nitrogen as N | 473.15 | mg/kg |
| Nandigram I -Parbatipur (Zone-13) Date: 10.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 6.74 | -- |
| 2 | Elect. Conductivity (1:5) | 390 | µs/ cm |
| 3 | Moisture | 7.74 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 55.64 | mg/kg |
| 6 | Magnesium as Mg | 21.92 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 5.5 | % |
| 9 | Oil & Grease | 0.41 | mg/kg |
| 10 | Phosphorous as P | 1.97 | mg/kg |
| 11 | Sodium as Na | 43.69 | mg/kg |
| 12 | Potassium as K | 71.24 | mg/kg |
| 13 | Nitrogen as N | 437.88 | mg/kg |
| Nandigram I - Manuchak Jalpai (Zone- 12) Date: 11.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 6.9 | -- |
| 2 | Elect. Conductivity (1:5) | 422 |  |
| 3 | Moisture | 7.12 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 52.21 | mg/kg |
| 6 | Magnesium as Mg | 20.68 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 5.2 | % |
| 9 | Oil & Grease | 0.47 | mg/kg |
| 10 | Phosphorous as P | 2.08 | mg/kg |
| 11 | Sodium as Na | 45 | mg/kg |
| 12 | Potassium as K | 73.12 | mg/kg |
| 13 | Nitrogen as N | 452.82 | mg/kg |
| Nandigram I – Jambari (Zone- 7) Date: 11.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.26 | -- |
| 2 | Elect. Conductivity (1:5) | 445 | µs/ cm |
| 3 | Moisture | 7.98 | % |
| 4 | Texture | Sandy |  |
| 5 | Calcium as Ca | 58.84 | mg/kg |
| 6 | Magnesium as Mg | 22.46 | mg/kg |
| 7 | Permeability | 10-6 to 10-3 | cm/s |
| 8 | Organic Matter | 5.8 | % |
| 9 | Oil & Grease | 0.61 | mg/kg |
| 10 | Phosphorous as P | 1.76 | mg/kg |
| 11 | Sodium as Na | 43.26 | mg/kg |
| 12 | Potassium as K | 68.94 | mg/kg |
| 13 | Nitrogen as N | 410 | mg/kg |
| Nandigram I – Garchakraberia (Zone- 2) Date: 12.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 6.85 | -- |
| 2 | Elect. Conductivity (1:5) | 403 | µs/ cm |
| 3 | Moisture | 7.15 | % |
| 4 | Texture | Sandy |  |
| 5 | Calcium as Ca | 53.39 | mg/kg |
| 6 | Magnesium as Mg | 19.78 | mg/kg |
| 7 | Permeability | 10-6 to 10-3 | cm/s |
| 8 | Organic Matter | 5.2 | % |
| 9 | Oil & Grease | 0.54 | mg/kg |
| 10 | Phosphorous as P | 1.68 | mg/kg |
| 11 | Sodium as Na | 41.87 | mg/kg |
| 12 | Potassium as K | 66.7 | mg/kg |
| 13 | Nitrogen as N | 390 | mg/kg |
| Nandigram II – Kamalpur (Zone – 1) Date: 12.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.18 | -- |
| 2 | Elect. Conductivity (1:5) | 489 | µs/ cm |
| 3 | Moisture | 7.6 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 55.26 | mg/kg |
| 6 | Magnesium as Mg | 21.82 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 5.8 | % |
| 9 | Oil & Grease | 0.5 | mg/kg |
| 10 | Phosphorous as P | 2.07 | mg/kg |
| 11 | Sodium as Na | 47 | mg/kg |
| 12 | Potassium as K | 74.26 | mg/kg |
| 13 | Nitrogen as N | 465 | mg/kg |
| Nandigram II - Kanakpur (Zone- 7) Date: 13.01.2020 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.26 | -- |
| 2 | Elect. Conductivity (1:5) | 472 | µs/ cm |
| 3 | Moisture | 8.2 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 59.24 | mg/kg |
| 6 | Magnesium as Mg | 23.02 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 6.1 | % |
| 9 | Oil & Grease | 0.54 | mg/kg |
| 10 | Phosphorous as P | 2.46 | mg/kg |
| 11 | Sodium as Na | 50.16 | mg/kg |
| 12 | Potassium as K | 76 | mg/kg |
| 13 | Nitrogen as N | 482.5 | mg/kg |
| Nandigram II – Birulia (Zone- 3) Date: 26.12.2019 |  |  |  |
| 1 | pH(at 18 ºC) Value (1:2) | 7.08 | -- |
| 2 | Elect. Conductivity (1:5) | 406 | µs/ cm |
| 3 | Moisture | 8.13 | % |
| 4 | Texture | Sandy Loam |  |
| 5 | Calcium as Ca | 72.18 | mg/kg |
| 6 | Magnesium as Mg | 27.14 | mg/kg |
| 7 | Permeability | 10-9 to 10-6 | cm/s |
| 8 | Organic Matter | 6.2 | % |
| 9 | Oil & Grease | 0.64 | mg/kg |
| 10 | Phosphorous as P | 1.64 | mg/kg |
| 11 | Sodium as Na | 41.34 | mg/kg |
| 12 | Potassium as K | 70.56 | mg/kg |
| 13 | Nitrogen as N | 442.32 | mg/kg |

(Sampling between 26.12.2019 – 13.01.2020)

Graphical presentation of results shown below.

Analyses of Results

- For Nandigram I and II blocks, the PM10 and PM2.5 values and also SO2, NO2, CO concentrations at all the monitoring locations are within the prescribed limit of National Ambient Air Quality Standard as well as WHO standard.

- Except sensitive area (zone 1) noise levels for all the locations are within the prescribed limit.

- All the surface water value shows the high DO concentration, mainly due to eutrophic pond or the temperature of the particular day when the sample was collected. BOD value is within the prescribed limit it shows the less organic matter presence in the water bodies.

- For ground water all the values except hardness are within the permissible limit. Concentration of heavy metals (Pb, Cd, CU and Hg) are below detection imit. Ground water should be treated before drinking.

# VII	SUMMARY OF KEY ISSUES AND REMEDIAL ACTIONS

- Status of EMP application discussed in earlier section. Some partial compliance issues have been noted during report period.

- Table 46 lists the implementation status of corrective action observed during November 2019 – April 2020 in work sites of North 24 pgs packages that were active during May – October 2019. Since activity of Bankura district packages started during the report period, implementation of corrective action will be reflected in next SEMR.  The implementation plan for partially completed corrective actions are presented in Table 47.

Table 46: Implementation of Corrective Action Plan (North 24 pgs)

| Sr no. | Pending issues (May – October 2019 report period) | Status | Further Action Required | Responsible- Implementation | Budget |
| --- | --- | --- | --- | --- | --- |
| 1 | Facilities in the worksite should include an office and toilet for workers (For package N 24 pgs/02A and 02B) | Partially improved. | Further improvement is required | Contractor | Contractor work cost and EMP budget |
| 2 | Facilities in the construction camps, especially sleeping arrangements, toilet facilities, provision of drinking water and fuel for cooking, waste and wastewater management needs to be improved (For package N 24 pgs/02A and 02B) | Partially improved. | Immediate improvement of housekeeping and basic facilities for workers, especially at Zone 18 of Haroa | Contractor | Contractor work cost and EMP budget |
| 3 | The ICDS functioning inside Zone 17: Shalipur pose risk to the children in terms of high noise and accidents (For package N 24 pgs/02A) | Proper separation of the ICDS area from work site has been done. | None. |  |  |
| 4 | Complete use of PPE, particularly use of working safety harness, shoes, hand gloves and ear plug (Both – N24P/02A – Haroa & N24P/02B Bhangar II) | Improvement noted. Use of helmet – noted | Complete supply and use of PPEs particularly safety harness for working at height, shoes, hand gloves – Immediately | Contractor | Contractor work cost and EMP budget |
| 5 | Barricading and caution tapes should be put along pipeline trenches at all pipeline laying sites. (For package N 24 pgs/02A and 02B) | Partially improved. | Further improvement is required | Contractor | Contractor work cost and EMP budget |
| 6 | Access should be provided in front of residences and commercial establishments during pipeline trenching. (For package N 24 pgs/02A and 02B) | Access is mostly provided now. | None |  |  |
| 7 | The pit latrines installation at Zone 10 and 13 of Bhangar – II Block as well as in WTP site of Subproject may cause faecal coliform contamination to groundwater (For package N 24 pgs/01 and 02B) | These latrines have not yet been replaced though mostly abandoned. | Considering onset of monsoon, all these latrines should either be replaced with systems promoted under Swachh Bharat or removed. | Contractor | Contractor work cost and EMP budget |
| 8 | Improvement of housekeeping, access to working area (Haroa – Zone1 of package N24P/02A, Bhangar II – Zone 10, 13 package N24P/02B) | Partially improved. | Further improvement is required | Contractor | Contractor work cost and EMP budget |
| 9 | Management of wastewater at camp sites (Haroa – Zone1 package N24P/02A, Bhangar II – Zone 10, 13 of package N24P/02B) | Partially improved. | Further improvement is required | Contractor | Contractor work cost and EMP budget |

- Table 47 provides the recommended corrective action plan that has been devised and target dates that have been set so as to remove these non-compliances. The concerned Contractors have been suitably advised. Contractors have also been advised to provide written commitment for implementation of corrective action plan.

Table 47: Implementation Plan for Pending/ New Corrective Actions (North 24 pgs and Bankura)

| Sr no. | New/ Pending issues | Action Required | Responsible- Implementation/ Budget | Target Date | Responsible – Monitoring | Responsible – Reporting | Indicator of Compliance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| North 24 pgs |  |  |  |  |  |  |  |
| 1 | Facilities in the worksite should include an office and toilet for workers (For package N 24 pgs/02A and 02B) | Immediate improvement in the site to provide basic facilities | Contractor/ Contractor work cost and EMP budget | Within 15th July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 2 | Facilities in the construction camps, especially sleeping arrangements, toilet facilities, provision of drinking water and fuel for cooking, waste and wastewater management needs to be improved (For package N 24 pgs 02A and 02B) | Immediate improvement of housekeeping and basic facilities for workers, especially at Zone 18 of Haroa and Zone 10, 13 of Bhangar II block | Contractor/ Contractor work cost and EMP budget | Within 15th July 2020. | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 3 | Use of PPEs e.g. safety shoes, gloves and goggles should be improved at all sites especially for activities like concrete braking while use of safety harness is mandatory during working at height. . (For package N 24 pgs/02A and 02B) | Although Contractor has provided adequate numbers of PPEs but use of these PPEs during hazardous activities are inadequate. Hence, management with respect to use of PPEs should be improved. | Contractor-PIU/ Contractor work cost and EMP budget | Immediate | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 4 | Barricading and caution tapes should be put along pipeline trenches at all pipeline laying sites. (For package N 24 pgs/02A and 02B) | Use of barricading and caution tapes should be ensured. | Contractor- PIU/ Contractor work cost and EMP budget | Immediate | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 5 | Labour camp located inside ICDS in Zone 16 pose risk to the children in terms of social distancing, high noise and accidents (package N 24 pgs/02A) | This labour camp should be immediately shifted from this location. | Contractor/ PIU Contractor work cost and EMP budget | Within 15th July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 6 | The pit latrines installed at Zone 10 and 13 of Bhangar – II Block as well as in WTP site of Subproject may cause faecal coliform contamination to groundwater (For package N 24 pgs/01 and 02B) | These pit latrines should at least be replaced with systems promoted under Swachh Bharat or be removed from site | Contractor- PIU/ Contractor work cost and EMP budget | Within 31st July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 7 | Improvement of housekeeping, access to working area (Haroa – Zone1 of package N24P/02A, Bhangar II – Zone 10, 13 package N24P/02B) | Immediate improvement of housekeeping and access to working area to be ensured. | Contractor/ Contractor work cost and EMP budget | Immediate | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 8 | Management of wastewater at camp sites (Haroa – Zone1 package N24P/02A, Bhangar II – Zone 10, 13 of package N24P/02B) | Further improvement in wastewater management to be ensured so that inappropriate discharge of wastewater is avoided. | Contractor/ Contractor work cost and EMP budget | Within 15th July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 9 | Discharge of wastewater from WTP (package N 24 pgs/01) site is a concern as there is no drainage in the site and may cause severe water logging during monsoon. | Discussion should be held with HIDCO to establish appropriately sized discharge line to evacuate water/ wastewater from the site | Contractor/ PIU Contractor work cost and EMP budget | 15th July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 10 | Health checkup of workers under package N 24pgs/01 pending during report period | Immediate arrangement of health checkup camp | Contractor/ Contractor work cost and EMP budget | Immediate | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 11 | Non availability of First aid box at few pipe laying locations | Immediate arrangement of first aid box at all work locations | Contractor/ Contractor work cost and EMP budget | Immediate | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| Bankura |  |  |  |  |  |  |  |
| 1. | Facilities in the worksite should include site office, labor hutment with toilet, drinking water, electricity, fuel for cooking, solid waste and waste water management etc. at WTP and Intake site for package BK/01 | Immediate improvement in the site to provide basic facilities | Contractor/ Contractor work cost and EMP budget | 31st July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and checking |
| 2. | Renewal of Workman Compensation, and All Risk Policy for package BK/01 | Immediate follow-up required | Contractor/ Contractor work cost and EMP budget | 31st July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and document checking |
| 3. | Baseline surface water quality results pending for package BK/01 | Immediate submission required. | Contractor/ Contractor work cost and EMP budget | 31st July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation and document checking |
| 4 | Non availability of proper work display board and grievance registration contact number board at all work sites for package BK/03, BK/02A, BK/02B and BK/04 | Immediate | Contractor/ Contractor work cost and EMP budget | 31st July 2020 | DSISC, PIU | PIU, PMU- PMC | Site observation |

If on-going construction, include %physical progress and expected date of completion

Specify (environmental clearance? Permit/consent to establish? Forest clearance? Etc.)

Specify if obtained, submitted and awaiting approval, application not yet submitted

Example: Environmental Clearance requires ambient air quality monitoring, Forest Clearance/Tree-cutting Permit requires 5 trees for every tree, etc.